| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of CL-MIMO Codebooks |  |
| Date Submitted | 2008–05–14 |  |
| Source(s) | Sangwoo Lee, Samsung Electronics Jiyun Seol, Samsung Electronics Keun Chul Hwang, Samsung Electronics Mune Joon, Samsung Electronics Inseok Hawng, Samsung Electronics Hassan Yaghoobi, Intel Corporation Peiying Zhu, Nortel Peiying Zhu, Nortel | E-mail: sangwoo07.lee@samsung.com jiyun.seol@samsung.com keunchul.hwang@samsung.com june.moon@samsung.com is91.hwang@samsung.com hassan.yaghoobi@intel.com pyzhu@nortel.com pyzhu@nortel.com |
| Re: | IEEE802.16Rev2/D4 |  |
| Abstract | This contribution provides modifications for IEEE802.16Rev2/D3 section 8.4.5.4.10.15 |  |
| Purpose | For discussion and approval by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of closed-loop MIMO codebooks

Sangwoo Lee, Jiyun Seol, Keun Chul Hwang, June Moon, Inseok Hwang

Samsung Electronics Co., LTD.

Hassan Yaghoobi

Intel Corporation

Peiying Zhu

Nortel

Peiying Zhu

Nortel

Introduction

In this contribution, we’d like to propose a way to further enhance the 16e codebooks from the computational complexity point of view. For example, while V(2, 1, 3) has the same values as the first entries in V(2, 2, 3), there are no such relationship in case of the codebooks for (V(4, 1, 3) and V(4, 2, 3)). By modifying the codebook generation rule, the computation burden for MS to search the codebook can be decreased without any performance loss in some implementation.

To reduce the codebook search complexity, we suggest to change the column selection rule for generating V(4,2,3). Current 16e codebook for V(4,2,3) is obtained with V(4,1,3) after expensingthe expansion withusing the Hhouseholder matrix and taking the third and forth columns of the resulting matrix. With this codebook,In some implementation, the MS should calculatesteps through V(4,1,3) and V(4,2,3) for rank selection. However, if the first and second columns are selected for V(4,2,3), MS can reduce computational complexity withby calculating only V(4,2,3) and reusing the intermediate results for V(4,1,3).

  - Current 16e codebook for V(4,2,3): V(4,2,3) = H(V(4,1,3)):,3:4

  - Proposed codebook for V(4,2,3): V(4,2,3) = H(V(4,1,3):,1:2

Since the subspace spanned by the first two columns and the subspace spanned by the last two columns are orthogonal and complementary, the two subspaces uniquely determine each other. Therefore, the performances of the two codebooks are identical for maximum likelihood receiver with equal power loading. For the implementation that steps through the rank 1 and rank 2 codebooks for rank selection, the change can reduce the computation complexity because the intermediate results for rank 2 codeword search can be used in the rank 1 codeword search.

With the proposed codebook, the computational complexity at MS can be reduced about 31%, since only the rank-2 codebook selection metric needs to be calculated and rank-1 metric can be reused. Figure 1 shows the performance of current 16e and proposed codebooks. Note that the performance of proposed codebook is the same as that of current 16e codebook.

Fig. 1. Performance comparison of current 16e V(4,2,3) and proposed V(4,2,3)

include the rank-1 codebook vectors in rank-2 and rank-3 codebook matrices for the codebooks which are generated with HE operation.

# Text Changes

[On page 834, line 55, insert paragraph and equation]

The fourth operation generates an N by 2 matrix from a unit N vector, vN, by taking the first 2 columns of H(vN) as shown in Equation (XX):

HF(vN)=H(vN):,1:2.				(XX)

[On page 834,correct  last paraghraphas below]

The three four operations jointly generate eleven matrix codebooks from vector codebooks as shown in Table 402, where each entry is the generating operation of one codebook.

55insert

[On page 786, Table 392, correct table as below]

[On page 835, Table 402, correct table as below]

| Nt, L / S | 2 | 3 | 4 |
| --- | --- | --- | --- |
| 2, 3 |  | -- | -- |
| 3, 3 |  |  | -- |
| 4, 3 |  |  |  |
| 3, 6 |  |  | -- |
| 4, 6 |  |  |  |