| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correction of ARQ feedback IE in Rev2/D4 |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Youngbin Chang, Geunhwi Lim, Yeongmoon Son Samsung Electronics* | E-mail: yb.chang@samsung.com geunhwi.lim@samsung.com |
|  | Giovanni Maggi, Aik Chindapol Nokia Siemens Networks* | E-mail: giovanni.maggi.ext@nsn.com aik.chindapol@nsn.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments of LB #26c |  |
| Abstract | Correction of ARQ feedback IE in Rev2/D4 |  |
| Purpose | Adopt the proposed text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correction of ARQ feedback IE in Rev2/D4

Youngbin Chang, Geunhwi Lim, Yeongmoon Son

Samsung Electronics

Giovanni Maggi, Aik Chindapol

Nokia Siemens Networks

# Introduction

# There was an editor's error in applying the resolution of comment #1076 in LB 26a (doc 80216-07_001r3.cmt). It was applied to Table 166 instead of Table 167 as stated in the accepted contribution C80216maint-08/005 http://www.ieee802.org/16/maint/contrib/C80216maint-08_005.pdf.

Now the two tables (previously 166 and 167) are 167 and 168.

- Revert Table 167 in D4 back to the original version in D2 (table 166)

- Apply the accepted resolution of comment #1076 in doc 80216-07_001r3.cmt, which is to adopt the contribution C80216maint-08/005

# Specification changes

Change #1: Revert Table 167 in D4 back to the original version in D2 (table 166)

[Replace the contents of table 167 (“ARQ Feedback IE format” Rev2/D4) on page 268 with the following:]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_Feedback_IE (LAST) { | variable | — |
| CID | 16 | The ID of the connection being referenced |
| LAST | 1 | 0 = More ARQ feedback IEs in the list 1 = Last ARQ feedback IE in the list |
| ACK Type | 2 | 0x0 = Selective ACK entry 0x1 = Cumulative ACK entry 0x2 = Cumulative with Selective ACK entry 0x3 = Cumulative ACK with Block Sequence ACK entry |
| BSN | 11 | — |
| Number of ACK Maps | 2 | If ACK Type == 01, the field is reserved and set to 00. Otherwise the field indicates the number of ACK maps: 0x0 = 1, 0x1 = 2, 0x2 = 3, 0x3= 4 |
| If (ACK Type != 01) { | — | — |
| for (i = 0; i< Number of ACK Maps + 1; ++i) { | — | — |
| if (ACK Type != 3) { | — | — |
| Selective ACK Map | 16 | — |
| } |  | — |
| else { | — | Start of Block Sequence ACK Map definition (16 bits) |
| Sequence Format | 1 | Number of block sequences associated with descriptor 0: 2 block sequences 1: 3 block sequences |
| if (Sequence Format = 0) { | — | — |
| Sequence ACK Map | 2 | — |
| Sequence 1 Length | 6 | — |
| Sequence 2 Length | 6 | — |
| Reserved | 1 | — |
| } | — | — |
| else{ | — | — |
| Sequence ACK Map | 3 | — |
| Sequence 1 Length | 4 | — |
| Sequence 2 Length | 4 | — |
| Sequence 3 Length | 4 | — |
| } | — | — |
| } | — | End of Block Sequence ACK Map definition |
| } | — | — |
| } | — | — |
| } | — | — |

Change #2: Apply the accepted resolution of comment #1076 in doc 80216-07_001r3.cmt, which is to adopt the contribution C80216maint-08/005

[Modify the contents of table 168 (“ARQ Feedback IE format with extended capability”, Rev2/D4) on page 272 as following:]

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE (LAST) { | variable | — |
| CID | 16 | The ID of the connection being referenced |
| LAST | 1 | 0 = More ARQ feedback IE in the list 1 = Last ARQ feedback IE in the list |
| ACK Type | 3 | 0x0 = Selective ACK entry 0x1 = Cumulative ACK entry 0x2 = Cumulative with Selective ACK entry 0x3 = Cumulative ACK with Block Sequence ACK entry 0x4 = Block Sequence ACK entry 0x5-0x7 : Reserved, set to zero |
| BSN | 11 | — |
| Number of ACK MapsReserved | 1 | ReservedSet to zero |
| if (ACK Type!= 001) { | — | — |
| If(ACK Type!=3)if (ACK Type == 0 ¦¦ ACK Type == 2) { | — | — |
| MAP Last Bit | 1 | 0: Another ACK Map follows 1: This is the last ACK Map |
| Selective ACK Map | 15 | — |
| } | — | — |
| else{ | — | — |
| Map Last bit | 1 | 0: Another ACK Map follows 1: This is the last ACK Map |
| Sequence Format | 1 | Number of block sequences associated with descriptor 0: 2 block sequences, 1: 3 block sequences |
| if(ACK Type == 3) { | — | — |
| if (Sequence Format == 0) { | — | — |
| Sequence ACK Map | 1 | This bit corresponds to the sequence 2 length field in the descriptor. |
| Sequence 1 Length | 6 | Sequence 1 field always represents NAK blocks |
| Sequence 2 Length | 7 | — |
| } | — | — |
| else { | — | — |
| Sequence ACK Map | 2 | The MSB of this field corresponds to the sequence 2 length field in the descriptor. |
| Sequence 1 Length | 4 | Sequence 1 field always represents NAK blocks |
| Sequence 2 Length | 4 | — |
| Sequence 3 Length | 4 | — |
| } | — | —End of Block Sequence ACK Map definition |
| } | — | — |
| if(ACK Type == 4) { | — | — |
| if (Sequence Format == 0) { | — | — |
| Sequence ACK Map | 2 | — |
| Sequence 1 Length | 6 | — |
| Sequence 2 Length | 6 | — |
| } | — | — |
| else{ | — | — |
| Sequence ACK Map | 3 | — |
| Sequence 1 Length | 4 | — |
| Sequence 2 Length | 4 | — |
| Sequence 3 Length | 3 | — |
| } | — | — |
| } | — | — |
| } | — | — |
| While(!Map Last bit) { | — | — |
| If(ACK Type!=3)if (ACK Type == 0 ¦¦ACK Type == 2) { | — | — |
| MAP Last Bit | 1 | 0: Another ACK Map follows 1: This is the last ACK Map |
| Selective ACK Map | 15 | — |
| } | — | — |
| else{ | — | —Start of Block Sequence ACK Map definition (16 bits) |
| MAP Last Bit | 1 | 0: Another ACK Map follows 1: This is the last ACK Map |
| Sequence Format | 1 | Number of block sequences associated with descriptor 0: 2 block sequences, 1: 3 block sequences |
| if (Sequence Format == 0) { | — | — |
| Sequence ACK Map | 2 | — |
| Sequence 1 Length | 6 | — |
| Sequence 2 Length | 6 | — |
| } | — | — |
| else{ | — | — |
| Sequence ACK Map | 3 | — |
| Sequence 1 Length | 4 | — |
| Sequence 2 Length | 4 | — |
| Sequence 3 Length | 3 | — |
| } | — | — |
| } | — | — |
| } | — | —End of Block Sequence ACK Map definition |
| } | — | — |
| } | — | — |