| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fix for type values of MIH TLVs. |  |
| Date Submitted | 2008-05-15 |  |
| Source(s) | Richard van Leeuwen Motorola | rvanleeuwen@motorola.com |
| Re: | IEEE802.16Rev2/D4 |  |
| Abstract | Fix for type values of MIH TLVs. |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Fix for type values of MIH TLVs

Richard van Leeuwen

Motorola

# Comments (3239 and 3240 and reply comment from Yeong Son)

- Table 549 does not show the type values for the new MIH message encoding TLVs.

- The assigned Type values (for common TLV encodings) for those MIH TLVs are duplicates and need to be re-assigned.

- The “Cycle” and “Delivery Method and Status Code” TLVs are only used in PKM-REQ/RSP messages. Their type values do not need to be globally unique and should not be classified as a common TLV encoding.

# Suggested remedy

- Add “MIHF frame”, “MIHF frame type”, “Query ID” TLVs to table 549.

- Reassign the TLV type values of “MIHF frame”, “MIHF frame type”, “Query ID” TLVs (using values 131 – 129).

- Move the “Cycle” and “Delivery Method and Status Code” TLVs to clause 11.9 (PKM-REQ/RSP management message encodings).

- Reassign the TLV type values of “Cycle” and “Delivery Method and Status Code” TLVs.

# Text Changes

Add MIH TLVs to Table 549 as follows:

Table 549—Type values for common TLV encodings

| Type | Name |
| --- | --- |
| 149 | HMAC Tuple |
| 148 | MAC Version Encoding |
| 147 | Current Transmit Power |
| 146 | Downlink Service Flow |
| 145 | Uplink Service Flow |
| 144 | Vendor ID Encoding |
| 143 | Vendor-Specific Information |
| 142 | SA-TEK-Update |
| 141 | CMAC tuple |
| 140 | Short-HMAC tuple |
| 139 | Enabled-Action-Triggered |
| 138 | SLPID_Update |
| 137 | Next Periodic Ranging |
| 136 | MAC Hash Skip Threshold |
| 135 | Paging Controller ID |
| 134 | Paging Informatioin |
| 133 | NSP List |
| 132 | Verbose NSP Name List |
| 131 | MIHF frame |
| 130 | MIHF frame type |
| 129 | Query ID |

Reassign type values as follows:

11.1.12 MIH message encodings

11.1.12.1 MIHF frame

This TLV is used to carry an MIHF frame. MIHF frames are specified in IEEE Std 802.21.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 142131 | variable | An MIHF frame. MIHF frames are specified in IEEE Std 802.21. | MOB_MIH-MSG PKM-REQ PKM-RSP SII-ADV |

11.1.12.2 MIHF frame type

This TLV indicates the service type of MIHF frame.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 143130 | 1 | 0: ES/CS MIH Capability Discovery 1: Event Service 2: Command Service 3: Information Service 4-255: Reserved | MOB_MIH-MSG PKM-REQ PKM-RSP SII-ADV |

Renumber subclause 11.1.12.5 and reassign type value as follows:

11.1.12.53 Query ID

The BS sends this TLV to the MS when it acknowledges receipt of an MIH Initial Request message encapsulating an MIH query, and sends it again when it sends the MIH response to the MIH query. The MS uses this TLV to correlate the MIH response with the MIH query.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 146129 | 1 | This value uniquely identifies a pending MIH query at the BS. Since MIH responses may be broadcast, the value of Query ID shall be unique per BS. | PKM-RSP SII-ADV |

Move subclause 11.1.12.3 to subclause 11.9 and reassign type value as follows:

11.1.12.3.9.40 MIH Cycle

This TLV includes the 8 LSB of the absolute frame number of the first frame where the MIH response is expected to be ready for transmittal to the MS, and the interval between subsequent frames where the MIH response may be transmitted (refer to 6.3.25).

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 14448 | 2 | The 8 MSB of this field is the 8 LSB of the absolute frame number when the BS may indicate that the MIH response is ready to be delivered to the MS by allocating bandwidth for the MS in the UL-MAP, if it is unicasting the MIH response, or when the BS may send an SIIADV message including the MIH response (refer to section 6.3.25), if the BS is broadcasting the MIH response. The 8 LSB of this field is the MIH Cycle Offset. The MIH Cycle Offset is used to indicate subsequent frames when the BS may allocate bandwidth for the MS in the UL-MAP or send an SII-ADV message including the MIH response. The subsequent frames are calculated by adding multiples of the MIH Cycle Offset to the original absolute frame number transmission opportunity. | PKM-RSP |

Move subclause 11.1.12.4 to subclause 11.9 and reassign type value as follows:

11.1.12.4.9.41 MIH Delivery Method and Status Code

This TLV is used by the MS and BS to negotiate a preferred delivery method (broadcast or unicast). Only the BS may transmit a Status Code value different from 0x00 (Null).

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 14549 | 1 | Bit #0: Unicast Bit #1: Broadcast Bit #2~6: Status code 0x00: Null 0x01: Requested information is not available. 0x02~0x1F: Reserved Bit#7: Reserved | PKM-REQ PKM-RSP |