| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Self-Organizing Networks |  |
| Date Submitted | 2008-06-05 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16m-08/016, “Call for Contributions on Project 802.16m System Description Document (SDD),” – Comments on P802.16m SDD |  |
| Abstract | This contribution proposes text for Self-Organizing networks. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Self-Organizing Networks

Joey Chou

Intel

- Introduction

This contribution proposes text for Self-Organizing networks.

- Proposed text

  - Support for Self-organization

In the highly competitive mobile communication market today, reduction of cost and complexity is the key to the success of the Next Generation Mobile Networks [1]. Traditionally, the wire-line or wireless networks, providing primarily voice centric services, are managed by centralized NMS (Network Management System) that lacks scalability and flexibility to cope with the volatile mobile Internet business. Therefore, it is a goal of operators to reduce capital and operational expenditures by utilizing Self-Organization mechanisms to automate the network deployment and maintenance processes, and collect real-time performance data to assist trend analysis and network planning.

The major drivers for Self-Organizing networks include the following:

- Increasing complexity in current mobile networks requires provisioning and tuning of huge amount of parameters in order to achieve optimal performance. SON is designed to shorten network deployment cycle time, and optimize network performance, coverage, and capacity.

- Constant changing of signal propagation in the wireless environment in providing mobile high speed data services poses great challenges to existing NMS in the performance monitoring of a MS at any given time and location. SON will utilize MS to collect huge amount of spatial-temporal data to insure customer satisfaction.

- In the early deployment phase of a new technology like 802.16, SON is engaged to shorten the time it takes to achieve system stability and technology maturity through the analysis of KPI (Key Performance Indicator).

A Self-Organizing system is based on a distributed network management model that supports the following functions [2]:

- Self-configuration – the process of bringing up network elements with minimum user or craftsperson intervention

- Plug and play for adding new NE or NE components

- Neighbor cell auto-discovery

- Handover parameter auto-discovery

- Self-optimization – the process of utilizing measurement data to optimize network performance, coverage, and capacity

- Load balancing

- Handover performance (handover historical information, ping-pong behavior prevention,

- Inter-cell interference mitigation

- Radio parameter optimization

- Transport parameter optimization

- Self-healing, Self-maintenance – the process of restoring system operation in the case of faults with minimum impacts to users

- Automatic cell / service outage detection

- Automatic NE fault recovery

- Automatic NE or system software upgrade

- Self test

- Self-planning – the process of collecting and analyzing network parameters to assist network or service planning

- Performance monitoring (e.g. network entry latency, average throughput at various QoS classes, cell edge throughput, …)

- Exceptions (e.g. number of network entry failures, number of DSx rejected, number of handoff failures, …)

- References

    - NGMN “Use Cases related to Self Organizing Network”, April 2007, Version 2.02

    - NGMN “Next Generation Mobile Networks Beyond HSPA & EVDO White Paper”, December 2006, Version 3.0