| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections for nibble alignment in MMO HARQ subburst IEs |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Seungkwon Cho, Seokhun Cho, Jaesun Cha, Young-il Kim ETRI | E-mail: skcho@etri.re.kr chosh@etri.re.kr jscha@etri.re.kr yikim@etri.re.kr |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d on P802.16Rev2/D5 |  |
| Abstract | Corrections for nibble alignment in MIMO HARQ subburst IEs |  |
| Purpose | Adopt the proposed text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Corrections for Nibble Alignment in MIMO HARQ subburst IEs

Seungkwon Cho, Seokhun Cho, Jaesun Cha, Young-il Kim

ETRI

# Problem statement

# Even though the standard explicitly requests that each subburst IE should be nibbled-aligned, most of MIMO HARQ subburst IEs, in IEEE P802.16Rev2/D5(June 2008), are not nibble aligned. Not only for the fast MAP decoding with low implementation complexity but also for maintaining the consistency throughout the standard, we need a nibble alignment before the loop starts and inside the loop.

# Specification changes

[Modify the corresponding sections as follows:]

[Modify the existing text in Table 351 in Page 771 of P802.16Rev2/D5 as shown below]

Table 351 – MIMO DL Chase HARQ Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_DL_Chase_HARQ_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | “Number of sub-bursts" in the 2D region is this filed value plus 1 |
| N ACK channel | 6 | N of HARQ ACK enabled sub-bursts in the 2D region. |
| Reserved | 2 | Shall be set to 0 |
| for(j=0;j< Nummber of sub-bursts;j++){ | - | - |
| MU Indicator | 1 | Indicates whether this DL burst is intended for multiple SS |
| Dedicated MIMO DL Control Indicator | 1 | - |
| ACK disable | 1 | When 'ACK Disable' == 1, the allocated sub-burst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For TDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if(MU indicator == 0){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| if(Dedicated MIMO DL Control Indicator == 1) { | - | - |
| Dedicated MIMO DL Control IE() | variable | - |
| } | - | - |
| Duration | 10 | - |
| Reserved | 3 | Shall be set to 0 |
| for(i=0;i<N_layer;i++){ | - | - |
| if(MU indicator == 1){ | - | - |
| RCID IE() | variable | Error: Reference source not found 참조 |
| } | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| if(ACK Disable == 0){ | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| Reserved | 1 | Shall be set to 0 |
| } else { | - | - |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| } | - | - |
| Padding | variable | Padding to nibble; shall be set to zero. |
| } | - | - |

[Modify the existing text in Table 352 in Page 773 of P802.16Rev2/D5 as shown below]

Table 352 – MIMO DL IR HARQ Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_DL_IR_HARQ_Sub-Burst IE() { | - | - |
| N sub burst | 4 | “Number of sub-bursts" in the 2D region is this filed value plus 1 |
| N ACK channel | 6 | Number of HARQ ACK enabled sub-bursts in the 2D region. |
| Reserved | 2 | Shall be set to 0 |
| for(j=0;j<Number of sub-bursts;j++){ | - | - |
| MU Indicator | 1 | Indicates whether this DL burst is intended for multiple SS |
| Dedicated MIMO DL Control Indicator | 1 | - |
| ACK disable | 1 | When 'ACK Disable' == 1, the allocated sub-burst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For TDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if(MU indicator == 0){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| if(Dedicated MIMO DL Control Indicator == 1) { | - | - |
| Dedicated MIMO DL Control IE() | variable | - |
| } | - | - |
| NSCH | 4 | In the case of vertical encoding, this value shall be half of an even numbered value based on 8.4.9.2.3.5.6 |
| Reserved | 1 | Shall be set to 0 |
| for(i=0;i<N_layer;i++){ | - | - |
| if(MU indicator == 1){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| NEP | 4 | - |
| if(ACK Disable == 0){ | - | - |
| SPID | 2 | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| Reserved | 1 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| } | - | - |
| Padding | variable | Padding to nibble; shall be set to zero. |
| } | - | - |

[Modify the existing text in Table 353 in Page 774 of P802.16Rev2/D5 as shown below]

Table 353 – MIMO DL IR HARQ for CC Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_DL_IR_HARQ_for_CC_Sub-Burst IE_ () { | - | - |
| N sub burst | 4 | "Number of sub-bursts" in the 2D region is this filed value plus 1 |
| N ACK channel | 6 | Number of HARQ ACK enabled sub-bursts in the 2D region. |
| Reserved | 2 | Shall be set to 0 |
| for(j=0;j<Number of sub-bursts;j++){ | - | - |
| MU Indicator | 1 | Indicates whether this DL burst is intended for multiple SS |
| Dedicated MIMO DL Control Indicator | 1 | - |
| ACK disable | 1 | When 'ACK Disable' == 1, the allocated sub-burst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For TDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if(MU indicator == 0){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| if(Dedicated MIMO DL Control Indicator == 1) { | - | - |
| Dedicated MIMO DL Control IE() | variable | - |
| } | - | - |
| Duration | 10 | - |
| Reserved | 3 | Shall be set to 0 |
| for(i=0;i<N_layer;i++){ | - | - |
| if(MU indicator == 1){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| if(ACK Disable == 0){ | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| SPID | 2 | - |
| Reserved | 3 | Shall be set to 0 |
| } else { | - | - |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| } | - | - |
| Padding | variable | Padding to nibble; shall be set to zero. |
| } | - | - |

[Modify the existing text in Table 354 in Page 776 of P802.16Rev2/D5 as shown below]

Table 354 – MIMO DL STC HARQ Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_DL_STC_HARQ_Sub-Burst IE() { | - | - |
| N sub burst | 4 | "Number of sub-bursts" in the 2D region is this filed value plus 1 |
| N ACK channel | 6 | Number of HARQ ACK enabled sub-bursts in the 2D region. |
| Reserved | 2 | Shall be set to 0 |
| for(j=0;j<Number of subbursts;j++){ | - | - |
| Tx count | 2 | 0b00: initial transmission 0b01: odd transmission 0b10: even transmission 0b11: Reserved |
| Duration | 10 | - |
| Sub-burst offset indication | 1 | Indicates the inclusion of sub-burst offset |
| Reserved | 2 | Shall be set to 0 |
| if (Sub-burst offset indication == 1) { | - | - |
| Sub-burst offset | 8 | Offset in slots with respect to the previous subburst defined in this data region. If this is the first subburst within the data region, this offset is with respect to slot 0 of the data region. |
| } | - | - |
| RCID IE() | variable | - |
| ACK disable | 1 | When 'ACK Disable' == 1, the allocated sub-burst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For TDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, for the burst, BS shall not perform HARQ retransmission and MS shall ignore AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if(Tx count == 00) { | - | - |
| Dedicated MIMO DL Control Indicator | 1 | - |
| if(Dedicated MIMO DL Control Indicator == 1) { | - | - |
| Dedicated MIMO DL Control IE() | variable | - |
| } | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| Reserved | 1 | Shall be set to 0 |
| } | - | - |
| if(ACK Disable == 0){ | - | - |
| ACID | 4 | - |
| } | - | - |
| } | - | - |
| Padding | variable | Padding to nibble; shall be set to zero. |
| } | - | - |

[Modify the existing text in Table 355 in Page 778 of P802.16Rev2/D5 as shown below]

Table 355 – Dedicated MIMO DL Control IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Dedicated_MIMO_DL_Control_IE() { | - | - |
| Length | 5 | Length of control information in Nibbles, including this field. |
| Control header | 3 | Bit#0: MIMO Control Info Bit#1: CQI Control Info Bit#2: Closed MIMO Control Info |
| N_layer | 2 | Number of coding/modulation layers 0b00 = 1 layer 0b01 = 2 layers 0b10 = 3 layers 0b11 = 4 layers |
| Reserved | 2 | Shall be set to 0 |
| if(MIMO Control Info == 1){ | - | - |
| Matrix | 2 | Indicates transmission matrix (See 8.4.8) 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Codebook |
| if(Dedicated Pilots == 1){ | - | Dedicated Pilots field in STC_Zone_IE() |
| Num_Streams | 2 | Indicates the number of beamformed streams which is equal to the number of pilot patterns 0b00 = 1 stream 0b01 = 2 streams 0b10 = 3 streams 0b11 = 4 streams |
| } else { | - | - |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| if(CQICH Control Info == 1){ | - | - |
| Period | 3 | Period (in frame) = 2^period |
| Frame offset | 3 | - |
| Duration | 4 | A CQI feedback is transmitted on the CQI channel indexed by the Allocation index for 10 x 2d frames. |
| CQICH_Num | 2 | Number of additional CQICHs assigned to this SS (0-3) |
| for(j=0;j<N_layer+1;j++){ | - | - |
| Allocation Indexl | 6 | Index to CQICH assigned to this layer. |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| CQICH_Num | 2 | Number of additional CQICHs assigned to this SS (0-3) |
| for(i=0;i<CQICH_Num;i++){ | - | - |
| Feedback type | 3 | Type of feedback on this CQICH |
| Allocation index | 6 | - |
| Reserved | 3 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| if (Closed MIMO Control Info == 1){ | - | - |
| if(MIMO Control Info == 1){ | - | - |
| MIMO mode = Matrix | - | - |
| }else{ | - | - |
| MIMO mode = Matrix in STC_Zone_IE() | - | - |
| } | - | - |
| if(MIMO mode == 00 or 01) { | - | - |
| Antenna Grouping Index | 3 | Indicates the index of antenna grouping. See 8.4.8.3.4 and 8.4.8.3.5 if((Matrix_indicator == 00) 000~010 = 0b101110~0b110000 in Table 391 else 000~101 = 0b110001~0b110110 in Table 391 |
| Reserved | 1 | Shall be set to 0 |
| } elseif (MIMO mode == 10) { | - |  |
| Num_stream | 2 | Indicates the number of streams in Table 496 for 3 Tx and Table 497 for 4 Tx. |
| Antenna Selection Index | 3 | Indicates the index of antenna selection. See 8.4.8.3.4 and 8.4.8.3.5 000~110 = 0b110000~0b110101 in Table 392 |
| Reserved | 3 | Shall be set to 0 |
| } elseif (MIMO mode == 11) { | - |  |
| Num_stream | 2 | Indicates the number of streams |
| Codebook Precoding Index | 6 | Indicates the index of precoding matrix W in the codebook (see 8.4.8.3.6) |
| } | - |  |
| } | - |  |
| Padding | var | Padding to Nibble; shall be set to zero. |
| } | - | - |

[Modify the existing text in Table 430 in Page 892 of P802.16Rev2/D5 as shown below]

Table 430 –MIMO UL Chase HARQ Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_UL_Chase_HARQ_Sub-Burst_IE() { | - | - |
| MU Indicator | 1 | Indicates whether this UL burst is intended for multiple SS |
| Dedicated MIMO UL Control Indicator | 1 | - |
| ACK disable | 1 | When ACK Disable == 1, the allocated sub-burst does not require an ACK to be transmitted by the BS in the HARQ ACK BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the sub-burst in the HARQ ACK BITMAP. For TDD SS, for the burst, MS shall not peform HARQ retransmission and ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if(MU indicator == 0){ | - | - |
| Reserved | 1 | Shall be set to 0 |
| RCID IE() | variable | - |
| if(Dedicated MIMO UL Control Indicator == 1) { | - | - |
| Dedicated MIMO UL Control IE() | variable | - |
| } | - | - |
| }else { | - | - |
| Matrix | 1 | Indicates transmission matrix (see 8.4.8) for MS with dual TX antennas 0=Matrix A 1=Matrix B Ignored by MS with single TX antenna |
| } | - | - |
| Duration | 10 | - |
| Reserved | 2 | Shall be set to 0 |
| for(i=0;i<N_layer;i++){ | - | N_layer = 2 when MU Indicator is set to 1. Otherwise, its value shall be delivered in Dedicated MIMO UL Control IE(). |
| if(MU indicator == 1){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| UIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| if (ACK Disable == 0) { | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| Reserved | 1 | Shall be set to 0 |
| } else { | - | - |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| } | - | - |

[Modify the existing text in Table 431 in Page 893 of P802.16Rev2/D5 as shown below]

Table 431 –MIMO UL IR HARQ Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_UL_IR_HARQ_Sub-Burst_IE() { | - | - |
| MU Indicator | 1 | Indicates whether this UL burst is intended for multiple SSs |
| Dedicated MIMO UL Control Indicator | 1 | - |
| ACK disable | 1 | When ACK Disable == 1, the allocated sub-burst does not require an ACK to be transmitted by the BS in the HARQ ACK BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the sub-burst in the HARQ ACK BITMAP. For TDD SS, for the burst, MS shall not peform HARQ retransmission and ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if(MU indicator == 0){ | - | - |
| Reserved | 1 | Shall be set to 0 |
| RCID IE() | variable | - |
| if(Dedicated MIMO UL Control Indicator == 1) { | - | - |
| Dedicated MIMO UL Control IE() | variable | - |
| } | - | - |
| }else { | - | - |
| Matrix | 1 | Indicates transmission matrix (see 8.4.8) for MS with dual TX antennas 0=Matrix A 1=Matrix B Ignored by MS with single TX antenna |
| } | - | - |
| NSCH | 4 | - |
| for(i=0;i<N_layer;i++){ | - | - |
| if(MU indicator == 1){ | - | - |
| RCID IE() | variable | - |
| } | - | - |
| NEP | 4 | - |
| if (ACK Disable == 0) { | - | - |
| SPID | 2 | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| Reserved | 1 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| } | - | - |

[Modify the existing text in Table 432 in Page 895 of P802.16Rev2/D5 as shown below]

Table 432 –MIMO UL IR HARQ for CC Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO UL IR HARQ for CC Sub-Burst IE() { | - | - |
| MU Indicator | 1 | Indicates whether this UL burst is intended for multiple SS |
| Dedicated MIMO UL Control Indicator | 1 | - |
| ACK disable | 1 | When ACK Disable == 1, the allocated sub-burst does not require an ACK to be transmitted by the BS in the HARQ ACK BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the sub-burst in the HARQ ACK BITMAP. For TDD SS, for the burst, MS shall not peform HARQ retransmission and ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (MU indicator == 0) { | - | - |
| Reserved | 1 | Shall be set to 0 |
| RCID IE() | variable | - |
| if (Dedicated MIMO UL Control Indicator == 1) { | - | - |
| Dedicated MIMO UL Control IE() | variable | - |
| } | - | - |
| }else { | - | - |
| Matrix | 1 | Indicates transmission matrix (see 8.4.8) for MS with dual TX antennas 0=Matrix A 1=Matrix B Ignored by MS with single TX antenna |
| } | - | - |
| Duration | 10 | - |
| Reserved | 2 | Shall be set to 0 |
| for(i=0;i<N_layer;i++) { | - | - |
| if (MU indicator == 1) { | - | - |
| RCID IE() | variable | - |
| } | - | - |
| UIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00 - No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| if (ACK Disable == 0) { | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| SPID | 2 | - |
| Reserved | 3 | Shall be set to 0 |
| } else { | - | - |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| } | - | - |
| } | - | - |

[Modify the existing text in Table 433 in Page 895 of P802.16Rev2/D5 as shown below]

Table 433 –MIMO UL STC HARQ Subburst IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MIMO_UL_STC_HARQ_Sub-Burst_IE() { | - | - |
| Tx count | 2 | 0b00: Initial transmission 0b01: Odd transmission 0b10: Even transmission 0b11: Reserved |
| Duration | 10 | - |
| Sub-burst offset indication | 1 | Indicates the inclusion of sub-burst offset |
| ACK disable | 1 | When ACK Disable == 1, the allocated sub-burst does not require an ACK to be transmitted by the BS in the HARQ ACK BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the sub-burst in the HARQ ACK BITMAP. For TDD SS, for the burst, MS shall not peform HARQ retransmission and ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| Reserved | 2 | Shall be set to zero |
| if (sub-burst offset indication == 1) { | - | - |
| Sub-burst offset | 8 | Offset in slots with respect to the previous sub-burst defined in this data region. If this is the first sub-burst within the data region, this offset is with respect to slot 0 of the data region. |
| } | - | - |
| RCID_IE() | variable | - |
| ACK disable | 1 | When ACK Disable == 1, the allocated sub-burst does not require an ACK to be transmitted by the BS in the HARQ ACK BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the sub-burst in the HARQ ACK BITMAP. For TDD SS, for the burst, MS shall not peform HARQ retransmission and ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. For FDD SS, the BS may set the ACID to a value other than 0 if that ACID is listed in the Aggregated HARQ Channels TLV. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (Tx count == 00) { | - | - |
| UIUC | 4 | - |
| Repetition Coding Informatsion | 2 | 0b00 - No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| Reserved | 2 | Shall be set to 0 |
| } | - | - |
| if (ACK Disable == 0) { | - | - |
| ACID | 4 | - |
| } | - | - |
| } | - | - |