| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Update of examples of MS’s successful re-entry registration notifications. |  |
| Date Submitted | 2008-06-18 |  |
| Source(s) | Richard van Leeuwen Motorola | rvanleeuwen@motorola.com |
| Re: | IEEE802.16Rev2/D5 |  |
| Abstract | Update of examples of MS’s successful re-entry registration notifications. |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Update of examples of MS’s successful re-entry registration notifications

Richard van Leeuwen

Motorola

# Comments

For optimized Handover, sub-clause 6.3.22.2.7 specifies that if optimization bit #12 is set, the MS may use any uniquely identifiable indication as a notification of Handover completion.

- The list of examples is not complete. More specifically, this list does not include MAC signaling header type I (6.3.2.1.2.1) like BR with UL Tx power report. MAC signaling headers type I, all include a CID field through which the target BS can unique identify the MS.

- Sub-clause 6.3.6.1 specifies: After HO, the MS shall make an aggregated bandwidth request before making an incremental bandwidth request. To satisfy this requirement, the list of examples in 6.3.22.2.7 should not include incremental bandwidth requests unequal to zero.

# Suggested remedy

- Add MAC signaling header type I to the list of examples

- Remove incremental bandwidth requests unequal to zero

- Correct typo in last example

# Text Changes

Change the text in sub-clause 6.3.22.2.7 as follows:

6.3.22.2.7 Network entry/reentry

…

When optimization bit #12 is set, the MS may use any uniquely identifiable indication as notification of

MS’s successful re-entry registration, following are examples of such indications:

- MS transmits data in unsolicited UL grant by the target BS (i.e., MS has pending UL data) using newly assigned transport CID.

- MS transmits Bandwidth request header of type 0b000 or 0b001 with BR per desired BW when MS has pending UL data using newly assigned transport CID.

- MS transmits Bandwidth request header of type 0b000 or 0b001 with BR=0 when MS has no pending UL data using newly assigned basic CID.

- MS transmits any MAC signaling header type I with its CID field set to a newly assigned transport or basic CID and (if included) its incremental BR field set to 0.

- MS transmits HARQ ACK using the ACKCH slot assigned by the target BS.

- MS transmits CQI code using the CQICH slot assigned by the target BS.