| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Version Reporting (Section 11.1.3) and Interoperability Support |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | Dan Gal Alcatel-Lucent Whippany, New Jersey | Voice: +1 973 428 7734 E-mail: dgal@alcatel-lucent.com |
| Re: | LB#26d on IEEE P802.16 Rev2/D5 |  |
| Abstract | MS/BS implementations conformant to different IEEE 802.16 Standard versions (conveyed in TLV 148) may be deployed and try to operate concurrently in the same cell and need to be supported in a predictable and more reliable manner. This contribution suggests appropriate text changes to section 11.1.3. |  |
| Purpose | Improve the current provisions of the balloted draft Rev2/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Version Reporting (Section 11.1.3) and TLV148 Problems

Dan Gal, Alcatel-Lucent

# Introduction

##### Please refer to section 11.1.3 (page 1153) of P802.16-Rev2/D5

The SS-BS interoperability facilitation by the MAC version exchange is too lenient where the respective MAC versions are different. In fact, the standard provides for a much more reliable network entry protocol – the capabilities exchange – to assess the BS’s ability to support an SS’s (or the SS ability to interoperate with a BS) conforming to a given Standard version.

Thus, to avoid confusion and in order to more accurately describe the actual process that takes place at network entry, the following text changes are recommended.

# Proposed text changes

11.1.3 MAC version encoding

This parameter specifies the version of IEEE 802.16 Standard to which the message originator conforms.

If the MAC version values exchanged between a BS and SS during network entry differ such that the BS version is greater than the SS version, the SS may attempt to perform normal operations, primarily relying on the capability exchange mechanism utilized at network entry. The BS may attempt to communicate with the SS per the version specified by the SS and per the common negotiated capabilities, or if the BS determines that it cannot support the SS due to lack of or conflicted capabilities, it may decline to interoperate with the SS.

If the MAC version values exchanged between a BS and SS during network entry differ such that the BS version is smaller than the SS version, the BS may attempt to perform normal operations, primarily relying on the capability exchange mechanism utilized at network entry. The SS may attempt to communicate with the BS per the version specified by the BS, or, if the SS determines that it cannot be supported by the BS due to lack of or conflicted capabilities, it may decline to interoperate with the BS.