| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ Reordering Timer |  |
| Date Submitted | 2008-06-24 |  |
| Source(s) | Mark Marsan and Hua Xu Motorola Inc. | Voice: 847-632-2176 E-mail: Hua.Xu@motorola.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #26d |  |
| Abstract | This document adds a TLV for HARQ reordering timer. |  |
| Purpose | Review and adapt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ Reordering Timer TLV

Mark Marsan and Hua Xu

Motorola Inc.

# Introduction

HARQ reordering capability can be enabled using PDU SN.  Once a missing PDU is identified by the receiver, a timer must be used by the receiver to prevent it from waiting forever for the missing PDU.   At timeout, the receiver forwards all the PDUs that have received in order to the upper layer.  This timer should be inline with the projected PDU error recovery time and therefore needs to be negotiated between the transmitter and receiver.

A TLV is added for this purpose.

# Proposed Text

Modify Table 598 on page 1274 line 13 as shown below.

Table 598 – Service flow encodings (continued)

| Type | Parameter |
| --- | --- |
| 35 | Global Service Class Name |
| 36 | Reserved HARQ Reordering Timer |
| 37 | SN Feedback Enabled |

Add a new section after section 11.13.36 on page 1302 line 59 as shown below.

# 11.13.xx  HARQ Reordering Timer

This TLV is valid only when the HARQ reordering is enabled.  It specifies the maximum number of frames that the receiver shall wait for a missing PDU.  A timer is set for each identified missing PDU and disabled when the missing PDU is received.  The timer shall start from the frame that a missing PDU is identified by the receiver.  Once the timer expires, the receiver sends the PDUs that are received in order to the upper layer.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].36 | 1 | Timer value in number of frames | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) DSA-REQ DSA-RSP |