| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ State Transfer During HO |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Mark Marsan and Hua Xu Motorola Inc. | Voice: 847-632-2176 E-mail: Hua.Xu@motorola.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #26d |  |
| Abstract | This document modify the HO Process Optimization TLV to enable HARQ State Transfer during HO. |  |
| Purpose | Review and adapt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ State Transfer During HO

Mark Marsan and Hua Xu

Motorola Inc.

# Introduction

Currently only ARQ state transfer is available in the HO Process Optimization TLV.  Since ARQ may not be enabled for some traffic, it is necessary to have both options (ARQ state transfer and HARQ state transfer) available to the BS.

# Proposed Text

Modify Table 576 on page 1194 line 50 as shown below.

Table 576 – RNG-RSP message encodings (continued)

| Name | Type (1 byte) | Length | Value (variable-length) | PHY scope |
| --- | --- | --- | --- | --- |
| HO Process Optimization | 21 | 2 | For each bit location, a value of “0” indicates the associated re-entry management messages shall be required a value of “1” indicated the re-entry management message should be omitted. Bit #0: Bit #1: … Bit #13: #14-#15: Reserved Bit #14: If bit #14 = 1, Full service and operational HARQ state transfer or sharing between Servicing BS and Target BS (All static and dynamic context, e.g., HARQ windows, timers, counters, state machines, etc.) Bit #15: Reserved. | All |
| HO ID | 22 | 1 | Identifier assigned by the Target BS for use in initial ranging during MS HO to it (see 6.3.21.5) | All |