| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for DCD/UCD TLV Handling |  |
| Date Submitted | 2008-07-14 |  |
| Source(s) | Zhifeng (Jeff) Tao, Raymond Yim, Jinyun Zhang Mitsubishi Electric Research Lab Toshiyuki Kuze, Kentaro Sawa Mitsubishi Electric Corp | Voice: 617-621-{7557, 7597, 7595} Fax: 617-621-7550 {tao, yim, jzhang}@merl.com Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | IEEE802.16Rev2/D5 |  |
| Abstract | Clarification for DCD/UCD TLV handling. |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification for DCD/UCD TLV Handling

Zhifeng (Jeff) Tao, Raymond Yim, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze, Kentaro Sawa

Mitsubishi Electric Corp

# Background

Message parameters in DCD/UCD may be encoded in a TLV form.  If an SS receives a DCD/UCD message, and finds some TLV contained therein unknown or unexpected, the SS shall ignore this particular TLV and move to the rest of DCD/UCD message.  In order to establish a consistent understanding of the standard, it is beneficial to clarify the exception handling behavior as proposed below.

# Proposed Text Change

[Update subclause 6.3.2.3.1 as follow]

6.3.2.3.1 DCD (DL channel descriptor) message

…

The message parameters following the Configuration Change Count shall be encoded in a TLV form (see 11.4).  If an SS inspects a TLV in the DCD message and finds it unknown or its value unexpected, the SS shall ignore and skip over this TLV.  All channel encodings (see 11.4.1) shall appear first before the Downlink_Burst_Profile encodings

…

[Update subclause 6.3.2.3.3 as follow]

6.3.2.3.3 UCD (UL channel descriptor) message

…

To provide for flexibility, the remaining message parameters shall be encoded in a TLV form (see 11.3).  If an SS inspects a TLV in the UCD message and finds it unknown or its value unexpected, the SS shall ignore and skip over this TLV.  All Channel encodings (see 11.3.1) shall appear first before the Uplink_Burst_Profile encodings.

…