| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Collaborative Spatial Multiplexing Pilot Pattern for Two Dual Tx Antenna SSs in UL AMC zone |  |
| Date Submitted | 6/12/2008 |  |
| Source(s) | Gokhan Korkmaz, Dave Pechner, Hemabh Shekhar, Arvind Raghavan. ArrayComm LLC | Voice: +1-408-9521856 E-mail: gkorkmaz@arraycomm.com dpechner@arraycomm.com hshekhar@arraycomm.com arvind@arraycomm.com |
| Re: | LB26d – IEEE802.16 REV2/D5. |  |
| Abstract | This contribution proposes pilot pattern for 2 dual antenna SSs in AMC zone doing collaborative spatial multiplexing . |  |
| Purpose | Adopt the proposed text changes into IEEE802.16 REV2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Collaborative Spatial Multiplexing Pilot Pattern for 2 Dual Tx Antenna SSs in UL AMC zone

Gokhan Korkmaz, Dave Pechner, Hemabh Shekhar, Arvind Raghavan

ArrayComm LLC

Purpose

Although there is a collaborative spatial multiplexing pilot pattern defined for 2 dual antenna SSs in UL PUSC zone, a corresponding pilot pattern is not defined in UL AMC zone.  This contribution clarifies what the pilot pattern and data mapping should be in the case of UL CSM for AMC with two users each with two transmit antennas.  The proposed pilot locations are identical to those used for single antenna UL AMC CSM.  The only modification is that some pilot values for the second antenna are negated.  This contribution also clarifies that Matrix A encoding (same as specified for UL PUSC 2Tx CSM) shall be used for UL AMC CSM from SS devices with two Tx antennas.

Note that

- Each SS shall transmit different pilot values from each antenna sharing the same pilot positions.

- Each SS uses pilot positions according to the UL CSM pilot pattern defined for a single transmit antenna. Note that these pilot positions are the same for UL Matrix A, UL Matrix B and DL AMC STC.

- The pilot values for the first antenna are identical to those for single antenna UL CSM.  The pilot values from the second antenna are the same as those for the first antenna with the exception that half of the pilots are negated.

The proposed pilot pattern is shown in Figure y

[+1, -1]

NULL

[+1, +1]

NULL

NULL

[+1, -1]

NULL

[+1, +1]

NULL

[+1,-1]

[+1, +1]

NULL

[+1, -1]

NULL

[+1, +1]

NULL

[+1, -1]

NULL

NULL

[+1, +1]

NULL

[+1,-1]

NULL

[+1, +1]

Pilot patterns [A,B] belonging to the first MS    Pilot patterns [C,D] belonging to the second MS

Figure y - Collaborative Spatial Multiplexing Pilot Pattern for 2 Dual Antenna SSs in AMC zone

Text Changes

[ Insert the following text at the end of section 8.4.8.4.2 Allocation of data subchannels on page 1046 ]

When collaborative spatial multiplexing is performed by 2 MSs with dual Tx antennas, the data mapping rule shall be identical to the data mapping rule in Matrix A.

[ Insert the following text and the figure at the end of section 8.4.8.4.1 Allocation of pilot subcarriers on page 1045 ]

When two dual Tx antenna MSs perform collaborative spatial multiplexing on the same subchannel, one MS shall use the pilot pattern A, B and the other SS shall use the pilot pattern C, D. Pilot patterns are illustrated in Figure x. Note that pilot polarity for each pattern is given in brackets, and the first pilot pattern shall be transmitted by one antenna, and the second pilot pattern transmitted by the other antenna..

[+1,-1]

Null

[+1,+1]

Null

[+1,-1]

Null

Null

[+1,+1]

Null

[+1,-1]

Null

[+1,+1]

[+1,-1]

Null

[+1,+1]

Null

Null

[+1,-1]

Null

[+1,+1]

Null

[+1,-1]

[+1,+1]

Null

5

4

3

2

1

0

5

4

3

2

1

0

symbol

symbol

0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

subcarrier

Pilot pattern [C, D]

Pilot pattern [A, B]

Figure x – Uplink pilot allocation for 2 dual antenna MSs performing collaborative spatial multiplexing