| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Pilot Structure for Uplink Using STC |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Louay Jalloul, Beceem | jalloul@beceem.com |
| Re: | Letter Ballot 26d |  |
| Abstract | We propose |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Pilot Structure for Uplink Using STC

Louay Jalloul, Beceem

# Motivation

Collaborative spatial multiplexing (CSM) is a method defined in the 16e standard to increase the uplink system throughput. In pilots in CSM are shared between the two users or two antennas. Since there is timing and frequency offset errors between the SS and the BS, the BS receiver must correct for these impairments. These timing and frequency errors translate to phase rolls along the frequency and time dimension. The problem with the current pilot structure for UL CSM is that the BS receiver can’t ascertain whether the phase roll was due to a timing error or a frequency error.

In this contribution, we propose new pilot patterns for UL STC that can help the BS receiver resolve the phase roll due to timing and frequency errors using a simple receiver.

# New Pilot Structure for Uplink Using STC

The new pilot patterns are shown in figure below. As seen from the figure below, the pilots of a given user are rotated from tile to tile in a given subchannel. Using this pilot pattern, a simple receiver can be constructed at the BS (see the details in the companion presentation S80216mait-08_260.ppt) to separate the phase roll contribution from each of timing and frequency errors.

Figure 1: New pilot pattern for UL STC.

# Performance Analysis and Simulations

See companion slides S80216mait-08_260.ppt. The results show that there is a significant loss from timing and frequency errors while using the current pilot patterns (in some cases causing an error floor above 10% BLER). The link performance results with the new pilot shows no loss relative to the case when there is no timing or frequency errors.

# Conclusion

We recommend adopting the newly proposed pilot structure for uplink.

# Text Changes

[Add the following text to Rev2/D5 in section 8.4.1.8.15, page 1022, line 16]

The s-th (s = 0,1) subscriber in the uplink MIMO CSM pair shall use tile structure t (t = 0,1) on the k-th tile, where t = mod(s+k,2) .

[Replace Figure 275 in Rev2/D5 in section  8.4.1.8.15, page 1022] with the figure below

Figure  275 –Mapping of data subcarriers in STTD mode.

tile structure 0

tile structure 1