| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Service Provider List Provisioning |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d, on P802.16Rev2/D5 |  |
| Abstract | This document proposes adding wmanIf2mBsNetworkServiceProviderTable to wmanIf2mBsConfiguration to enable the provisioning of Network Service Provider list. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Network Service Provider List Provisioning

Joey Chou

Intel

# Introduction

This document proposes adding wmanIf2mBsNetworkServiceProviderTable to wmanIf2mBsConfiguration to enable the provisioning of Network Service Provider list.

# Proposed Text

# 2.1	MIB Module Description

13.1.4.1.1 wmanIf2mBsConfiguration

Figure 330 shows the structure of wmanIf2mBsConfiguration subtree.

[Replaced Figure 330 as the following]

Figure 330—wmanIf2mBsConfiguration structure

[Add subclause 11.1.4.1.1.7]

# 13.1.4.1.1.7 wmanIf2mBsNetworkServiceProviderTable

# wmanIf2mBsNetworkServiceProviderTable contains the list of NetworkService Provider to be sent by SBC-RSP or broadcast by SII-ADV message.

# 2.2	ASN.1 Notations – wmanIf2BsCm

It proposes the following new code be added to the wmanIf2mBsConfiguration.

WmanIf2mNspId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"24-bit Network Service Provider Identifiers "

REFERENCE

"Subclause 11.1.11.1"

SYNTAX      OCTET STRING (SIZE(3))

--

-- wmanIf2mBsNetworkServiceProviderTable

--

wmanIf2mBsNetworkServiceProviderTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNetworkServiceProviderEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the list of NetworkService Provider to

be sent by SBC-RSP or broadcast by SII-ADV message."

REFERENCE

"Subclause 11.1.11"

::= { wmanIf2mBsConfiguration 7 }

wmanIf2mBsNetworkServiceProviderEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNetworkServiceProviderEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex, wmanIf2mBsNspListIndex }

::= { wmanIf2mBsNetworkServiceProviderTable 1 }

WmanIf2mBsNetworkServiceProviderEntry ::= SEQUENCE {

wmanIf2mBsNspListIndex                  Integer32,

wmanIf2mBsNspIdentifier                 WmanIf2mNspId,

wmanIf2mBsVerboseNspNameLength          Integer32,

wmanIf2mBsVerboseNspName                OCTET STRING,

wmanIf2mBsNspRowStatus                  RowStatus}

wmanIf2mBsNspListIndex OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Index to NSP list."

::= { wmanIf2mBsNetworkServiceProviderEntry 1 }

wmanIf2mBsNspIdentifier OBJECT-TYPE

SYNTAX      WmanIf2mNspId

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Network Service Provider identifier."

::= { wmanIf2mBsNetworkServiceProviderEntry 2 }

wmanIf2mBsVerboseNspNameLength OBJECT-TYPE

SYNTAX      Integer32 (0 .. 255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Length of the Verbose NSP Name."

::= { wmanIf2mBsNetworkServiceProviderEntry 3 }

wmanIf2mBsVerboseNspName OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (255))

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The verbose name of the NSP."

::= { wmanIf2mBsNetworkServiceProviderEntry 4 }

wmanIf2mBsNspRowStatus OBJECT-TYPE

SYNTAX          RowStatus

MAX-ACCESS      read-create

STATUS          current

DESCRIPTION

"This object is used to ensure that the write, create,

delete operation to multiple columns is guaranteed to be

treated as atomic operation by agent."

::= { wmanIf2mBsNetworkServiceProviderEntry 5 }