| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Indicator in Persistent Allocation |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Changyoon Oh, Samsung Electronics Co., LTD | Voice: +82-31-279-7529 E-mail: changyoon.oh@samsung.com changyoon.oh@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Ballot #26d |  |
| Abstract | Clarification and text change in error recovery procedure of persistent allocation are proposed. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Indicator in Persistent Allocation

Changyoon Oh

Samsung Electronics

# Introduction

Error Recovery Procedure in Persistent allocation is clarified. More specifically, the use of Change Indicator is clarified. Appropriate text change is proposed.

# Proposed Text

[Modify the section 6.3.27.1.8.1 & 6.3.27.1.8.2 on page 489, as follows]

6.3.27.1.8.1 Downlink Persistent Allocation Error Recovery

The MS that fail to decode the DL-MAP in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.289) if any one of the following conditions is true in frame K + ap:

- Condition 1. The MS receives a Persistent HARQ DL MAP IE for the assigned Persistent Region ID with the allocation flag set to 1, which includes the MS’s RCID.  This is a new persistent allocation which replaces the previous persistent allocation for the assigned Persistent Region ID.

- Condition 2. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 0.

- Condition 3. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the Retransmission Flag of at least one  sub-burst IE is set to 1

- Condition 4. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the HARQ region definition for the assigned Persistent Region ID is different than the stored HARQ region definition (see Section 6.3.27.1.6.1).

Otherwise, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and shall stop using terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

The MS that fails to decode the DL MAP in frame K  shall stop persistent allocation in frame K+ Allocation Period (ap),  where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.29) if the following condition is true in frame K + ap:

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1.

The MS shall continue Persistent Allocation unless the MS receives deallocation of Persistent HARQ DL MAP IE  in frame K+ap.

Once the MS  stops the Persistent Allocation, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

6.3.27.1.8.2 Uplink Persistent Allocation Error Recovery

The MS that fail to decode the UL-MAP in a frame(frameMAP), which is relevant to a frame in which it has a persistent UL resource allocation (frame K), shall resume using the persistent allocation in the frame relevant to frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true in frameMAP + ap:

- Condiiton 1. The MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID. This is a new persistent allocation which replaces the previous persistent allocation for the assigned Persistent Region ID.

- Condiiton 2. The MS receives a Persistent HARQ UL MAP IE for the assigned Persistent Region ID, which has the Change Indicator for the assigned Persistent Region ID set to 0.

- Condiiton 3. The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the Retransmission Flag of at least one  sub-burst IE is set to 1.

- Condition 4. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the HARQ region definition for the assigned Persistent Region ID is different than the stored HARQ region definition (see Section 6.3.27.1.6.2).

Otherwise, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and shall stop using the persistent assignment including the data allocation, and the MAP NACK allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and shall stop using  the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

The MS that fails to decode the UL MAP in frame K  shall stop persistent allocation in frame K+ Allocation Period (ap),  where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.4.30) if the following condition is true in frame K + ap:

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1.

The MS shall continue Persistent Allocation unless the MS receives deallocation of Persistent HARQ UL MAP IE  in frame K+ap.

Once the MS  stops the Persistent Allocation, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

[Modify the section 6.3.27.1.5 on page 487, as follows]

6.3.27.1.5   Change Indicator

A Change Indicator for each HARQ Region shall be included in the Persistent HARQ DL MAP IE.
A Change Indicator for each HARQ Region shall be included in the Persistent HARQ UL MAP IE.

Change indicator in frame K shall be used by the MSs who failed to receive the DL-MAP or UL-MAP in frame K-ap to decide if they can resume using their persistent allocation in frame K.

MS who failed to receive the DL-MAP or UL-MAP in frame K-ap, shall interpret the change indicator= '0' as there were no changes, such as deallocation, Persistent HARQ Region change in the Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE  in K-ap.

MS who failed to receive the DL-MAP or UL-MAP in frame K-ap,  shall interpret the change indicator='1' as there were change(s) in the Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE  in K-ap.

The base station sets the change indicator to 0 in frame K in case there are sufficient evidences that all the MSs that involved in the changes in frame K-ap have received the change information.

[Modify the section 6.3.27.1.7 on page 488, as follows]

6.3.27.1.7 Persistent Region ID

The resources for persistent allocations are relative to the boundary of a HARQ region, which is identified by a Persistent Region ID.  The position of a mobile station’s persistent resource shall be determined based on the HARQ region definition and the slot offset, which is assigned in the Persistent HARQ DL MAP IE or the Persistent HARQ UL MAP IE.   The slot offset is counted from the lowest numbered OFDMA symbol in the lowest numbered subchannel of HARQ Region.  This operation applies to both DL and UL in TDD and H-FDD operation