| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Indicator in Persistent Allocation |  |
| Date Submitted | 2008-06-23 |  |
| Source(s) | Changyoon Oh, Samsung Electronics Co., LTD | Voice: +82-31-279-7529 E-mail: changyoon.oh@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Ballot #26d |  |
| Abstract | Clarification and text change in error recovery procedure of persistent allocation are proposed. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Indicator in Persistent Allocation

Changyoon Oh

Samsung Electronics

# Introduction

Error Recovery Procedure in Persistent allocation is clarified. More specifically, the use of Change Indicator is clarified. Appropriate text change is proposed.

In addition, MAP ACK/NACK in Table 536 (Normalized C/I per modulation) is clarified.

Table 536 Normalized C/N per modulation in Rev2/D5 includes Normalized C/N values for MAP ACK/NACK Channel, which is used for Persistent Allocation acknowledgement. More specifically, two separate lines are added to the Table 536 to specify Normalized C/N values for MAP ACK/NACK Channel.   This causes the backward compatibility issue in Normalized C/N override TLV (TLV #175) and Normalized C/N override 2 TLV (TLV #177), which are used to override Normalized C/N values in Table 536. Because two values are added in the Table 536 and the length of TLV (TLV #175 and TLV #177)  are not enough to specify those additional two values.

# Proposed Text

[Remedy #1]:

[Modify the section 6.3.27.1.8.1 & 6.3.27.1.8.2 on page 489, as follows]

6.3.27.1.8.1 Downlink Persistent Allocation Error Recovery

The MS that fail to decode the DL-MAP in frame K may resume using the persistent allocation in frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.289) if any one of the following conditions is true in frame K + ap:

- Condition 1. The MS receives a Persistent HARQ DL MAP IE for the assigned Persistent Region ID with the allocation flag set to 1, which includes the MS’s RCID.  This is a new persistent allocation which replaces the previous persistent allocation for the assigned Persistent Region ID.

- Condition 2. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 0.

- Condition 3. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the Retransmission Flag of at least one  sub-burst IE is set to 1

- Condition 4. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the HARQ region definition for the assigned Persistent Region ID is different than the stored HARQ region definition (see Section 6.3.27.1.6.1).

Otherwise, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

If the MS fails to decode the DL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL resource allocation (frame K) and fails to decode the DL-MAP in frameMAP + allocation period, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and shall stop using terminate the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

The MS that fails to decode the DL MAP in frame K  shall stop persistent allocation in frame K+ Allocation Period (ap),  where ap is a field of the Persistent HARQ DL MAP IE (Section 8.4.5.3.29) if the following condition is true in frame K + ap:

- The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1.

The MS shall continue Persistent Allocation unless the MS receives deallocation of Persistent HARQ DL MAP IE  in frame K+ap.

Once the MS  stops the Persistent Allocation, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

6.3.27.1.8.2 Uplink Persistent Allocation Error Recovery

The MS that fail to decode the UL-MAP in a frame(frameMAP), which is relevant to a frame in which it has a persistent UL resource allocation (frame K), shall resume using the persistent allocation in the frame relevant to frame K + Allocation Period (ap), where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.3.28) if any one of the following conditions is true in frameMAP + ap:

- Condiiton 1. The MS receives a Persistent HARQ UL MAP IE with the allocation flag set to 1, which includes the MS’s RCID. This is a new persistent allocation which replaces the previous persistent allocation for the assigned Persistent Region ID.

- Condiiton 2. The MS receives a Persistent HARQ UL MAP IE for the assigned Persistent Region ID, which has the Change Indicator for the assigned Persistent Region ID set to 0.

- Condiiton 3. The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the Retransmission Flag of at least one  sub-burst IE is set to 1.

- Condition 4. The MS receives a Persistent HARQ DL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1, and the HARQ region definition for the assigned Persistent Region ID is different than the stored HARQ region definition (see Section 6.3.27.1.6.2).

Otherwise, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and shall stop using the persistent assignment including the data allocation, and the MAP NACK allocation.

If the MS fails to decode the UL-MAP in a frame (frameMAP) which is relevant to a frame in which it has a persistent DL/UL resource allocation (frame K) and fails to decode the UL-MAP in frameMAP + allocation period, the MS shall transmit an indication to the BS and shall stop using  the persistent assignment including the data allocation and the MAP NACK allocation.  The allocation period is indicated in the subburst IE of the Persistent HARQ UL MAP IE.

The MS that fails to decode the UL MAP in frame K  shall stop persistent allocation in frame K+ Allocation Period (ap),  where ap is a field of the Persistent HARQ UL MAP IE (Section 8.4.5.4.30) if the following condition is true in frame K + ap:

- The MS receives a Persistent HARQ UL MAP IE, which has the Change Indicator for the assigned Persistent Region ID set to 1.

The MS shall continue Persistent Allocation unless the MS receives deallocation of Persistent HARQ UL MAP IE  in frame K+ap.

Once the MS  stops the Persistent Allocation, the MS should attempt to send the Persistent Allocation Error Event extended subheader to the BS and stop using the persistent assignment including the data allocation, the MAP NACK allocation, and the HARQ ACK channel allocation if any.

[Modify the section 6.3.27.1.5 on page 487, as follows]

6.3.27.1.5   Change Indicator

A Change Indicator for each HARQ Region shall be included in the Persistent HARQ DL MAP IE.
A Change Indicator for each HARQ Region shall be included in the Persistent HARQ UL MAP IE.

Change indicator in frame K shall be used by the MSs who failed to receive the DL-MAP or UL-MAP in frame K-ap to decide if they can resume using their persistent allocation in frame K.

MS who failed to receive the DL-MAP or UL-MAP in frame K-ap, shall interpret the change indicator= '0' as there were no changes, such as deallocation, Persistent HARQ Region change in the Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE  in K-ap.

MS who failed to receive the DL-MAP or UL-MAP in frame K-ap,  shall interpret the change indicator='1' as there were change(s) in the Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE  in K-ap.

The base station sets the change indicator to 0 in frame K in case there are sufficient evidences that all the MSs that involved in the changes in frame K-ap have received the change information.

[Remedy #2]:

[Modify the section 8.4.10.3.1 on page 1002, as follows]

Table 536—Normalized C/N per modulation

| Modulation/ FEC rate | Normalized C/N |
| --- | --- |
| ACK region | –3.0 |
| FAST FEEDBACK MAP ACK Channel MAP NACK Channel | 0 |
| MAP ACK Channel | 0.0 |
| MAP NACK Channel | 0.0 |
| CDMA code | 3 |
| QPSK 1/3 | 0.5 |
| QPSK 1/2 | 6 |
| QPSK 2/3 | 7.5 |
| QPSK 3/4 | 9 |
| 16-QAM-1/2 | 12 |
| 16-QAM-2/3 | 14.5 |
| 16-QAM-3/4 | 15 |
| 16-QAM-5/6 | 17.5 |
| 64-QAM-1/2 | 18 |
| 64-QAM-2/3 | 20 |
| 64-QAM-3/4 | 21 |
| 64-QAM-5/6 | 23 |