| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DL CINR reporting during Network Re-entry |  |
| Date Submitted | 2008-06-20 |  |
| Source(s) | Jaejeong Shim Youngsam Park Kanggyu Lee Hyeyeon Jeong Samsung Electronics Co., LTD. | Voice: +82 (31) 279-0191 E-mail:youngsam.park@samsung.com E-mail:kanggyu.lee@samsung.com E-mail: hyeyeon.jeong @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB #26d |  |
| Abstract | DL CINR reporting using RNG-REQ message |  |
| Purpose | Adopt for inclusion in 802.16 Revision 2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DL CINR reporting during Network Re-entry

Jaejeong Shim, Youngsam Park, Kanggyu Lee, Hyeyeon Jung
Samsung Electronics

# Problem Statement

BS decides DL MCS level based on the channel estimation reported by MS. MS reports DL channel quality information using REP-REQ/RSP messages exchange or CQI report. However, when MS conducts network re-entry to a BS(i.e. HO or reentry from idle mode), MS cannot report DL channel quality information before it exchanges REP-REQ/RSP messages with the BS or receives CQI channel allocation from the BS. Before MS reports DL channel quality information BS cannot apply DL AMC mechanism to MS. Therefore BS has to allocate DL bursts with the most robust MCS level, which decreases DL throughput. Meanwhile, if MS includes DL channel quality information in RNG-REQ message, it would be an efficient method for maintaining DL throughput before explicit REP-REQ/RSP or CQI Report.

# Proposed Text Changes