| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DL CINR reporting during Network Re-entry |  |
| Date Submitted | 2008-07-11 |  |
| Source(s) | Youngsam Park Kanggyu Lee Hyeyeon Jeong Jaejeong Shim Samsung* | Voice: +82 (31) 279-0191 E-mail:youngsam.park@samsung.com E-mail:kanggyu.lee@samsung.com E-mail: hyeyeon.jeong @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Revision 2 / Letter Ballot |  |
| Abstract | DL CINR reporting using RNG-REQ message |  |
| Purpose | Adopt for inclusion in 802.16 Revision 2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DL CINR reporting during Network Re-entry

Youngsam Park, Kanggyu Lee, Hyeyeon Jung, Jaejeong Shim
Samsung

# Problem Statement

BS decides DL MCS level based on the channel estimation information reported by MS. MS reports DL channel quality using REP-REQ/RSP messages exchange or CQI. However, when MS conducts network re-entry to a BS(i.e. HO or reentry from idle mode), MS cannot report DL channel quality before the MS exchanges REP-REQ/RSP messages with the BS or receives CQI channel allocation from the BS. Before MS reports DL channel quality information BS cannot apply an appropriate DL AMC mechanism to MS. Therefore BS has to allocate DL bursts with the most robust MCS level, which decreases DL throughput. Meanwhile, if MS includes DL channel quality information in RNG-REQ message, it would be an efficient method for maintaining DL throughput before explicit REP-REQ/RSP or CQI Report.

# Proposed Text Changes

(Add to to Table 569 – RNG-REQ message encodings, pages 1150)

| Name | Type(1bytes) | Length | Value |  |
| --- | --- | --- | --- | --- |
| Preamble Physical CINR Report | XX | 1 | preamble physical CINR |  |

(Change subclause 6.3.22.2.7, pages 443)

If an MS RNG-REQ includes a serving BSID and Ranging Purpose Indication TLV with Bit #0 set to 1, and

the target BS had not previously received MS information over the backbone network, then the target BS

may make an MS information request of the serving BS over the backbone network and the serving BS may

respond. Regardless of having received MS information from the serving BS, the target BS may request MS

information from another network entity via the backbone network.

An MS may report its DL CINR information to the target BS by including Preamble Physical CINR Report TLV in RNG-REQ message. The BS may allocate DL burst with appropriate MCS level using the reported DL CINR before it receives REP-RSP or CQI report from the MS.