| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Solution for DREG-REQ loss when BS initiates Idle Mode |  |
| Date Submitted | 2008-06-18 |  |
| Source(s) | Li Nan，Li Wang，Qu Hongyun， Wang Wenhuan ZTE Corporation | Voice: E-mail: li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d, on P802.16Rev2/D5 |  |
| Abstract | This contribution proposed a solution for lost of DREG-REQ message during BS initiated Idle Mode. |  |
| Purpose | To be discussed and accepted by TG maint. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Solution for DREG-REQ loss when BS initiates Idle Mode

Li Nan，Li Wang，

Qu Hongyun，Wang Wenhuan

ZTE Corporation

# Problem description

In the section 6.3.24.1 of the current 802.16e standard, the serving BS may signal for an MS to begin idle mode by sending a DREG-CMD message with action code 0x05 in unsolicited manner and start T46 timer as well as management resource holding timer at the same time. MS shall enter idle mode after it sends DREG-REQ message with the De-registration_Request Code parameter = 0x02 in response to the unsolicited DREG-CMD message.

However, once MS has entered idle mode it could not guarantee availability to any BS for DL traffic. That may cause the problem that the MS could not respond to the BS during retransmission of DREG-CMD due to the error delivery of DREG-REQ at the first time. In this case, the BS would transmit DREG-CMD messages repeatedly until DREG command retry count is exhausted and decide to delete this MS’s connection information at expiration of management resource holding timer. That is a waste of BS’s resource.

Proposed solutions

It is proposed that MS should set a MS-ENTER-IDLE-Timer for waiting a possible retransmission of DREG-CMD from BS after it sends a DREG-REQ. When MS receives DREG-CMD again before MS-ENTER-IDLE-Timer expires, it shall send a DREG-REQ again and check the ENTER-IDLE-Timer Retries.MS shall restart the MS-ENTER-IDLE-Timer and increase the ENTER-IDLE-Timer Retries by one when it resend a DREG-REQ. If the ENTER-IDLE-Timer Retries is exhausted，MS shall enter idle mode.

The default value of MS-ENTER-IDLE-Timer is recommended bigger than or equal to Timer T46. The maximum value of ENTER-IDLE-Timer Retries is recommended to three times.

Proposed content changed in standard

Add the following rows to Table 544:

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MS | MS-ENTER- IDLE-Timer | Time MS waits to enter idle Mode after it sends DREG-REQ with De-registration_Request_Code 0x02 in case of unsolicited idle mode initiation from BS. | - | - | - |
| MS | ENTER-IDLE-Timer Retries | Number of retries on MS-ENTER-IDLE-Timer | - | - | 3 |

Change the end of the first paragraph of section 6.3.24.1 as follows:

MS shall enter idle mode set a MS-ENTER-IDLE-Timer for waiting a possible retransmission of DREG-CMD from BS after it sends DREG-REQ message with the De-registration_Request Code parameter = 0x02 in response to the unsolicited DREG-CMD message with action code 0x05.

When MS receives DREG-CMD again before MS-ENTER-IDLE-Timer expires, it shall resend a DREG-REQ. MS shall restart the MS-ENTER-IDLE-Timer and increase the ENTER-IDLE-Timer Retries by one when it resend a DREG-REQ. If the ENTER-IDLE-Timer Retries is exhausted，MS shall enter idle mode.

# References

P802.16Rev2/D5 (June 2008)(Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005,IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007)