| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Dedicated MIMO DL Control IE |  |
| Date Submitted | 2008-06-21 |  |
| Source(s) | Mark Marsan Joe Schumacher Motorola | E-mail: mailto:j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot 26d |  |
| Abstract | Text for the Dedicated MIMO DL Control IE is ambiguous and can be interpreted in ways that are not interoperable. |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification for Dedicated MIMO control IE in HARQ DL MAP IEs

Joe Schumacher and Mark Marsan

Motorola

# Problem Overview

When making DL allocations in an STC zone using HARQ MAP IEs certain MIMO parameters are expected to change over time, such as MIMO Matrix.   The BS must use the Dedicated MIMO DL Control IE to indicate MIMO parameters for each sub-burst.  The Dedicated MIMO DL Control IE is discussed in section 8.4.5.3.21 of the standard.

When an MS receives a MIMO HARQ burst allocation with Dedicated MIMO DL Control Indicator set to 1, the MS shall store the information in Dedicated MIMO DL Control IE. When an MS receives a MIMO HARQ burst allocation with Dedicated MIMO DL Control Indicator is set to 0, the MS shall use the stored Dedicated MIMO DL Control information from the last burst allocation where this information was included.

The phrase “When an MS receives a MIMO HARQ burst allocation” is not specific enough. Does this mean any prior sub-burst to any CID or only sub-bursts with a CID that matches a CID assigned to the MS?  Also, the phrase “the MS shall store the information in Dedicated MIMO DL Control IE” is also not specific enough. How long does the MS store the information? Is it forever? Is it until the end of the frame? Is it until the end of the sub-burst IE?

Section 8.4.5.3.21 has been interpreted two different ways:

- Option 1: for all MSs – The Dedicated MIMO DL Control IE is used to define MIMO parameters for the current and later sub-bursts for all MSs during a sub-burst IE (e.g. MIMO DL Chase HARQ Sub-burst IE or MIMO DL IR HARQ Sub-burst IE or MIMO DL STC HARQ Sub-burst IE).  Once a Dedicated MIMO DL Control IE is present in a sub-burst, all later sub-bursts in the sub-burst IE use the same MIMO parameters defined by the Dedicated MIMO DL Control IE.

- Option 2: Per MS – If an MS decodes a HARQ sub-burst using a CID that is assigned to the MS, and the sub-burst contains a Dedicated MIMO DL Control IE, the MS retains the MIMO parameters for all its DL allocations in the current and subsequent frames until it encounters the next Dedicated MIMO DL Control IE that is addressed to one of the MS’s CIDs.

Option 2 has several issues.  The first problem is the interaction with the N_layer parameter.   N_layer is used for vertical encoding of MIMO bursts and can have values from 1 to 4.   The value of N_layer is defined in the Dedicated MIMO DL Control IE, which allows individual sub-bursts to use different values of N_layer.   Each sub-burst contains a loop that assigns DIUC, ACID and other parameters per layer so the length of the sub-burst is determined by the N_layer value.  Since each sub-burst does not have a length field, the MS must know the N_layer value not only for a sub-burst assigned to it but for all prior sub-bursts.  This is very simple for option 1 since the value of N_Layer is based on the information contained in that sub-burst IE.  Option 2 requires the MS to track all dedicated DL MIMO Control IEs not only for itself but for all allocations to other MSs in all prior frames.  This is not possible since the MS may not have been present in the BS when prior allocations were made (because of handover). Even if the MS was present, the memory required for storage of this information is quite excessive.   Therefore, option 2 is not a viable option.

A second issue with option 2 is the synchronization issue if the BS changes information (e.g MIMO Matrix) contained in the Dedicated MIMO DL Control IE and the MS cannot decode the DL-MAP.  For example, when the BS changes the MIMO matrix from A to B and the MS cannot decode the DL-MAP containing the Dedicated MIMO DL Control IE the MS will continue to decode allocations in later frames using MIMO Matrix A but the BS will encode using MIMO matrix B.  This results in a 100% failure rate.  This problem is not possible in Option 1 since the MIMO information is defined completely within a single frame and does not require any knowledge of MIMO information from previous frames.

# Proposed Text Changes

Modify section 8.4.5.3.21, page 757, line 14 of Rev2/d4 as indicated below:

When an MS receives a MIMO HARQ burst allocation with Dedicated MIMO DL Control Indicator set to 1, the MS shall store the information in Dedicated MIMO DL Control IE.  The information contained in the Dedicated MIMO DL Control IE  is used for all allocations to all MSs until the next occurrence of  the Dedicated MIMO DL control IE or until the end of the current sub-burst IE. When an MS receives a MIMO HARQ burst allocation with Dedicated MIMO DL Control Indicator is set to 0, the MS shall use the stored Dedicated MIMO DL Control information from the last burst allocation where this information was included.