| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MBS logical channel clarification |  |
| Date Submitted | 2008-07-08 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d, on P802.16Rev2/D5 |  |
| Abstract | This contribution proposes text and diagrams to clarify how a SS can receive MBS MSDU by showing the MBS frame structure. It adds the PSH (Packing Subheader) in MPDU to fix the hole in the standard. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MBS Logical Channel Clarification

Joey Chou

Intel

# Introduction

The use of Logical Channel ID (LCID) in a MBS is confusing. LCID is in the MBS MAP, but, not in MPDU or MSDU. The standard also has a hole, since it does not specify how a SS can separate and receive multiple MBS MSDUs that are embedded in a MBS MPDU.

This contribution proposes text and diagrams to clarify how a SS can receive MBS MSDU by showing the MBS frame structure. It adds the PSH (Packing Subheader) in MPDU to fix the hole in the standard.

This contribution proposes new ASN.1 code to wmanIf2mMib and wmanIf2fMib to provide MBS content ID provisioning.

# Proposed Text

2.1	Changes to Multicast and broadcast service (MBS)

6.3.23 Multicast and broadcast service (MBS)

[Page 473, Line 1]

Logical Channel IDs, which pairs with Multicast CID in the Extended MBS DATA IE, is allocated to each MBS Contents ID value in the order that it is included in the MBS Contents IDs TLV (11.13.37). As a result, an SS can receive multiple MBS messages for an MBS connection with different MBS contents distinguished by Logical Channel ID belonging to a Multicast CID. The BS shall allocate MBS SDUs in the order defined in the Extended MBS DATA IE.

Figure xx shows an example of MBS frame structure. Each MSDU inside the MBS MPDU shall have a PSH to enable a SS to receive the desired MBS content among multiple MSDU carried in a MBS MPDU.

Figure xx—Example of MBS Frame Structure

Table xx shows the example of MBS CID, MBS Content ID, and Logical Channel ID mapping. The binding of CID and MBS Content ID are created from DSA messages.

Table xx—Example of MBS CID, MBS Content ID, and LCID Mapping

  - Changes to wmanIf2mMib

wmanIf2mBsMbsContentTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsMbsContentEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the MBS contents IDs that are used to

distinguish the logical MBS connections within a MBS CID.

The provision of MBS content IDs will be used in DSA

message to create a multicast CID. The provision of logical

channel ID along with multicast CID will be used in the MBS

MAP message."

REFERENCE

"Subclause 11.13.37"

::= { wmanIf2mBsServiceFlow 6 }

wmanIf2mBsMbsContentEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsMbsContentEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2mBsMbsCid,

wmanIf2mBsMbsContentId }

::= { wmanIf2mBsMbsContentTable 1 }

WmanIf2mBsMbsContentEntry ::= SEQUENCE {

wmanIf2mBsMbsCid                        Integer32,

wmanIf2mBsMbsContentId                  Integer32,

wmanIf2mBsMbsLogicalChannelId           Integer32}

wmanIf2mBsMbsCid OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"MBS CID"

::= { wmanIf2mBsMbsContentEntry 1 }

wmanIf2mBsMbsContentId OBJECT-TYPE

SYNTAX      Integer32  (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"MBS content ID."

::= { wmanIf2mBsMbsContentEntry 2 }

wmanIf2mBsMbsLogicalChannelId OBJECT-TYPE

SYNTAX      Integer32  (0 .. 255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object pairs with Multicast CID in MBS_DATA_IE. It is

allocated to each MBS Contents IDs."

::= { wmanIf2mBsMbsContentEntry 3 }

  - Changes to wmanIf2fMib

wmanIf2fBsMbsContentTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2fBsMbsContentEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the MBS contents IDs that are used to

distinguish the logical MBS connections within a MBS CID.

The provision of MBS content IDs will be used in DSA

message to create a multicast CID. The provision of logical

channel ID along with multicast CID will be used in the MBS

MAP message."

REFERENCE

"Subclause 11.13.37"

::= { wmanIf2fBsMib 6 }

wmanIf2fBsMbsContentEntry OBJECT-TYPE

SYNTAX      WmanIf2fBsMbsContentEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2fBsMbsCid,

wmanIf2fBsMbsContentId }

::= { wmanIf2fBsMbsContentTable 1 }

WmanIf2fBsMbsContentEntry ::= SEQUENCE {

wmanIf2fBsMbsCid                        Integer32,

wmanIf2fBsMbsContentId                  Integer32,

wmanIf2fBsMbsLogicalChannelId           Integer32}

wmanIf2fBsMbsCid OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"MBS CID"

::= { wmanIf2fBsMbsContentEntry 1 }

wmanIf2fBsMbsContentId OBJECT-TYPE

SYNTAX      Integer32  (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"MBS content ID."

::= { wmanIf2fBsMbsContentEntry 2 }

wmanIf2fBsMbsLogicalChannelId OBJECT-TYPE

SYNTAX      Integer32  (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object pairs with Multicast CID in MBS_DATA_IE. It is

allocated to each MBS Contents IDs."

::= { wmanIf2fBsMbsContentEntry 3 }