| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MBS logical channel Provisioning |  |
| Date Submitted | 2008-07-17 |  |
| Source(s) | Joey Chou Intel Tricci So Jerry Chow ZTE | E-mail: joey.chou@intel.com E-mail: tso@zteusa.com E-mail: jchow@zteusa.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d, on P802.16Rev2/D5 |  |
| Abstract | This contribution proposes text and ASN.1 code to provide the provisioning of MBS content ID and logical channel ID for MBS. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MBS Logical Channel Provisioning

Joey Chou

Intel

# Introduction

This contribution proposes text and ASN.1 code to provide the provisioning of MBS content ID and logical channel ID for MBS .

# Proposed Text

2.1	Changes to wmanIf2BsCm

13.1.3.2 wmanIf2BsCm

[Add the following tables to Figure 323]

- wmanIf2BsMbsContentTable

- wmanIf2BsMbsServiceFlowTable

- wmanIf2BsMbsClassifierRuleTable,

- wmanIf2BsPhsRuleTable

- wmanIf2BsQoSProfileTable

- wmanIf2BsArqAttributeTable

[Insert the following paragraphs to subclause 13.1.3.2.9]

13.1.3.2.9 wmanIf2BsMbsContentTable

wmanIf2BsMbsContentTable contains the MBS contents IDs that are used to distinguish the logical MBS connections within a MBS CID.

13.1.3.2.10 wmanIf2BsMbsServiceFlowTable

wmanIf2mBsMbsServiceFlowTable contains the database for MBS service flows that include logical channel IDs.

13.1.3.2.11 wmanIf2BsMbsClassifierRuleTable

wmanIf2mBsMbsClassifierRuleTable contains packet classifier rules associated with MBS service flows.

13.1.3.2.12 wmanIf2BsPhsRuleTable

wmanIf2BsPhsRuleTable contains PHS rule dictionary entries.

13.1.3.2.13 wmanIf2BsQoSProfileTable

wmanIf2BsQoSProfileTable contains QoS profiles that are associated with service flows or CIDs.

13.1.3.2.14 wmanIf2BsArqAttributeTable

wmanIf2BsArqAttributeTable contains ARQ parameters that are associated with the Service Flows.

  - Changes to wmanIf2Mib

13.2.3 wmanIf2BsMib

[Add the following tables to wmanIf2Mib]

wmanIf2BsMbsContentTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMbsContentEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the MBS contents IDs that are used to

distinguish the logical MBS connections within a MBS CID.

The provision of MBS content IDs will be used in DSA

message to create a multicast CID. The provision of logical

channel ID along with multicast CID will be used in the MBS

MAP message."

REFERENCE

"Subclause 11.13.37"

::= { wmanIf2BsCm 9 }

wmanIf2BsMbsContentEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMbsContentEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2BsMbsCid,

wmanIf2BsMbsContentId }

::= { wmanIf2BsMbsContentTable 1 }

WmanIf2BsMbsContentEntry ::= SEQUENCE {

wmanIf2BsMbsContentId                   Integer32,

wmanIf2BsMbsLogicalChannelId            Integer32}

wmanIf2BsMbsContentId OBJECT-TYPE

SYNTAX      Integer32  (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"MBS content ID."

::= { wmanIf2BsMbsContentEntry 1 }

wmanIf2BsMbsLogicalChannelId OBJECT-TYPE

SYNTAX      Integer32  (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object pairs with Multicast CID in MBS_DATA_IE. It is

allocated to each MBS Contents IDs."

::= { wmanIf2BsMbsContentEntry 2 }

wmanIf2BsMbsServiceFlowTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMbsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the database for MBS service flows

that include logical channel IDs. For MBS service flows

that do not include Logical Channel IDs, the service flow

database is stored in wmanIf2BsServiceFlowTable."

REFERENCE

"Subclause 6.3.14"

::= { wmanIf2BsCm 10 }

wmanIf2BsMbsServiceFlowEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMbsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2BsSsSfId }

::= { wmanIf2BsMbsServiceFlowTable 1 }

WmanIf2BsMbsServiceFlowEntry ::= SEQUENCE {

wmanIf2BsSsSfId                         Unsigned32,

wmanIf2BsMbsProvGlobalServiceClass      WmanIf2TcGlobalSrvClass,

wmanIf2BsMbsAdmitGlobalServiceClass     WmanIf2TcGlobalSrvClass,

wmanIf2BsMbsActiveGlobalServiceClass    WmanIf2TcGlobalSrvClass,

wmanIf2BsMbsProvQoSProfileIndex         Integer32,

wmanIf2BsMbsAdmitQoSProfileIndex        Integer32,

wmanIf2BsMbsActiveQoSProfileIndex       Integer32,

wmanIf2BsMbsArqAttributeIndex           Integer32,

wmanIf2BsMbsServiceFlowState            WmanIf2TcSfState,

wmanIf2BsMbsCid                         Integer32,

wmanIf2BsMbsSfCsSpecification           WmanIf2TcCsType,

wmanIf2BsMbsSfReqTxPolicy               WmanIf2TcReqTxPolicy,

wmanIf2BsMbsSfTargetSaid                Integer32,

wmanIf2BsMbsSfEstablishTime             TimeStamp,

wmanIf2BsMbsSfTerminateTime             TimeStamp,

wmanIf2BsMbsSfFixedVsVariableSdu        WmanIf2TcSduType,

wmanIf2BsMbsSfFragmentSeqNumType        WmanIf2TcFsnType,

wmanIf2BsMbsSfMbsService                WmanIf2TcMbsType}

wmanIf2BsSsSfId OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow."

::= { wmanIf2BsMbsServiceFlowEntry 1 }

wmanIf2BsMbsProvGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2TcGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the ProvisionedQoSParamSet for this

service flow. When '0' is returned from reading this object

, it means either no global service class is defined, or

its Qos profile may be defined in

wmanIf2BsMbsProvQoSProfileIndex."

REFERENCE

"Subclause 6.3.14.4.1 Table 185"

::= { wmanIf2BsMbsServiceFlowEntry 2 }

wmanIf2BsMbsAdmitGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2TcGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the AdmittededQoSParamSet for this

service flow. When '0' is returned from reading this object

, it means either no global service class is defined, or

its Qos profile may be defined in

wmanIf2BsMbsAdmitQoSProfileIndex. AdmittededQoSParamSet is

a subset of ProvisionedQoSParamSet."

REFERENCE

"Subclause 6.3.14.4.1 Table 185"

::= { wmanIf2BsMbsServiceFlowEntry 3 }

wmanIf2BsMbsActiveGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2TcGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the ActiveQoSParamSet for this service

flow. When '0' is returned from reading this object, it

means either no global service class is defined, or its Qos

profile may be defined in

wmanIf2BsMbsActiveQoSProfileIndex. ActiveQoSParamSet is a

subset of AdmittededQoSParamSet."

REFERENCE

"Subclause 6.3.14.4.1 Table 185"

::= { wmanIf2BsMbsServiceFlowEntry 4 }

wmanIf2BsMbsProvQoSProfileIndex OBJECT-TYPE

SYNTAX      Integer32 (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2BsQoSProfileTable

that defines the ProvisionedQoSParamSet of a service flow.

If WmanIf2TcSfState = 'provisioned', then

ProvisionedQoSParamSet is the QoS profile for this service

flow. When '0' is returned from reading this object, it

means the QoS profile either is not defined, or is defined

in wmanIf2BsMbsProvQoSProfileIndex."

REFERENCE

"Subclause 6.3.13 and 6.3.14"

::= { wmanIf2BsMbsServiceFlowEntry 5 }

wmanIf2BsMbsAdmitQoSProfileIndex OBJECT-TYPE

SYNTAX      Integer32 (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2BsQoSProfileTable

that defines the AdmittedQoSParamSet of a service flow. If

WmanIf2TcSfState = 'admitted', then AdmittedQoSParamSet is

the QoS profile for this service flow. When '0' is returned

from reading this object, it means the QoS profile either

is not defined, or is defined in

wmanIf2BsMbsAdmitQoSProfileIndex. AdmittededQoSParamSet is

a subset of ProvisionedQoSParamSet."

REFERENCE

"Subclause 6.3.13 and 6.3.14"

::= { wmanIf2BsMbsServiceFlowEntry 6 }

wmanIf2BsMbsActiveQoSProfileIndex OBJECT-TYPE

SYNTAX      Integer32 (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2BsQoSProfileTable

that defines the ActiveQoSParamSet of a service flow. If

WmanIf2TcSfState = 'active', then ActiveQoSParamSet is the

QoS profile for this service flow. When '0' is returned

from reading this object, it means the QoS profile either

is not defined, or is defined in

wmanIf2BsMbsActiveQoSProfileIndex. ActiveQoSParamSet is a

subset of AdmittedQoSParamSet."

REFERENCE

"Subclause 6.3.13 and 6.3.14"

::= { wmanIf2BsMbsServiceFlowEntry 7 }

wmanIf2BsMbsArqAttributeIndex OBJECT-TYPE

SYNTAX      Integer32 (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in

wmanIf2BsArqAttributeTable that defines the ARQ

attributes for a service flow. When '0' is returned from

reading this object, it means the ARQ attributes are not

defined for this service flow."

REFERENCE

"Subclause 11.13.18"

::= { wmanIf2BsMbsServiceFlowEntry 8 }

wmanIf2BsMbsServiceFlowState OBJECT-TYPE

SYNTAX      WmanIf2TcSfState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsMbsServiceFlowState determines the state of a

service flow."

REFERENCE

"Subclause 6.3.14.2"

::= { wmanIf2BsMbsServiceFlowEntry 9 }

wmanIf2BsMbsCid OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier points to the connection being

created by DSA for this service flow. When '0' is returned

from reading this object, it means no CID has been assigned

to this service flow yet."

::= { wmanIf2BsMbsServiceFlowEntry 10 }

wmanIf2BsMbsSfCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2TcCsType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1"

::= { wmanIf2BsMbsServiceFlowEntry 11 }

wmanIf2BsMbsSfReqTxPolicy OBJECT-TYPE

SYNTAX      WmanIf2TcReqTxPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service flow.

An attribute is enabled by setting the corresponding bit

position to 1."

REFERENCE

"Subclause 11.13.12"

::= { wmanIf2BsMbsServiceFlowEntry 12 }

wmanIf2BsMbsSfTargetSaid OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The target SAID parameter indicates the SAID onto which the

service flow being set up shall be mapped."

REFERENCE

"Subclause 11.13.17"

::= { wmanIf2BsMbsServiceFlowEntry 13 }

wmanIf2BsMbsSfEstablishTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

established that means wmanIf2BsMbsServiceFlowState is

either in 'provisioned', 'admitted', or 'active' state."

::= { wmanIf2BsMbsServiceFlowEntry 14 }

wmanIf2BsMbsSfTerminateTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

terminated that means wmanIf2BsMbsServiceFlowState is

in 'inactive' state."

::= { wmanIf2BsMbsServiceFlowEntry 15 }

wmanIf2BsMbsSfFixedVsVariableSdu OBJECT-TYPE

SYNTAX      WmanIf2TcSduType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies whether the SDUs on

the service flow are variable-length (0) or fixed-length

(1). The parameter is used only if packing is on for the

service flow. The default value is 0, i.e.,variable-length

SDUs."

REFERENCE

"Subclause 11.13.15"

DEFVAL      { variableLength }

::= { wmanIf2BsMbsServiceFlowEntry 16 }

wmanIf2BsMbsSfFragmentSeqNumType OBJECT-TYPE

SYNTAX      WmanIf2TcFsnType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter indicates the size of the FSN

for the connection that is being setup.

'0' indicates 3 bits FSN

'1' indicates 11 bit FSN"

REFERENCE

"Subclause 11.13.22"

DEFVAL      { elevenBits }

::= { wmanIf2BsMbsServiceFlowEntry 17 }

wmanIf2BsMbsSfMbsService OBJECT-TYPE

SYNTAX      WmanIf2TcMbsType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter indicates whether the MBS

service is being requested or provided for a connection"

REFERENCE

"Subclause 11.13.23"

::= { wmanIf2BsMbsServiceFlowEntry 18 }

wmanIf2BsMbsClassifierRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMbsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains packet classifier rules associated

with logical channel IDs in MBS service flows."

::= { wmanIf2BsCm 11 }

wmanIf2BsMbsClassifierRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMbsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each packet classifier rule

."

INDEX     { ifIndex,

wmanIf2BsSsSfId,

wmanIf2BsMbsLogicalChannelId,

wmanIf2BsMbsClassifierRuleId }

::= { wmanIf2BsMbsClassifierRuleTable 1 }

WmanIf2BsMbsClassifierRuleEntry::= SEQUENCE {

wmanIf2BsMbsClassifierRuleId            Integer32,

wmanIf2BsMbsClassifierRulePriority      Integer32,

wmanIf2BsMbsClassifierRuleIpTosLow      Integer32,

wmanIf2BsMbsClassifierRuleIpTosHigh     Integer32,

wmanIf2BsMbsClassifierRuleIpTosMask     Integer32,

wmanIf2BsMbsClassifierRuleIpProtocol    Integer32,

wmanIf2BsMbsClassifierRuleIpSrcAddr     InetAddress,

wmanIf2BsMbsClassifierRuleIpSrcMask     InetAddress,

wmanIf2BsMbsClassifierRuleIpDestAddr    InetAddress,

wmanIf2BsMbsClassifierRuleIpDestMask    InetAddress,

wmanIf2BsMbsClassifierRuleSrcPortSt     Integer32,

wmanIf2BsMbsClassifierRuleSrcPortEnd    Integer32,

wmanIf2BsMbsClassifierRuleDestPortSt    Integer32,

wmanIf2BsMbsClassifierRuleDestPortEnd   Integer32,

wmanIf2BsMbsClassifierRuleDestMacAddr   MacAddress,

wmanIf2BsMbsClassifierRuleDestMacMask   MacAddress,

wmanIf2BsMbsClassifierRuleSrcMacAddr    MacAddress,

wmanIf2BsMbsClassifierRuleSrcMacMask    MacAddress,

wmanIf2BsMbsClassifierRuleEnetType      Integer32,

wmanIf2BsMbsClassifierRuleEnetProtocol  Integer32,

wmanIf2BsMbsClassifierRuleUserPriLow    Integer32,

wmanIf2BsMbsClassifierRuleUserPriHigh   Integer32,

wmanIf2BsMbsClassifierRuleVlanId        Integer32,

wmanIf2BsMbsClassifierRuleIpv6FlLabel   WmanIf2TcIpv6FlowLabel,

wmanIf2BsMbsClassifierRulePkts          Counter64,

wmanIf2BsMbsClassifierRuleBitMap        WmanIf2TcClassifierMap,

wmanIf2BsMbsClassifierRuleAssocPhsi     Integer32}

wmanIf2BsMbsClassifierRuleId  OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index is assigned to each classifier in the classifiers

table"

REFERENCE

"Subclause 11.13.19.3.4.14 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 1 }

wmanIf2BsMbsClassifierRulePriority OBJECT-TYPE

SYNTAX      Integer32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value specifies the order of evaluation of the

classifiers. The higher the value the higher the priority.

The value of 0 is used as default in provisioned service

flows classifiers. The default value of 64 is used for

dynamic service flow classifiers. If the referenced

parameter is not present in a classifier, this object

reports the default value as defined above"

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL      { 0 }

::= { wmanIf2BsMbsClassifierRuleEntry 2 }

wmanIf2BsMbsClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX      Integer32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 3 }

wmanIf2BsMbsClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX      Integer32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 4 }

wmanIf2BsMbsClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX      Integer32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The mask value is bitwise ANDed with TOS byte in an IP

packet and this value is used for the range checking of

TosLow and TosHigh. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 5 }

wmanIf2BsMbsClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX      Integer32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 6 }

wmanIf2BsMbsClassifierRuleIpSrcAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2BsMbsClassifierRuleIpSrcMask value

equals the wmanIf2BsMbsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 7 }

wmanIf2BsMbsClassifierRuleIpSrcMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2BsMbsClassifierRuleIpSrcMask value equals the

wmanIf2BsMbsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 8 }

wmanIf2BsMbsClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2BsMbsClassifierRuleIpDestMask value equals the

wmanIf2BsMbsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 9 }

wmanIf2BsMbsClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2BsMbsClassifierRuleIpDestMask value equals the

wmanIf2BsMbsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 10 }

wmanIf2BsMbsClassifierRuleSrcPortSt OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared

. This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 11 }

wmanIf2BsMbsClassifierRuleSrcPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 12 }

wmanIf2BsMbsClassifierRuleDestPortSt OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 13 }

wmanIf2BsMbsClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of

65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 14 }

wmanIf2BsMbsClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2BsMbsClassifierRuleDestMacMask equals the value of

wmanIf2BsMbsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 15 }

wmanIf2BsMbsClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2BsMbsClassifierRuleDestMacMask equals the value of

wmanIf2BsMbsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 16 }

wmanIf2BsMbsClassifierRuleSrcMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches this entry when its source MAC

address  bitwise ANDed with

wmanIf2BsMbsClassifierRuleSrcMacMask equals the value of

wmanIf2BsMbsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 17 }

wmanIf2BsMbsClassifierRuleSrcMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2BsMbsClassifierRuleSrcMacMask equals the value of

wmanIf2BsMbsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 18 }

wmanIf2BsMbsClassifierRuleEnetType OBJECT-TYPE

SYNTAX      INTEGER {none(0),

ethertype(1),

dsap(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 19 }

wmanIf2BsMbsClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If wmanIf2BsMbsClassifierRuleEnetType is none(0), this

object is ignored when considering whether a packet matches

the current rule. If wmanIf2BsMbsClassifierRuleEnetType is

ethertype(1), this object gives the 16-bit value of the

EtherType that the packet must match in order to match the

rule. If wmanIf2BsMbsClassifierRuleEnetType is dsap(2),

the lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 20 }

wmanIf2BsMbsClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2BsMbsClassifierRulePriLow and

wmanIf2BsMbsClassifierRulePriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 21 }

wmanIf2BsMbsClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2BsMbsClassifierRulePriLow

and wmanIf2BsMbsClassifierRulePriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 22 }

wmanIf2BsMbsClassifierRuleVlanId OBJECT-TYPE

SYNTAX      Integer32 (0..4095)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 23 }

wmanIf2BsMbsClassifierRuleIpv6FlLabel OBJECT-TYPE

SYNTAX      WmanIf2TcIpv6FlowLabel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2BsMbsClassifierRuleEntry 24 }

wmanIf2BsMbsClassifierRulePkts OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of packets that have

been classified using this entry."

::= { wmanIf2BsMbsClassifierRuleEntry 25 }

wmanIf2BsMbsClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2TcClassifierMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2BsMbsClassifierRuleEntry 26 }

wmanIf2BsMbsClassifierRuleAssocPhsi OBJECT-TYPE

SYNTAX      Integer32 (1..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The Associated PHSI has a value between 1 and 255, which

shall mirror the PHSI value of a PHS rule. This object

provide index to wmanIf2BsPhsRuleTable. Packets matching

the Packet Classification Rule containing the Associated

PHSI parameter shall undergo PHS according to the

corresponding PHS rule. A value '0' indicates that no PHS

rule is associated with this classifier rule."

REFERENCE

"Subclause 11.13.19.3.4.13 in IEEE Std 802.16-2004"

::= { wmanIf2BsMbsClassifierRuleEntry 27 }

wmanIf2BsPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When all classification rules associated with

the PHS rule are deleted, then the PHS rule shall also be

deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsCm 12 }

wmanIf2BsPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2BsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsSsSfId,

wmanIf2BsPhsRuleId }

::= { wmanIf2BsPhsRuleTable 1 }

WmanIf2BsPhsRuleEntry::= SEQUENCE {

wmanIf2BsPhsRuleId                      Integer32,

wmanIf2BsPhsRulePhsField                OCTET STRING,

wmanIf2BsPhsRulePhsMask                 OCTET STRING,

wmanIf2BsPhsRulePhsSize                 Integer32,

wmanIf2BsPhsRulePhsVerify               WmanIf2TcPhsRuleVerify}

wmanIf2BsPhsRuleId OBJECT-TYPE

SYNTAX      Integer32 (1 .. 255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsPhsRuleEntry 1 }

wmanIf2BsPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsPhsRuleEntry 2 }

wmanIf2BsPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2BsPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsPhsRuleEntry 3 }

wmanIf2BsPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2BsPhsRuleEntry 4 }

wmanIf2BsPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2TcPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2BsPhsRuleEntry 5 }

wmanIf2BsQoSProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsQoSProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains QoS profiles that are associated with

service flows or CIDs via the wmanIf2BsQoSProfileIndex.

The following table shows the required parameters for

different UL grant scheduling type.

0   - not required

1   - required

0-1 - optional

QoS Parameters                BE  ertPS  UGS  rtPS  nrtPS

---------------------------------------------------------

Traffic priority              0-1  0-1    0    0-1   0-1

Max sustained traffic rate    0-1  0-1    1    0-1   0-1

Min reserved traffic rate      0    1    0-1    1     1

Minimum traffic burst          0   0-1    0    0-1   0-1

Tolerated jitter               0   0-1    1     0     0

Maximum latency                0    1     1     1     0

Unsolicited Grant Interval     0    1     1     0     0

SDU size                       0    0    0-1    0     0

Unsolicited Polling Interval   0    0     0     1     0"

REFERENCE

"Subclause 6.3.14.4"

::= { wmanIf2BsCm 13 }

wmanIf2BsQoSProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2BsQoSProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each QoS parameter Set."

INDEX     { ifIndex, wmanIf2BsQoSProfileIndex }

::= { wmanIf2BsQoSProfileTable 1 }

WmanIf2BsQoSProfileEntry ::= SEQUENCE {

wmanIf2BsQoSProfileIndex                Integer32,

wmanIf2BsQosServiceClassName            OCTET STRING,

wmanIf2BsQosUlGrantScheduleType         WmanIf2TcSchedulingType,

wmanIf2BsQosTrafficPriority             Integer32,

wmanIf2BsQosMaximumSustainedRate        Unsigned32,

wmanIf2BsQosMinimumReservedRate         Unsigned32,

wmanIf2BsQosMaximumTrafficBurst         Unsigned32,

wmanIf2BsQosToleratedJitter             Unsigned32,

wmanIf2BsQosMaxLatency                  Unsigned32,

wmanIf2BsQosUnsolicitedGrantInterval    Unsigned32,

wmanIf2BsQosSduSize                     Unsigned32,

wmanIf2BsQosUnsolicitedPollInterval     Unsigned32}

wmanIf2BsQoSProfileIndex OBJECT-TYPE

SYNTAX      Integer32 (1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry in the

wmanIf2BsQoSProfileTable"

::= { wmanIf2BsQoSProfileEntry 1 }

wmanIf2BsQosServiceClassName  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(2..128))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the Null-terminated string of ASCII

characters. It refers to a predefined BS service

configuration to be used for a service flow."

REFERENCE

"Subclause 11.13.3"

::= { wmanIf2BsQoSProfileEntry 2 }

wmanIf2BsQosUlGrantScheduleType OBJECT-TYPE

SYNTAX      WmanIf2TcSchedulingType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the Uplink grant scheduling type

that shall be enabled for the associated uplink service

flow upstream service flow. If the parameter is not

present in the corresponding 802.16 QOS Parameter Set of

an upstream service flow, the default value is assumed."

REFERENCE

"Subclause 11.13.11"

DEFVAL	    { bestEffort }

::= { wmanIf2BsQoSProfileEntry 3 }

wmanIf2BsQosTrafficPriority OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority assigned

to a service flow. For uplink service flows, the BS should

use this parameter when determining precedence in request

service and grant generation, Higher numbers indicate

higher priority"

REFERENCE

"Subclause 11.13.5"

::= { wmanIf2BsQoSProfileEntry 4 }

wmanIf2BsQosMaximumSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate of the

service. The rate is expressed in bits per second and

pertains to the SDUs at the input to the Convergence

Sublayer."

REFERENCE

"Subclause 11.13.6"

::= { wmanIf2BsQoSProfileEntry 5 }

wmanIf2BsQosMinimumReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved for this

service flow. It specifies the minimum amount of data to be

transported on behalf of the service flow when averaged

over time."

REFERENCE

"Subclause 11.13.8"

::= { wmanIf2BsQoSProfileEntry 6 }

wmanIf2BsQosMaximumTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that must be

accommodated for the service. It defines the maximum

continuous burst the system should accommodate for the

service assuming the service is not currently using any of

its available resources."

REFERENCE

"Subclause 11.13.7"

::= { wmanIf2BsQoSProfileEntry 7 }

wmanIf2BsQosToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay variation (jitter)

for the connection."

REFERENCE

"Subclause 11.13.13"

::= { wmanIf2BsQoSProfileEntry 8 }

wmanIf2BsQosMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the maximum latency between the

ingress of a packet to the Convergence Sublayer and the

forwarding of the SDU to its Air Interface."

REFERENCE

"Subclause 11.13.14"

::= { wmanIf2BsQoSProfileEntry 9 }

wmanIf2BsQosUnsolicitedGrantInterval OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the nominal interval between

successive data grant opportunities for a service flow."

REFERENCE

"Subclause 11.13.20"

::= { wmanIf2BsQoSProfileEntry 10 }

wmanIf2BsQosSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the length of the SDU for a

fixed-length SDU service flow. It is used only if packing

is on and the service flow is indicated as carrying

fixed-length SDUs. If this object is omitted in the QoS

parameter set, it should return 0 that means the

variable-length service flow."

REFERENCE

"Subclause 11.13.16"

::= { wmanIf2BsQoSProfileEntry 11 }

wmanIf2BsQosUnsolicitedPollInterval OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximal nominal interval between

successive polling grants opportunities for this Service

Flow."

REFERENCE

"Subclause 11.13.21"

::= { wmanIf2BsQoSProfileEntry 12 }

wmanIf2BsArqAttributeTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsArqAttributeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains ARQ parameters that are associated

with the Service Flows."

REFERENCE

"Subclause 11.13.18"

::= { wmanIf2BsCm 14 }

wmanIf2BsArqAttributeEntry OBJECT-TYPE

SYNTAX      WmanIf2BsArqAttributeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each created service flow

for a given MacAddress."

INDEX     { ifIndex, wmanIf2BsArqIndex  }

::= { wmanIf2BsArqAttributeTable 1 }

WmanIf2BsArqAttributeEntry::= SEQUENCE {

wmanIf2BsArqIndex                     Integer32,

wmanIf2BsArqEnable                    TruthValue,

wmanIf2BsArqWindowSize                Integer32,

wmanIf2BsArqTxRetryTimeout            Integer32,

wmanIf2BsArqRxRetryTimeout            Integer32,

wmanIf2BsArqBlockLifetime             Integer32,

wmanIf2BsArqSyncLossTimeout           Integer32,

wmanIf2BsArqDeliverInOrder            TruthValue,

wmanIf2BsArqRxPurgeTimeout            Integer32,

wmanIf2BsArqBlockSizeReq              WmanIf2TcArqBlockSize,

wmanIf2BsArqBlockSizeRsp              Integer32,

wmanIf2BsArqAckProcessingTime         Integer32}

wmanIf2BsArqIndex OBJECT-TYPE

SYNTAX      Integer32 ( 1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry in the

in the wmanIf2BsArqAttributeTable."

::= { wmanIf2BsArqAttributeEntry 1 }

wmanIf2BsArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

REFERENCE

"Subclause 11.13.18.1"

::= { wmanIf2BsArqAttributeEntry 2 }

wmanIf2BsArqWindowSize	 OBJECT-TYPE

SYNTAX	    Integer32 (1 .. 1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged fragments

at any time."

REFERENCE

"Subclause 11.13.18.2"

::= { wmanIf2BsArqAttributeEntry 3 }

wmanIf2BsArqTxRetryTimeout	 OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 65535)

UNITS	    "100 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates transmitter delay, including sending (e.g., MAC

PDUs) and receiving (e.g., ARQ feedback) delays and other

implementation dependent processing delays. If the

transmitter is the BS, it may include other delays such as

scheduling and propagation delay."

REFERENCE

"Subclause 11.13.18.3"

::= { wmanIf2BsArqAttributeEntry 4 }

wmanIf2BsArqRxRetryTimeout	 OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 65535)

UNITS	    "100 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates receiver delay, including receiving (e.g., MAC

PDUs) and sending (e.g., ARQ feedback) delays and other

implementation-dependent processing delays. If the

receiver is the BS, it may include other delays such as

scheduling and propagation delay."

REFERENCE

"Subclause 11.13.18.3"

::= { wmanIf2BsArqAttributeEntry 5 }

wmanIf2BsArqBlockLifetime OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be managed

by the transmitter ARQ machine, once initial transmission

of the fragment has occured. If transmission or

retransmission of the fragment is not acknowledged by the

receiver before the time limit is reached, the fragment is

discarded. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.4"

::= { wmanIf2BsArqAttributeEntry 6 }

wmanIf2BsArqSyncLossTimeout OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 65535 )

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss of

synchronization of the sender and receiver state machines.

A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.5"

::= { wmanIf2BsArqAttributeEntry 7 }

wmanIf2BsArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered by the

receiving MAC to its client application in the order in

which data was handed off to the originating MAC."

REFERENCE

"Subclause 11.13.18.6"

::= { wmanIf2BsArqAttributeEntry 8 }

wmanIf2BsArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means

Infinite."

REFERENCE

"Subclause 11.13.18.7"

::= { wmanIf2BsArqAttributeEntry 9 }

wmanIf2BsArqBlockSizeReq OBJECT-TYPE

SYNTAX	    WmanIf2TcArqBlockSize

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an ARQ

block included in DSA-REQ and RSG-REQ. This parameter shall

be established by negotiation during the connection

creation dialog."

REFERENCE

"Subclause 11.13.18.8"

::= { wmanIf2BsArqAttributeEntry 10 }

wmanIf2BsArqBlockSizeRsp OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an ARQ

block included in DSA-RSP and RSG-RSP.

Bit 0-3: encoding for selected block size (P)

Bit 4-7: set to 0

where:

The selected block size is equal to 2^(P+4), P<=6 and

M<=N"

REFERENCE

"Subclause 11.13.18.8"

::= { wmanIf2BsArqAttributeEntry 11 }

wmanIf2BsArqAckProcessingTime OBJECT-TYPE

SYNTAX	    Integer32 (0 .. 255)

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the number of ms required by the

ARQ receiver to process the received ARQ blocks and provide

a valid ACK or NAK."

REFERENCE

"Subclause 11.13.18.9"

::= { wmanIf2BsArqAttributeEntry 12 }

  - Changes to wmanIf2TcMib

13.2.3 wmanIf2TcMib

[Add the following Textual Conventions to wmanIf2TcMib]

WmanIf2TcSfState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the state of a service flow.

'inactive' - A service flow is inactive, when the MS owns

this service flow has handoff to another BS.

'provisioned' - A service flow is provisioned, but not

yet activated.

'admitted' - This maps to the 1st phase of the two-phase

activation model that the bandwidth a service

flow is reserved. But, no traffic can be

sent on this service flow yet.

'active' - This maps to the 2nd phase of the two-phase

activation model that bandwidth is granted,

(e.g., is actively sending UL maps containing

unsolicited grants for a UGS service flow)."

REFERENCE

"Subclause 6.3.14.2"

SYNTAX      INTEGER {inactive(0),

provisioned(1),

admitted(2),

active(3)}

WmanIf2TcClassifierMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4"

SYNTAX      BITS {priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12)}