| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections of Group DSx TLV. |  |
| Date Submitted | 2008-06-21 |  |
| Source(s) | Henri Moelard Motorola | moelard@motorola.com |
| Re: | IEEE802.16Rev2/D5 |  |
| Abstract | Corrections, clarifications and enhancements of Group DSx TLV. |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections of Group DSx TLV

Henri Moelard

Motorola

# Comments

- Rev2/D5 sub-clause 11.13.41 specifies a Type value of 50 for the Group parameter Create/Change TLV. This value should have a prefix of “[145/146]” as it is a service flow management encodings TLV.

- Service flow encoding TLVs (types value < 50, or cst) embedded in the Group parameter Create/Change sub-TLVs are specified by their full Type value, including the [145/146] prefix. This is superfluous and adding unnecessary overhead.

- The definition does not specify that Classification Rules and PHS Rules can be specified partly in the Common Parameters TLV (common part) and partly in the SFID parameter list TLV (specific part), whereas that was intended.

- Sub-clause 6.3.2.3.10 (DSA-REQ message) specifies that “A DSA-REQ message shall not contain parameters for more than one service flow.”. Same for 6.3.2.3.13 (DSC-REQ message): “A DSC-REQ message shall not carry parameters for more than one service flow.” That is no longer true, and should be corrected.

# Suggested remedy

- Add prefix “[145/146]” to the Type value 50 in sub-clause 11.13.41.

- Specify that service flow encoding TLVs (types value < 50, or cst) embedded in the Group parameter Create/Change sub-TLVs are specified without the [145/146] compound encapsulation.

- Specify that Classification Rules and PHS Rules can be specified partly in the Common Parameters TLV (common part) and partly in the SFID parameter list TLV (specific part). Both parts are to be related by means of the Index that is to be included in both parts.

- Sub-clauses 6.3.2.3.10 (DSA-REQ message) and 6.3.2.3.13 (DSC-REQ message) are to be corrected as shown below.

# Text Changes

Change the text in sub-clause 6.3.2.3.10 (page 121, line 37) as follows:

6.3.2.3.10 DSA-REQ message

A DSA-REQ message shall not contain parameters for more than one service flow, unless the Group parameter Create/Change TLV (11.13.41) is used.

Change the text in sub-clause 6.3.2.3.13 (page 124, line 50) as follows:

6.3.2.3.13 DSC-REQ (DSC request) message

A DSC-REQ message shall not carry parameters for more than one service flow, unless the Group parameter Create/Change TLV (11.13.41) is used.

Change the text in sub-clause 11.13.41 as follows:

11.13.41 Group parameter Create/Change TLV

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Group parameter Create/ Change | [145/146].50 | variable | Compound Only one instance of the Group parameter Create/ Change TLV may be included in any DSx message | DSA-REQ, DSA-RSP, DSC-REQ, DSC-RSP |

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Common Parameters | [145/146].50.1 | variable | Common Parameters is a compound TLV value that encapsulates the common related service flow management encodings that are common to all service flows specified in this Group parameter Create/Change TLV. Only common related service flow encodings shall be included in this TLV. All the rules and settings that apply to the service flow management encodings when used in a DSA or DSC message apply to the contents encapsulated in this TLV. If included in the Group parameter Create/Change TLV, Common Parameters shall be the first attribute of the Group parameter Create/Change TLV. Common Parameters shall be included only once in a Group parameter Create/Change TLV. |

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Qty SFID request | [145/146].50.2 | 1 | Qty SFID request is the quantity of service flows, of the same common parameter set configuration, that the MS is requesting. Qty SFID request shall only be sent by the MS, only as the last attribute of a Group parameter Create/Change TLV, and only in a DSA-REQ. |

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| SFID List | [145/146].50.3 | nx4 | List of n SFIDs. See 11.13.1. |

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| SFID parameter list | [145/146].50.4 | variable | SFID parameter list is a compound TLV value that encapsulates an SFID and associated noncommon service flow management encodings for that service flow, specified in this Group parameter Create/Change TLV. See the following Table for the format of the SFID parameter list. All the rules and settings that apply to the service flow management encodings when used in a DSA or DSC message apply to the contents encapsulated in this TLV. If included in the Group parameter Create/Change TLV, SFID parameter list shall be the last attribute. SFID parameter list may be included more than once in a DSx Group Create/Change TLV. |

Add the following text at the end of sub-clause 11.13.41:

Service flow encoding TLVs embedded in the Group parameter Create/Change sub-TLVs are embedded here without the [145/146] compound encapsulation, as shown in the following example:

T=146	(DL service flow management encodings TLV)

L

V= {	T=50	(Group parameter Create/Change TLV)

L

V= {	T=1	(Common Parameters TLV)

L

V= {	T=4	(MBS Service TLV)

L=1

V=2

T= etc.	(other common service flow encodings TLVs

L=

V=

…

}

T=3	(SFID List TLV)

L=4*n

V= n * SFID value

T=4	(SFID parameter list TLV)

L

V= {	SFID value

T=2	(CID TLV)

L=2

V= Multicast CID value

T= etc.	(other non-common service flow encodings TLVs)

L=

V=

…

}

}

Packet Classification Rules and PHS Rules may be specified partly in the Common Parameters TLV (common part) and partly in the SFID parameter list TLV (specific part). In this case both parts of a rule are related to each other by means of their Index, Packet Classification Rule Index (11.13.19.3.4.14) and PHSI (11.13.19.3.7.1) respectively, that is to be included in both parts.