| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAP error handling procedures |  |
| Date Submitted | 2008-07-08 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, and Kiseon Ryu LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: E-mail: chegal@lge.com, sixs@lge.com and ksryu@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26d, on P802.16Rev2/D5. |  |
| Abstract | This proposal clarifies the MAP error handling procedures for persistent scheduling. |  |
| Purpose | Discuss and accept the proposed specification changes into IEEE 802.16Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAP error handling procedures

Jeongki Kim, Youngsoo Yuk, and Kiseon Ryu,

LG Electronics

# Introduction

# In the IEEE 802.16Rev2/D5 standard, the persistent allocation method is defined. When using the persistent allocation, MS can receive the DL burst and transmit the UL burst using the resource allocated persistently without the DL/UL-MAP message for the MS. In the frame which the persistent resource is allocated, when MS fails to decode the MAP messages (which may include Persistent HARQ DL/UL MAP IE for the MS), the MS may perform the operation of persistent allocation, that is, MS may receive the DL burst or transmit the UL burst at predefined resource. That results in misuse of resource or collision, especially in the resource shifting scenario. Therefore, MS should not perform the operation for persistent allocation at the frame in which MS fails to decode MAP message. However, the IEEE 802.16Rev2/D5 doesn’t define this operation clearly.

# Proposed Text Changes

[note to the editor: Make the following changes to IEEE P802.16Rev2/D5 as shown below;

Blue = insertions relative to R2D5

]

6.3.27.1 Error handling procedures

The BS may assign MS’s having persistent resource allocations a persistent MAP NACK channel and non-persistent MAP ACK channel. The MAP NACK channel is persistent for the whole time the persistent resource allocation exists.

The MS shall transmit at the MAP ACK signal via assigned MAP ACK channel to acknowledge the correct reception of a Persistent HARQ DL MAP IE or Persistent HARQ UL MAP IE, which includes the RCID of the MS. The MAP ACK channel carries one codeword indicating an ACK as described in 8.4.5.4.10.16.

The MAP NACK channel is a shared channel used by the MS to indicate MAP decoding errors. The BS may assign the same MAP NACK channel index to one or more MSs therefore more than one MS may transmit using a particular MAP NACK channel at the same time. The MAP NACK channel carries one codeword indicating a NACK as described in 8.4.5.4.10.17.

The MAP ACK and MAP NACK channels correspond to resources within the Fast Feedback region (section 8.4.5.4.10).

When MS fails to decode the MAP message which may include Persistent HARQ DL/UL MAP IE for the MS in the frame which the persistent resource is allocated for the MS, the MS shall not perform the operation for persistent allocation in the frame in which MS fails to decode MAP message.