| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PCINR Formulas for MIMO Mode |  |
| Date Submitted | 2008-07-15 |  |
| Source(s) | Sylvain Labonte SEQUANS Communications Based on discussions with, and inputs from: Djordje Tujkovic, Louay Jalloul, Beceem Amir Francos, Alvarion Dave Pechner, Arvind Raghavan, Arraycomm Itay Lusky, Altair Semiconductor Rotem Avivi Intel Ji-Yun Seol Samsung And many others | Voice: +33 1 41 02 82 99 E-mail: sylvain@sequans.com |
| Re: | LB#26d comment 4182 |  |
| Abstract | This paper presents the formulas MSs can use to determine PCINR in MIMO mode. |  |
| Purpose | This is my reply to the proposed remedy of LB#26d-4182. Please incorporate the text of this contribution in IEEE P802.16 Rev2 Draft 6. instead of the one proposed in LB#26d-4182. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Formulas for PCINR in MIMO Mode

Sylvain Labonte, SEQUANS Communications

# Proposed Text Change 1

P.839, line 49: change the text as follows:

" For a vertically encoded MIMO system, the averaged CINR is defined per tone k () as shown in equation (62):

(62)

where k denotes a sub-carrier index,  is the per-tone receiver-constrained mutual information conditioned on knowing the MIMO channel knowledge on tone k. Note that  is the transmitted signal,  is the post-processing received signal and  is the MIMO channel matrix on tone k that defines the channel matrix between Rx and Tx antennas. For an LMMSE receiver used in MIMO matrix B reception, the individual post-detector-processing signal-to-noise ratios are given as  as shown in Figure 233 and in equation (63). For an LMMSE MIMO matrix B receiver the per tone mutual information is given by:

(63)

where  denotes the CINR per layer and per tone".

# Proposed Text Change 2

P. 840, line 1: change text as follows:

“In this case, the average CINR per tone over the spatial layers is given by:

(64)

# Proposed Text Change 3

p. 840 add the following text on line 58:

8.4.5.4.10.1.1 PCINR Formulas for Matrix A and Matrix B with ML Receiver

For Matrix B the equations take the form:

where p is an index for a pair of pilots,  is the average noise plus interference over receive antennas and pair of pilots, N is the number of streams (in Matrix B, N = 2). Here it is assumed that covariance matrix of noise plus interference can be expressed as  where I is the 2x2 identity matrix. This metric is applicable to ML receiver.

For Matrix A the equations take the form:

where p is an index for a pair of pilots,  is the average noise plus interference over receive antennas and pair of pilots. It is assumed that covariance matrix of noise plus interference can be expressed as  where I is an identity matrix.

8.4.5.4.10.1.1.1 Standard-compliant approximations

Implementations adopting the following formulations are also standard-compliant.

Definitions

Referring to Figure 1, the following is defined:

- l is the index of an OFDMA symbol, ,

- j is the index of a “column”, and has resolution of two OFDMA symbols, ,

- sub-carrier block is a set of physically adjacent sub-carriers,

- m is the index of sub-carrier block, ,

- k is the  index of a pair of pilots within one sub-carrier block within one column,

- ∆ is a subset of the columns in the zone, ,

- is the cardinality of ∆, where ,

- Λ is a subset of the OFDMA symbols in the zone, ,

- is the cardinality of Λ, .

Note that the number of sub-carrier blocks is implementation-dependent with the only constraint that .  Also note that when working in segmented PUSC, only the active pilots in the sub-carrier block should be considered.

Figure 1: PUSC STC zone illustration. The white and black circles indicate the pilot tones.

Capacity Averaging

The average CINR over a zone can be given by

where q=0 or q=1 depending on the MS specific implementation. This is to say that MS specific implementation can drop the “1” inside the log term.

The capacity can be averaged over a zone as follows:

For Matrix B, we have

,

and for Matrix A we have

,

where z=0 or z=1 depending on the MS specific implementation. This is to say that the MS specific implementation can drop the “I” inside the determinant for the capacity expression for Matrix B and the “1” inside the log term for the capacity expression for Matrix A.

Noise averaging

The noise may be averaged over the m-th sub-carrier block as follows:

where  is the noise plus interference averaged over the receive antennas on a single pilot  position (l,k).