| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PCINR Formulas for MIMO Mode |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | Itay Lusky, Altair Semiconductor Dave Pechner, Arvind Raghavan, Arraycomm Amir Francos, Alvarion Djordje Tujkovic, Louay Jalloul, Beceem Rotem Avivi, Dov Andelman Intel Ji-Yun Seol Samsung Sylvain Labonte, SEQUANS | Voice: +33 1 41 02 82 99 E-mail: sylvain@sequans.com |
| Re: | LB#26d comment 4182 |  |
| Abstract | This paper presents the formulas MSs can use to determine PCINR in MIMO mode. |  |
| Purpose | This is my reply to the proposed remedy of LB#26d-4182. Please incorporate the text of this contribution in IEEE P802.16 Rev2 Draft 6. instead of the one proposed in LB#26d-4182. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Formulas for PCINR in MIMO Mode

Itay Lusky,  Altair Semiconductor

Dave Pechner, Arvind Raghavan, Arraycomm

Amir Francos, Alvarion

Djordje Tujkovic, Louay Jalloul, Beceem

Rotem Avivi, Dov Andelman Intel

Ji-Yun Seol Samsung

Sylvain Labonte SEQUANS

# Proposed Text Changes

P.839, line 21: change the text as follows: “

8.4.5.4.10.1 Fast DL measurement feedback

MIMO-capable MS shall measure post-processing CINR for each individual layers as shown in Figure 244. When the FFSH's Feedback Type field is 00, the MS shall report the post-processing average CINR (Avg_CINR) as defined in Equation (62). When BS requests MS feedback through CQICH_Alloc_IE() or CQICH_Enhanced_Alloc_IE() with feedback_type field = 00, MS shall report Avg_CINR or individual layer CINR as described in 8.4.5.4.12 and 8.4.5.4.16 as described below.

(Note to editor: Remove all following text in red )

For a vertically encoded MIMO system, the averaged CINR (Avg_CINR) is defined as shown in

Equation (62).

(62)

Where  is the receiver-constrained mutual information conditioned on knowing the channel knowledge. Note that d is the transmitted signal, y is the post-processing receive signal, and H is the channel matrix between Tx and Rx antennas. For an LMMSE receiver, the individual post-detector-processing signal-to-noise ratios are given as CINR1, ...,CINRN, as shown in Figure 244 and in Equation (63).

(63)

“In this case,

(64)

When the individual post-detector processing CINR is high, the average CINR is

(65)

# For ML MIMO detectors case,

# (66)

# Where

is an N x N identity matrix

is the correlation matrix of interference plus noise measured at MS

For a single layer MIMO system (Matrix A and Matrix B) denote  p as the index for a pair of pilots,  is the average noise plus interference variance over MS’s receive antennas and pair of pilots. Further denote  as the receiver-constrained mutual information conditioned on the channel knowledge, wherein   is the transmitted signal, is the post-processing receive signal and is the channel matrix between Tx and Rx antennas.

For Matrix B the average CINR is given by:

(62)

where  N  is the number of streams, i.e. N = 2 is this case.

For Matrix A the average CINR is given by:

(63)

where  denotes the Frobenius norm of a matrix.

(Note to editor: Add the following after section 8.4.5.4.10.1)

8.4.5.4.10.1.1 Standard Compliant Approximations

Standard-compliant approximations of Equations (62) and (63) are shown below.

Referring to Figure xxx, the following are defined:

- l is the index of an OFDMA symbol, ,

- j is the index of a “column”, and has resolution of two OFDMA symbols, ,

- sub-carrier block is a set of physically adjacent sub-carriers,

- m is the index of sub-carrier block, ,

- k is the  index of a pair of pilots within one sub-carrier block within one column,

- ∆ is a subset of the columns in the zone, ,

- is the cardinality of ∆, where ,

- Λ is a subset of the OFDMA symbols in the zone, ,

- is the cardinality of Λ, .

Note that the number of sub-carrier blocks is implementation-dependent with the only constraint that .  Also note that when working in segmented PUSC, only the active pilots in the sub-carrier block should be considered.

Figure xxx: PUSC STC zone illustration. The white and black circles indicate the pilot tones.

The average CINR over a zone can be given by

where q=0 or q=1 depending on the MS specific implementation. This is to say that MS specific implementation can drop the “1” inside the log term.

The capacity can be averaged over a zone as follows:

For Matrix B, we have

,

and for Matrix A we have

,

where z=0 or z=1 depending on the MS specific implementation. This is to say that the MS specific implementation can drop the “I” inside the determinant for the capacity expression for Matrix B and the “1” inside the log term for the capacity expression for Matrix A.

The noise can be averaged over the m-th sub-carrier block as follows:

where  is the noise plus interference variance averaged over the MS's receive antennas on a single pilot  position (l,k).