| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Recovery of H-FDD group switch failure |  |
| Date Submitted | 2008-07-17 |  |
| Source(s) | Andrea Bacioccola, Zexian Li, Roberto Albanese Nokia* Giovanni Maggi, Aik Chindapol Nokia Siemens Networks* | Voice: E-mail: Andrea.Bacioccola@nokia.com, Zexian.Li@nokia.com, Roberto.Albanese@nokia.com, Giovanni.Maggi.ext@nsn.com Aik.Chindapol@nsn.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26d |  |
| Abstract | This contribution proposed a recovery scheme for H-FDD group switch failure |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Recovery of H-FDD group switch failure

Andrea Bacioccola, Roberto Albanese, Zexian Li,

Nokia

Giovanni Maggi, Aik Chindapol,

Nokia Siemens Networks

# Problem description

In the IEEE 802.16Rev2/D5 standard, Group Switching procedures for H-FDD MSs are defined in 8.4.4.1.1.

The BS may instruct MS to switch from one group to the other one. In addition, the BS may require the MS to explicitly acknowledge receiving a group switch instruction, in which case the BS shall assign a one-time ACK channel.

The current specification draft does not define a method for the MS to signal to the BS regarding the successful execution of the group switching procedure and the mechanisms to recover in case of MS fails to switch to the new group. In addition, the BS has no way in knowing whether MS will continue trying to stay in the new group despite unsuccessful attempts or return to the original group.

Furthermore, the current specification does not provide any mechanism for the BS to recover from an error situation where the MS correctly acknowledges the switch command, but the BS is not able to decode it (e.g. a failure in the transmission/reception of the ACK message).

The proposed changes shown below define a method for the MS and BS to recover from the group switch failure.

# Proposed Changes

Change #1: Modify the following sentences in Sec 8.4.4.1.1 as follows:

The BS may request the MS to explicitly acknowledge a group switch instruction in which case the BS shall assign a one-time MAP ACK channel. When either the DL HARQ Chase Subburst IE or the DL HARQ IR CTC Subburst IE, or the DL HARQ IR CC Subburst IE is used for signaling a group switch, the one-time CQICH channel shall be allocated via setting the LSB #0 of the Dedicated DL Control Indicator to 1 and using the Allocation Index field to indicate the location.

When the H-FDD Group Switch IE is used for signaling a group switch, a one-time CQICH channel may be assigned in that IE (see 8.4.5.3.28). In case the BS includes a one-time CQICH allocation in the IE that contains the group switch instruction, the MS shall acknowledge reception of the instruction - with a MAP ACK command, as described in section 8.4.5.4.10.16 - in the assigned CQICH channel. The MS should initiate an uplink communication with the BS to confirm the success of group switch operation.

The BS shall include the group switch completion time TLV in the UCD message to indicate the number of frames after which the MS is expected to return to the original group after failed attempts to switch to the new group. If the MS cannot join the new group within the group switch completion time, the MS shall deem the group switch instruction invalid, return to Group 1 and initiate an uplink communication with the BS. The BS shall interpret such communication as an indication from the MS that it failed to execute the group switch instruction.

After the MS receives a group switch instruction in frame n, the MS shall switch to the new group and decode the downlink subframe in frame n+H-FDD_Group_Switch_Delay+m, where m denotes the current group number (1 or 2) and H-FDD_Group_Switch_Delay is specified in the UCD. The MS may ignore the downlink subframe of the current group in frame n+H-FDD_Group_Switch_Delay+1 (the subframe immediately preceding the transition). See Figure 228.

Change #3: Add the following field in Table 563 (UCD PHY-specific channel encodings—WirelessMAN-OFDMA):

| Name | Type (1 byte) | Length | Value |
| --- | --- | --- | --- |
| Group Switch Completion Time | xxx | 1 | The number of frames after which the MS is expected to return to its original group following failed attempts to switch to the new group in accordance with Section 8.4.4.41. The reference point is the frame in which the first map is expected to be received in the new group in accordance to Figure 228. |

# References

[IEEE802.16-Rev2/D5]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D5 (June 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.