| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Extended-3 DUIC/UIUC Code Space |  |
| Date Submitted | 2008-09-11 |  |
| Source(s) | Sean McBeath, Huawei Technologies Zhigang Rong, Huawei Technologies Anthony Soong, Huawei Technologies Yair Bourlas, Nextwave Mo-Han Fong, Nortel Networks | Voice: E-mail: smcbeath@huawei.com zrong@huawei.com asoong@huawei.com YBourlas@nextwave.com mhfong@nortel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract | This contribution provides an extended-3 DIUC/UIUC code space to accommodate future expandability |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D6a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Extended-3 DUIC/UIUC Code Space

Sean McBeath, Anthony Soong, Zhigang Rong, Huawei Technologies

Yair Bourlas, Nextwave

Mo-Han Fong, Nortel Networks

Introduction

IEEE 802.16Rev2 has currently utilized all of the Extended-2 DIUC codes.  This prevents the natural extension to Extended-3 DUIC.  In addition, there is only one Extended-2 UIUC code left, which should be reserved for Extended-3 UIUC.  In this contribution, we introduce a new Extended-3 DIUC/UIUC code space to address these issues and allow future expandability for the standard.

Text Changes

[note to the editor: Make the following changes to IEEE P802.16Rev2/D6a as shown below;

Blue = insertions relative to R2D6a

Red = deletions relative to R2D6a

[] = notes to editor

]

8.3.5.3.2.2 DL-MAP Extended-2 IE encoding format

[Modify Table 325 as follows and add a new table for Extended-3 DIUC]

Table 325- Extended-2 DIUC code assignment for DIUC=14

| Extended-2 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | MBS MAP IE |
| 01 | HO Anchor Active DL MAP IE |
| 02 | HO Active Anchor DL MAP IE |
| 03 | HO CID Translation MAP IE |
| 04 | MIMO in Another BS IE |
| 05 | Macro-MIMO DL Basic IE |
| 06 | Skip IE |
| 07 | HARQ DL MAP IE |
| 08 | HARQ ACK IE |
| 09 | Enhanced DL MAP IE |
| 0A | Closed-loop MIMO DL Enhanced IE |
| 0B | MIMO DL Basic IE |
| 0C | MIMO DL Enhanced IE |
| 0D | Persistent HARQ DL MAP IE |
| 0E | AAS SDMA DL IE |
| 0F | Power Boosting IE |
| 0F | Extended-3 DIUC |

Table XYZ defines the encoding for extended-3 DIUC that shall be used by DL-MAP Extended-3 IEs.

Table XYZ- Extended-3 DIUC code assignment for Extended-2 DIUC=15

| Extended-3 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | Power Boosting IE |
| 01-0F | Reserved |

[Modify the following entries in section 8.3.5.3.30]

8.3.5.3.30   Power Boosting IE

In the DL-MAP, BS may transmit DIUC=14 with the Power Boosting IE (), as shown in Table 371, to signal the update of power boosting information for persistent allocations assigned to MSs by the Persistent HARQ DL MAP IE. The power boosting information in the Power Boosting IE shall be applied to persistent allocation associated with the R_CID and ACID in the Power Boosting IE.

Table 371 - Power Boosting IE

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| Power boosting IE{ |  |  |
| Extended-2 DIUC | 4 | 0x0F (Extended-3 DIUC) |
| Length | 8 | Length in bytes |
| Extended-3 DIUC | 4 | 0x00 |
| RCID_Type | 2 | 0b00 : Normal CID 0b01 : RCID11 0b10 : RCID7 0b11 : RCID3 |
| Number of RCIDs | 4 |  |
| for(i=0;i< Number of RCID; i++){ | 1 |  |
| R_CID | Variable |  |
| A_CID | 4 | Start of ACID |
| Boosting | 3 | 0b000:Normal (not boosted) 0b001:+6dB 0b010:-6dB 0b011:+9dB 0b100:+3dB 0b101:-3dB 0b110:-9dB 0b111:-12dB |
| } |  |  |
| Padding | Variable | Padding to byte for the unspecified portion of this IE (i.e. not including the first two fields, “Extended-2 DIUC” “Length”); shall be set to 0. |
| } |  |  |

[Modify the following entries in section 8.3.5.4.4.2]

8.3.5.4.4.2 UL-MAP Extended-2 IE format

Table 379 - Extended-2 UIUC code assignment for UIUC = 11

| Extended-2 Type (hexadecimal) | Usage |
| --- | --- |
| 00 | CQICH Enhanced Allocation IE |
| 01 | HO Anchor Active UL-MAP IE |
| 02 | HO Active Anchor UL-MAP IE |
| 03 | Anchor BS Switch IE |
| 04 | UL Sounding Command IE |
| 05 | ReservedExtended-3 UIUC |
| 06 | MIMO UL Enhanced IE |
| 07 | HARQ UL MAP IE |
| 08 | HARQ ACKCH Region Allocation IE |
| 09 | MIMO UL Basic IE |
| 0A | Mini-subchannel allocation IE |
| 0B | UL_PC_Bitmap IE |
| 0C | Persistent HARQ UL MAP IE |
| 0D | FDD Paired Allocation IE |
| 0E | AAS SDMA UL IE |
| 0F | Feedback Polling IE |

Table XYZ defines the encoding for extended-3 UIUC that shall be used by UL-MAP Extended-3 IEs.

Table XYZ- Extended-3 UIUC code assignment for Extended-2 UIUC=05

| Extended-3 UIUC (hexadecimal) | Usage |
| --- | --- |
| 00-0F | Reserved |