| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on paging offset in paging information |  |
| Date Submitted | 2008-11-13 |  |
| Source(s) | Nan Li ,Li Wang, Hongyun Qu, Sean Cai ZTE Corporation | Voice: E-mail: li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Revision 2 |  |
| Abstract | This contribution proposed clarification on paging offset in. paging information. |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarifications on paging offset in paging information

Nan Li, Li Wang, Hongyun Qu,Sean Cai

ZTE Corporation

# Problem description

In the current IEEE 802.16 Rev2 D7 standard, section 11.1.8.3, the definition of PAGING_OFFSET is as follows:

PAGING_OFFSET - determines the frame within the cycle in which the paging message is transmitted. Must be smaller than PAGING_CYCLE value.

But, in section 6.3.23.5, description is as follows:

A BS paging interval shall occur during the N frames beginning with the frame whose frame number, Nframe, meets the condition

Nframe modulo PAGING_CYCLE == PAGING_OFFSET

on each BS, where N is Paging Interval Length. A BS shall broadcast at least one, but may broadcast more

than one BS Broadcast Paging messages during the MS Paging Listening Interval.

A BS Broadcast Paging message in a specific BS paging interval only pages those MS MAC address hash particular to the Paging Interval Length, Frame number, PAGING_CYCLE and PAGING_OFFSET. The BS Broadcast Paging messages shall be transmitted during the MS Paging Listening Interval and the MS will begin decoding for any BS Broadcast Paging message during the entire BS paging interval.

So it can be concluded that paging offset is not the frame in which the paging message is transmitted. Paging offset should be the start of the paging interval.

Proposed changes

[Change text in 11.1.8.3 (page 1166 as indicated:]

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 134 | 7 | Bits #0-15: PAGING_CYCLE - cycle in which the paging message is transmitted within the paging group. Bits #16-31:PAGING_OFFSET - determines the frame within the cycle from which the paging interval starts in which the paging message is transmitted. Must be smaller than PAGING_CYCLE value. Bits #32-47: Paging-group-ID - ID of the paging group the MS is assigned to. Bits #48-55: Paging Interval Length - Max duration in frames of Paging Listening interval. Used in calculation of Paging listening interval; value must be between 1 and 5 frames (default=2). | RNG-RSP DREG-CMD |

# References

IEEE Std 802.16Rev2-D7 ((Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005,IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007)