| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Capability Exchange for BS-control Handover |  |
| Date Submitted | 2008-09-17 |  |
| Source(s) | Giovanni Maggi, Aik Chindapol Nokia Siemens Networks Brian Shim Samsung | Voice: E-mail: giovanni.maggi.ext@nsn.com Aik.chindapol@nsn.com brian.shim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract | This contribution proposes capability exchange for BS-controlled handover. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D6a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Capability Exchange for BS-controlled Handover

Giovanni Maggi, Aik Chindapol

Nokia Siemens Networks

Brian Shim

Samsung

# Problem description

# According to the current standard, when the BS sets the “HO type support” in the DCD message to indicate the support of BS_controlled_HO, there may be confusion to the legacy MS that do not support this capability.

In order to ensure inter-operability and avoid confusion, it is proposed to add a capability exchange TLV for this feature.

# Proposed Changes

[Modify the section 11.8.15 on page 1255, as follows]

| 11.8.15 Extended capability The extended capability field specifyies the extended capability support for the specified features. For each bit, a value of 0 indicates “not supported” while 1 indicates “supported”. If the TLV is not transmitted, the default value of each mentioned capability is “not supported”. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Type | Length | Value (variable-length) | Scope |
| Extended capability | 184 | 1 | Bit 0: This bit describes the capability to support ARQ Map Last Bit concept and the optimized Sequence Block as defined in Table 169. The feature is enabled only in case both MS and BS support it. Bit 1: This bit describes the capability to support BS_Controlled_HO (see 6.2.21.2.2). If the MS does not support this capability, it may ignore the BS_Controlled_HO flag in the DCD. Bits 12-7: Reserved, set to zero. | SBC-RSPREQ SBC-REQRSP |

# References

[IEEE802.16-Rev2/D6]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D6 (July 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.