| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Annex on FDD-TDD Coexistence using scheduling |  |
| Date Submitted | 2008-09-11 |  |
| Source(s) | Mariana Goldhamer Alvarion ltd. 21A, Ha Barzel Street Tel Aviv | E-mail: mariana.goldhamer@alvarion.com |
| Re: | 802.16 Rev 2 SB |  |
| Abstract |  |  |
| Purpose | Provide text for the associated comment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for Annex on FDD-TDD Coexistence using scheduling

Mariana Goldhamer

Alvarion Ltd.

# Introduction

In a number of frequency bands, as European 2.5-2.7GHz and 3.4-3.8GHz, the situations when the FDD networks and TDD networks are deployed in same geographical area are frequent enough to rise the problem of FDD-TDD interference.

This problem cannot be resolved with radio filters, especially in the case of MS-MS interference. MS receiver blocking at levels in the range -40dBm…-50dBm, measured as 6dB degradation of the sensitivity level, require a very high separation distance between the TDD and FDD MS.

The text below gives an example for resolving the TDD-FDD coexistence problem using scheduling.

# Text proposal

Insert:

## Annex N

Improving the FDD-TDD coexistence by appropriate scheduling

In a number of frequency bands, as European 2.5-2.7GHz and 3.4-3.8GHz, the situations when the FDD networks and TDD networks are deployed in same geographical area are frequent enough to raise the problem of FDD-TDD interference.

This problem cannot be resolved with radio filters, especially in the case of MS-MS interference. MS receiver blocking at levels in the range -40dBm…-50dBm, measured as 6dB degradation of the sensitivity level, require a very high separation distance between the TDD and FDD MS.

The approach for improved coexistence between wireless networks deployed in adjacent frequency channels or frequency bands combines:

- Procedures for the creation of an 802.16 Coordinated Coexistence Frame (CXCF), including:

  - A. protected intervals for the operation of the wireless networks affected by harmful interference

  - B. un-protected intervals

- Synchronization by using GPS or similar synchronization sources in order to achieve the absolute time synchronization of the Coordinated Coexistence Frame

- Scheduling the wireless communications such to reduce the experienced interference

The coexistence between the FDD MS and TDD MS is problematic due to the fact that mobile terminals can often be in each-other vicinity. In such a case, when one device is transmitting the other one can be in receive state. The lack of efficiency of the filters may cause the blocking of the radio receiver front end. Additionally, the out-of-channel transmissions of a terminal may enter within the operational frequency channel of the other terminal. These situations conduct to the interruption of the FDD and TDD MS communication with the Base Station, creating reciprocal harmful interference.

In fig. xx is presented an example for resolving the harmful interference by insertion of silence intervals within the FDD up-link transmissions and scheduling of the affected communication within the protected intervals. The Coexistence Frame used by a FDD network is shown in the upper part, while the CXCF used by a TDD network is presented in the lower part of this figure. The FDD network operates on two frequencies (the two upper rows), and its protected and un-protected allocations are shown on the rows below.

The transmissions of the interfering FDD and TDD MSs are scheduled at the same time.

Protected allocation #1 FDD

BS Tx, SS Rx

CCXF

Un-protected allocation #1 FDD

BS Tx/SS Rx

Un-protected allocation #2

FDD

Protected allocation #1

TDD

Un-protected allocation #1 TDD

Protected allocation #2

TDD

Un-protected allocation #2 TDD

Frame 4

BS Tx, SS Rx

Frame 1

BS Tx, SS Rx

Frame 2

BS Tx, SS Rx

Frame 3

BS Tx, SS Rx

802.16 Frame

SS Tx

SS Tx

SS Tx

SS Tx

FDD,

BS Tx, SS Rx

FDD,

SS Tx, BS Rx

Protected allocation #2 FDD

BS Tx, SS Rx

Protected allocation #1 FDD

SS Tx, BS Rx

Un-protected allocation #1 FDD

BS Tx/SS Rx

Silence

Protected allocation #2 FDD

SS Tx, BS Rx

Un-protected allocation #2 FDD

BS Tx/SS Rx

Silence

FDD,

BS Tx, SS Rx

FDD,

SS Tx, BS Rx

Frame 4

BS Tx, SS Rx

Frame 2

BS Tx, SS Rx

Frame 4

BS Tx, SS Rx

Frame 2

BS Tx, SS Rx

Frame 1

SS Tx

Frame 3

SS Tx

Silence

Silence

Frame 1

BS Tx, SS Rx

Frame 3

BS Tx, SS Rx

TDD

TDD

Fig. xx Example of FDD/TDD 802.16 protected allocations resolving SS-SS interference

The solution for the traffic scheduling uses the full TDD frames, while some silence intervals are scheduled during the up-link of the FDD transmissions. Taking into account that the BWA traffic is asymmetrical, the up-link being less loaded, the silence interval do not have an important influence on the system capacity.

End text insertion