| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Persistent HARQ DL/UL MAP IE |  |
| Date Submitted | 2008-11-11 |  |
| Source(s) | Li Wang, Hongyun Qu, Ling Xu, Sean Cai ZTE Corporation | Voice: E-mail: wangli@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Revision 2 |  |
| Abstract | HARQ related parameters should be removed from Persistent HARQ DL/UL MAP IE when disable HARQ. |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Persistent HARQ DL/UL MAP IE

Li Wang, Hongyun Qu, Ling Xu

ZTE Corporation

# Problem description

In the current standard IEEE 802.16 Rev2 D7, HARQ related parameters are always filled in the Persistent HARQ DL/UL MAP IE even if HARQ disable. That’s a waste of bandwidth especially have many sub-bursts. The Persistent HARQ DL/UL MAP IE refers to the HARQ DL/UL MAP IE, but it also support non-HARQ mode.

Additionally the element of SPID should be removed from Persistent HARQ DL/UL MAP IE when uses the persistent allocation because SPID is always zero at the first sub-packet transmission.

# Proposed solutions

HARQ related parameters should be removed from Persistent HARQ DL/UL MAP IE when HARQ disable. It should according to the element of ACK disable to determine if Fills the HARQ related parameters.

We propose that Uses similar mode to modify the table 365-371 in the section 8.4.5.3.29 and the table 442-449 in the section 8.4.5.4.30.

Proposed content changed in standard

[note to the editor: Make the following changes to IEEE P802.16Rev2/D7 as shown below;

Blue = editorial insertions relative to R2D7

8.4.5.3.29 Persistent HARQ DL MAP Allocation IE

### Table 365 - Persistent DL HARQ Chase Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_DL_HARQ_Chase_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource shifting indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (j=0;j<Number of sub bursts; j++) { |  |  |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation Flag == 0) { |  |  |
| RCID_IE() | variable | -- |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| If (Resource shifting indicator ==1) { |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID_IE() | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| Slot Offset | variable | 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| Sub-burst DIUC indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Sub-burst DIUC indicator ==1) { |  |  |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (ACK disable== 0) { |  |  |
| ACK channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| Dedicated DL control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of dedicated DL control indicator ==1){ |  | - |
| Duration (d) | 4 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d–1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE () | variable |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |

Table 366 – Persistent DL HARQ IR CTC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_DL_HARQ_IR_CTC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource shifting |
| For (j=0;j<Number of sub bursts; j++) { |  |  |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| Allocation Flag | 1 | 0 = de-allocate 1 = allocate |
| If (Allocation Flag == 0) { |  |  |
| RCID_IE() | variable | -- |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| if (Resource Shifting Indicator ==1) { |  |  |
| NEP | 4 |  |
| NSCH | 4 |  |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID IE() | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| NEP | 4 |  |
| NSCH | 4 |  |
| } |  |  |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| SPID | 2 |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (ACK disable== 0) { |  |  |
| ACK channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| If (Persistent Flag == 0){ |  |  |
| SPID | 2 | When persistent Flag == 1, MS shall regard SPID as 0. |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| Dedicated DL control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of dedicated DL control indicator ==1){ |  | - |
| Duration (d) | 4 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d–1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE () | variable |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |

Table 367 – Persistent DL HARQ IR CC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_DL_HARQ_IR_CC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (j=0;j<Number of sub bursts; j++) { |  |  |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation Flag == 0) { |  |  |
| RCID_IE() | variable | -- |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| if (Resource Shifting Indicator ==1) { |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID_IE() | variable | -- |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| Slot Offset | variable |  |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP NACK Channel index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| Sub-burst DIUC indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Sub-burst DIUC indicator==1) { |  |  |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| SPID | 2 | -- |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (ACK disable== 0) { |  |  |
| ACK channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| If (Persistent Flag == 0){ |  |  |
| SPID | 2 | When persistent Flag == 1, MS shall regard SPID as 0. |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| Dedicated DL control Indicator | 2 | LSB #0 indicates inclusion of CQI control LSB #1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of dedicated DL control indicator ==1){ |  | - |
| Duration (d) | 4 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d–1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE () | variable |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to 0. |
| } |  |  |

Table 368 – Persistent MIMO DL Chase HARQ Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_MIMO_DL_Chase_HARQ_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (j=1;j<Number of sub bursts; j++) { |  |  |
| MU Indicator | 1 | Indicates whether this DL burst is intended for multiple MS 0 = Single MS 1 = multiple MS |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| Dedicated MIMO DL Control Indicator | 1 | 0 == MS shall use the stored Dedicated MIMO DL Control information from the last burst allocation where this information was included. 1 = MS uses the Dedicated MIMO DL control information is this IE |
| If (MU Indicator == 0) { |  |  |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation flag == 0) { |  |  |
| RCID IE() | variable | -- |
| If (Resource Shifting Indicator== 1) |  |  |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (allocation Flag == 1) { |  |  |
| RCID IE() | variable | -- |
| If (Dedicated MIMO DL Control indicator == 1) { |  |  |
| Dedicated MIMO DL Control IE () | variable |  |
| } |  |  |
| Persistent flag | 1 | 0 = non-persistent allocation 1 = persistent allocation |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | See definition above in this IE |
| } |  |  |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| Slot Offset | variable | See definition above in this IE |
| ACK Disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit.The CRC shall be appended at the end of each subburst regardless of the ACK disable bit. |
| If (persistent flag ==1){ |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. . When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| } |  |  |
| For (i=0; i<N_Layers;i++) { |  |  |
| DIUC | 4 | -- |
| Repetition Coding Indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| If (ACK Disable == 0) { |  |  |
| ACK Channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| } |  |  |
| } |  |  |
| If (MU Indicator == 1) { |  |  |
| If (Dedicated MIMO DL Control indicator == 1) { |  |  |
| Dedicated MIMO DL Control IE () | variable |  |
| } |  |  |
| Layer Relevance Bitmap | 4 | 4 bit bitmap indicating if layer processing should be skipped. The bit position indicates the layer. The bit value: 0 = skip the layer; 1 = process the layer |
| For (i=0; i<N_Layers; i++) { |  | For each instance of the for-loop, when the corresponding bit in the Layer Relevance Bitmap is set to 0, the subsequent fields within this instance of for-loop are omitted. |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation flag == 0) { |  | De-allocate |
| RCID IE () | variable |  |
| If (Resource Shifting Indicator == 1) { |  |  |
| Slot Offset | variable | See definition above in this IE |
| Duration | variable | See definition above in this IE |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID IE () | variable |  |
| Persistent flag | 1 |  |
| Slot Offset | variable | See definition above in this IE |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | See definition above in this IE |
| } |  |  |
| ACK Disable | 1 | See definition above in this IE |
| DIUC | 4 | -- |
| Repetition Coding Indication | 2 | See definition above in this IE |
| If (ACK Disable == 0) { |  |  |
| ACK Channel | 8 |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| ACID | 4 |  |
| AI_SN | 1 |  |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period | 5 | See definition above in this IE |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | See definition above in this IE |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | See definition above in this IE |
| MAP NACK Channel Index | 6 | See definition above in this IE |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to zero |
| } |  |  |

Table 369: Persistent MIMO IR HARQ Subburst

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_MIMO_DL_IR_HARQ_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (j=0;j<Number of sub bursts; j++) { |  |  |
| MU indicator | 1 | Indicates whether this DL burst is intended for multiple MS 0 = Single MS 1 = multiple MS |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| Dedicated MIMO DL Control Indicator | 1 | 0 == MS shall use the stored Dedicated MIMO DL Control information from the last burst allocation where this information was included. 1 = MS uses the Dedicated MIMO DL control information is this IE |
| If (MU Indicator == 0) { |  |  |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation flag == 0) { |  |  |
| RCID IE() | variable | -- |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| If (Resource Shifting Indicator== 1) |  |  |
| NEP | 4 |  |
| NSCH | 4 |  |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID IE() | variable | -- |
| Persistent flag | 1 | 0 = non-persistent allocation 1 = persistent allocation |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| NEP | 4 |  |
| NSCH | 4 |  |
| } |  |  |
| Slot Offset | variable | See definition above in this IE |
| If (Dedicated MIMO DL Control indicator == 1) { |  |  |
| Dedicated MIMO DL Control IE () | variable |  |
| } |  |  |
| ACK Disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit.The CRC shall be appended at the end of each subburst regardless of the ACK disable bit. |
| If (Persistent Flag ==1){ |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| } |  |  |
| For (i=0; i<N_Layers;i++) { |  |  |
| NEP | 4 | -- |
| NSCH | 4 | -- |
| If (ACK Disable == 0) { |  |  |
| ACK Channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| If (Persistent Flag == 0){ |  |  |
| SPID | 2 | When persistent Flag == 1, MS shall regard SPID as 0. |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| SPID | 2 |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| } |  |  |
| If (MU Indicator == 1) { |  |  |
| If (Dedicated MIMO DL Control indicator == 1) { |  |  |
| Dedicated MIMO DL Control IE () | variable |  |
| } |  |  |
| Layer Relevance Bitmap | 4 | 4 bit bitmap indicating if layer processing should be skipped. The bit position indicates the layer. The bit value: 0 = skip the layer; 1 = process the layer |
| For (i=0; i<N_Layers; i++) { |  | For each instance of the for-loop, when the corresponding bit in the Layer Relevance Bitmap is set to 0, the subsequent fields within this instance of for-loop are omitted. |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation flag == 0) { |  |  |
| RCID IE () | variable |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| If (Resource Shifting Indicator == 1) { |  |  |
| NEP | 4 |  |
| NSCH | 4 |  |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID IE () | variable |  |
| Persistent flag | 1 |  |
| Slot Offset | variable | See definition above in this IE |
| ACK Disable | 1 | See definition above in this IE |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| NEP | 4 | -- |
| NSCH | 4 | -- |
| } |  |  |
| If (ACK Disable == 0) { |  |  |
| ACK Channel | 8 | -- |
| If (Persistent Flag == 0){ |  |  |
| SPID | 2 | When persistent Flag == 1, MS shall regard SPID as 0. |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| SPID | 2 | -- |
| ACID | 4 | -- |
| AI_SN | 1 | -- |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period | 5 | See definition above in this IE |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | See definition above in this IE |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | See definition above in this IE |
| MAP NACK Channel Index | 6 | See definition above in this IE |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to zero |
| } |  |  |

Table 370 – Persistent MIMO DL IR HARQ for CC Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_MIMO_DL_IR_HARQ_CC_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (j=0;j<Number of sub bursts; j++) { |  |  |
| MU indicator | 1 | Indicates whether this DL burst is intended for multiple MS 0 = Single MS 1 = multiple MS |
| Allocation Flag | 1 | 1 = allocate 0 = de-allocate |
| Dedicated MIMO DL Control Indicator | 1 | 0 == MS shall use the stored Dedicated MIMO DL Control information from the last burst allocation where this information was included. 1 = MS uses the Dedicated MIMO DL control information is this IE |
| If (MU Indicator == 0) { |  |  |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation flag == 0) { |  |  |
| RCID IE() | variable | -- |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| If (Resource Shifting Indicator== 1) |  |  |
| Duration |  | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (allocation Flag == 1) { |  |  |
| RCID IE() | variable | -- |
| Persistent flag | 1 | 0 = non-persistent allocation 1 = persistent allocation |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | See definition above in this IE |
| } |  |  |
| Slot Offset | variable | See definition above in this IE |
| Dedicated MIMO DL Control Indicator | 1 | -- |
| ACK Disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit.The CRC shall be appended at the end of each subburst regardless of the ACK disable bit. |
| If (Persistent Flag ==1){ |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| } |  |  |
| For (i=0; i<N_Layers;i++) { |  |  |
| DIUC | 4 | -- |
| Repetition Coding Indication | 2 | 0b00: No Repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| If (ACK Disable == 0) { |  |  |
| ACK Channel | 8 | Indicates the ACK channel to be used for this sequence of sub-bursts as defined in 8.4.5.4.25. |
| If (Persistent Flag == 0){ |  |  |
| SPID | 2 | When persistent Flag == 1, MS shall regard SPID as 0. |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| SPID | 2 |  |
| } |  |  |
| } |  |  |
| If (MU Indicator == 1) { |  |  |
| If (Dedicated MIMO DL Control indicator == 1) { |  |  |
| Dedicated MIMO DL Control IE () | variable |  |
| } |  |  |
| Layer Relevance Bitmap | 4 | 4 bit bitmap indicating if layer processing should be skipped. The bit position indicates the layer. The bit value: 0 = skip the layer; 1 = process the layer |
| For (i=0; I<N_Layers; i++) { |  | For each instance of the for-loop, when the corresponding bit in the Layer Relevance Bitmap is set to 0, the subsequent fields within this instance of for-loop are omitted. |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| If (Allocation flag == 0) { |  |  |
| RCID IE () | variable |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. |
| If (Resource Shifting Indicator == 1) { |  |  |
| Slot Offset | variable | See definition above in this IE |
| Duration | variable | See definition above in this IE |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| If (Allocation Flag == 1) { |  |  |
| RCID IE () | variable |  |
| Persistent flag | 1 |  |
| Slot Offset | variable | See definition above in this IE |
| ACK Disable | 1 | See definition above in this IE |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | See definition above in this IE |
| } |  |  |
| DIUC | 4 | -- |
| Repetition Coding Indication | 2 | See definition above in this IE |
| If (ACK Disable == 0) { |  |  |
| ACK Channel | 8 |  |
| If (Persistent Flag == 0){ |  |  |
| SPID | 2 | When persistent Flag == 1, MS shall regard SPID as 0. |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | Initial AI_SN for each ACID |
| } |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| AI_SN | 1 | -- |
| SPID | 2 | -- |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period | 5 | See definition above in this IE |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | See definition above in this IE |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | See definition above in this IE |
| MAP NACK Channel Index | 6 | See definition above in this IE |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to zero |
| } |  |  |

Table 371 – Persistent MIMO DL STC HARQ Subburst IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Persistent_MIMO_DL_STC_HARQ_Sub-Burst_IE() { |  |  |
| N sub burst | 4 | Number of sub-bursts in the 2D rectangular region is this field value plus 1 |
| Resource Shifting Indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| For (j=0; j<Number of sub bursts; j++) { |  | -- |
| Allocation Flag | 1 | -- |
| Group Indicator | 1 | TDD mode: reserved, set to 0. Used for FDD/H-FDD case only; to indicate the group assignment of the MS (see 8.4.4.1 and 8.4.4.1.1) 0b0: Group #1 0b1: Group #2 |
| if (Allocation Flag == 0) { |  | // De-allocate |
| RCID_IE () | variable |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| If (Resource Shifting Indicator ==1) { |  | // resource shifting is allowed |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Retransmission Flag | 1 | 0: Deallocation command in Relevant Frame 1: Retransmission of deallocation command in Relevant Frame – Allocation Period |
| } |  |  |
| } |  |  |
| if (Allocation Flag ==1) { |  | // allocation |
| RCID_IE() | variable |  |
| Persistent Flag | 1 |  |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. When ACK Disable == 0 and persistent Flag == 1, MS shall regard SPID as 0. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (Power boost per sub-burst ==1) { |  |  |
| Boosting | 3 | 0b000: Normal (not boosted) 0b001: +6dB 0b010: –6dB 0b011: +9dB 0b100: +3dB 0b101: –3dB 0b110: –9dB 0b111: –12dB; Note that if the Persistent flag is set, the boosting value applies to each instance of the persistent allocation. |
| } |  |  |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this subburst. Otherwise, this subburst will use the same allocation period as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation period (ap) | 5 | Period of the persistent allocation is this field value plus 1 (unit is frame) |
| If (ACK disable== 0) { |  |  |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. |
| MAP NACK Channel Index | 6 | Index to a shared MAP NACK channel within the Fast Feedback region. When MAP NACK Channel Index = 0b111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| } |  |  |
| Tx count | 2 | Tx count shall be set to ‘0’ when Persistent Flag is set to ‘1’. |
| Duration Indicator | 1 | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst. Otherwise, this subburst will use the same Duration as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Duration Indicator == 1) { | - | - |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| } |  |  |
| Slot Offset | Variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| ACK disable | 1 | When ACK Disable == 1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the subburst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN and SPID, which shall be set to 0 by BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| if (Tx count == 0) { |  |  |
| Dedicated MIMO DL Control indicator | 1 | -- |
| If (Dedicated MIMO DL Control indictor ==1) { |  |  |
| Dedicated MIMO DL Control IE () | variable |  |
| } |  |  |
| DIUC | 4 | -- |
| Repetition Coding Indicator | 2 | -- |
| } |  |  |
| If (ACK disable == 0) { |  |  |
| ACID | 4 | Initial value of HARQ channel identifier |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to nibble; shall be set to zero |
| } |  |  |

The table 442-449 in the section 8.4.5.4.30 should be modified in the similar way.

# References

IEEE Std 802.16Rev2-D7 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” October, 2008.