| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixes to HO management encodings |  |
| Date Submitted | 2008-09-11 |  |
| Source(s) | Achim Brandt, Aik Chindapol Nokia Siemens Networks | Voice: E-mail: achim.brandt@nsn.com Aik.chindapol@nsn.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract | This contribution proposes fixes for HO management encodings. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D6a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Fixes to HO management encodings

Achim Brandt, Aik Chindapol

Nokia Siemens Networks

# Problem description

The current “Value” entries for Resource_Retain_time and Alternate_Target_BS TLVs are not correct. They need to be swapped.

# Proposed Changes

[Modify the section 11.15 HO Management Encodings on page 1323, as follows]

11.15 HO management encodings

11.15.1 Resource_Retain_Time

The Resource_Retain_Time is the duration for MS’s connection information that will be retained in serving

BS. This value is measured in 100 ms. If this value is set to 0, the serving BS will retain the MS’s connection

information during Resource Retain Time negotiated at early registration stage. If this value is set to

nonzero, it is the proposed Resource Retain Time by serving BS and the serving BS will retain the MS’s

connection information during that time after reception of MOB_HO-IND message (HO_IND_type=0b00).

| Type | Length | Value (variable-length) | Scope |
| --- | --- | --- | --- |
| 1 | 2 | bits 63-16: Target_BS_ID bits 15-0: Preamble Index/Subchannel Index 0: The serving BS will retain the MS’s connection information during Resource Retain Time negotiated at registration stage. 1-65535: Resource Retain Time (100 ms unit) | MOB_BSHO-REQ MOB_BSHO-RSP |

11.15.2 Alternate Target BS

The MS may include the Alternate_Target_BS TLV in a MOB_HO-IND message in which the

HO_IND_type is set to 0b10 (HO reject, see 6.2.2.3.50) in order to indicate its preferred HO target. If the

MS includes this TLV, the MS shall set bits 63-16 to the BS_ID of the proposed HO target, and shall set bits

15-0 to the preamble of the target BS.

| Type | Length | Value (variable-length) | Scope |
| --- | --- | --- | --- |
| 1 | 8 | 0: The serving BS will retain the MS’s connection information during Resource Retain Time negotiated at registration stage. 1-65535: Resource Retain Time (100 ms unit) bits 63-16: Target_BS_ID bits 15-0: Preamble Index/Subchannel Index | MOB_HO-IND |

# References

[IEEE802.16-Rev2/D6]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D6 (July 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.