| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handoff Average CINR Computations |  |
| Date Submitted | 2008-09-11 |  |
| Source(s) | Louay Jalloul, (Beceem) Djordje Tujkovic, Wee Peng Goh (Nextwave) | Voice: +1 (408) 387-5048 E-mail: jalloul@beceem.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract |  |  |
| Purpose | Review and approve for 802.16 Revision 2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Computation of the Average and Standard Deviation of the CINR for Band AMC Operation

Louay Jalloul, Djordje Tujkovic, (Beceem)

Wee Peng Goh (Nextwave)

# Introduction

The method for computing the average CINR for handoff as outlined in the IEEE 802.16e-2006, Rev2/D6 is performed by averaging instantaneous ratios of signal power to noise plus interference power, this type of averaging results in a bias and will impact the system performance.

# Problem Definition for Average CINR

The IEEE 802.16e standard specifies that mean CINR for handoff shall be derived from the multiplicity of single messages. A single-pole IIR filter is used to average of instantaneous CINRs. This suggests that mean CINR for handoff is derived as the expectation of instantaneous ratios of signal power and noise plus interference power (expectation of ratios, EOR) as given by

This type of averaging over fading channels deviates from the true signal-to-noise ratio defined as the mean (expectation) of signal power divided by the mean (expectation) of noise plus interference power (ratio of expectations, ROE).

The amount of bias from true CINR is not constant but instead depends on statistics of desired and interfering BS channels as shown in Appendix. Consequently, handoff decisions will not be consistent for users throughout the cell which will affect the system performance.

Figure 1 shows an example of the bias introduced by the currently specified method in the standard for averaging the CINR for handoff. The serving BS channel is modeled according to Veh-A power delay profile while interfering BS channel is assumed to be frequency flat. The subscriber is moving at 60Km/hr. The method currently suggested in standard would result in CINR bias of as much as 8dB. The figure also depicts the performance for averaging logarithmic values of instantaneous ratios of signal power to noise plus interference power (denoted as EO[dB]R in figure). This method is often discussed as an alternative method to alleviate problems with standard defined metric for average SINR. As seen from Figure 1 (also shown analytically in Appendix), this method suffers from similar problem with bias. In turn, the ROE proposed here results in an accurate averaging without any bias.

Figure 1: Average SINR for different averaging methods

# Proposed Resolution for Handoff Average CINR and Text Changes

The average CINR used for handoff should be calculated as a dB value of the IIR averaged received signal power divided by the IIR averaged noise plus interference power.

Modify the text on line 47 page 1122 of Rev2/D6 to:

When the MS is required to measure CINR of the i-th BS for handover, the mean CINR statistic (in dB) shall be derived from a multiplicity of single messages using Equation (155).

(155)

where

Change equation (156) to the following:

(156)

Add the following text before line 60 page 1122 of Rev2/D6:

where  is the average RSSI of the desired BS,  is the average RSSI of the  interfering BS and  is the variance of the noise.

Add the following text after line 61 page 1122 of Rev2/D6:

Whenever measurement is missing in a given frame, average values of received Signal power and Noise plus Interference power are repeated from the previous frame. The RSSI and IIR averaged received signal and IIR averaged Noise plus interference power for calculating the average CINR should be calculated from multiplicity of single messages as

where n denotes the number of consecutive frames in which no measurement is made. In frames where no measurement is made, the MS shall report the latest averaged results.

Appendix (informative and not to be included in the standard text)

Expectation of ratios versus ratio of expectations

The ratio of the expected signal and expected interference power (ROE) and expectation of instantaneous ratios of signal and interference powers (EOR) are given as

ROE=CINR=ESEI 	(4)

EOR=ESI 	(5)

The ROE in (4) is also marked as the true CINR which averages out fast fading variations without changing the relative ratio of individual mean BS powers. Let us zoom into EOR:

+∞1ipI(i)di 	(6)

where the last two steps stem from fact that signal and interference power are independent. The interference power is a chi-squared random variable with M degrees of freedom. Hence, the probability density function (PDF) pI(i)of the inverse of interference power is given by the so called inverse chi-squared distribution

pI(i;M)=2−M2Γ(M2)i−M2−1e−1(2i)	(7)

whose mean is given by

+∞1ipI(i,M)di=1EIMM−2	(8)

We finaly write the EOR as

EOR=ESEIMM−2CINRMM−2	(9)

Notice that for flat fading channel, where the number of degrees of freedom is equal to M=2, the EOR grows to infinity

EOR|M=2→∞	(10)

Applying the IIR filter, which acts as a moving average filter will to some extent, bound the bias in EOR mean CINR. Nevertheless, the impact from bias will be still rather dramatic in practice.

As an alternative method to ROE filtering, another method denoted hereafter by expectation of dB ratio (EOdBR) is often discussed. In this method the dB instead of linear value of instantaneous CINR measurement is passed through IIR filter in section 8.4.11.3

EOdBR=ElogSI	(6)

Without loss of generality, let us assume that instantaneous signal and inference powers can be represented as the product of constant representing the mean power and the random chi-square variable with unit power with given number of degrees of freedom. That is

S=S0χMs2I=I0χMI2	(7)

with

S0=ESI0=EIE(χMs2)=1E(χMI2)=1	(8)

where MS and MI denote the number of degrees of freedom in signal and interference channels, respectively. Let us zoom into EOdBR from (6)

EOdBR=Elog(S)−Elog(I)Elog(S0)+Elog(χMS2)−Elog(I0)−Elog(χMI2)ElogS0I0+Elog(χMS2)−Elog(χMI2)logS0I0+ΔMS,MI	(9)

With assumptions in (7) and (8), the dB value of true CINR (ROE), denoted hereafter by ROEdB, is given by

ROEdB=logESEIlogS0I0	(10)

Comparing (9) and (10), it is apparent that in general EOdBR has bias compared to true CINR (ROEdB). The sign and value of bias depends on the number of degrees of freedom in signal and interference channel. Only in the special case when MS and MI are equal, the bias term is equal to zero.

Therefore, the ROE represents the only liable bias free solution.