| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on periodic ranging flow charts |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai Institute for Information Industry Mark Marsan, Joseph Schumacher Motorola | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | P802.16 Revision 2 |  |
| Abstract | Comment on periodic ranging |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on periodic ranging flow charts

Kanchei(Ken) Loa, Yi-Hsueh Tsai
Institute for Information Industry
Mark Marsan, Joseph Schumacher
Motorola

Introduction

In Figure 1 (see Figure 100 of P802.16REV2/D8), the BS will send the unsolicited RNG-RSP message in order to initiate the SS to send ranging code when the “Ranging code not received”. However, when MS perform periodic ranging, the MS shall choose randomly a Ranging Slot (with random selection with equal probability from available Ranging Slots in a single frame) at the time to perform the ranging, then it chooses randomly a Periodic Ranging Code and sends it to the BS (as a CDMA code), therefore, the periodic ranging code sent from the MS is anonymous. Thus the BS can not know which MS has already sent the periodic ranging code. As a result the, Figure 100 of P802.16REV2/D8 is unable to implement. In addition, Figure 101 is a subset of Figure 100. This contribution proposes the revise the flow charts about handling periodic ranging at the BS (Figure 100 and Figure 101).

Figure 1 “Figure 100” in P802.16REV2/D8

Figure 2 “Figure 101” in P802.16REV2/D8

Spec Change

[Delete Figure 100 and Figure 101]

[Insert following figure as indicated]

NOTE —Means ranging is within the tolerable limits of the BS.

Figure 100—Handle CDMA periodic ranging code at BS