| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on Persistent Allocation IE Format |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | Zhaohua Lu, Li Wang, Sean Cai Ying Liu, Hongyun Qu ZTE Corporation | Voice: E-mail: lu.zhaohua@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Revision 2 |  |
| Abstract | HARQ related parameters should be removed from Persistent HARQ DL/UL MAP IE when disable HARQ. |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on Persistent Allocation IE Format

Zhaohua Lu, Li Wang, Sean Cai, Ying Liu, Hongyun Qu

ZTE Corporation

# Problem description

In current P80216Rev2_D7, there are redundant IE format design on resource de-allocation of persistent allocation, for example, Persistent DL HARQ Chase Subburst IE format in Page 798, Persistent DL HARQ IR CTC Subburst IE format in Page 804, Persistent DL HARQ IR CC Subburst IE format in Page 807, Persistent MIMO DL Chase HARQ Subburst IE format in Page 811, Persistent MIMO DL IR HARQ Subburst IE format in Page 816, Persistent MIMO DL IR HARQ CC Subburst IE format in Page 822, Persistent MIMO DL STC HARQ CC Subburst IE format in Page 826, Persistent UL HARQ Chase Subburst IE format in Page 916, etc.

These IEs all have similar resource de-allocation format design like Table 1, as we know that MS has stored Resource allocation information when it come into PA process, so if field “Resource shifting indicator = 1”, as shows in Table 2, there is no need to transmit field “RCID_IE()”, because MS can know if it should or not use pre-allocated persistent allocation resource by de-allocation resource information, which is described by field “Duration” and “Slot Offset”. That is to say, when MS receive relevant PA IE, and get that “Resource shifting indicator = 1” and de-allocation resource information by field “Duration” and “Slot Offset”, if these de-allocation resource have been allocated to it for persistent allocation, it should not use these resource, and vice versa.

Table 1 Current PA IE Format for Resource De-allocation

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Resource shifting indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| for ( j=0; j<Number of sub bursts; j++) { | __ | __ |
| … |  |  |
| if( Allocation Flag = = 0){ | __ | __ |
| RCID_IE() | variable | This field is redundant if “Resource shifting indicator = 1”, because MS can know if it should or not use pre-allocated persistent allocation resource by de-allocation resource information, which is described by field “Duration” and “Slot Offset”. |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| If (Resource shifting indicator = =1) { | __ | __ |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| … |  |  |
| } | __ | __ |
| … |  |  |
| } | __ | __ |
| … |  |  |
| } | __ | __ |
| … |  |  |

Table 2 Suggested PA IE Format for Resource De-allocation

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Resource shifting indicator | 1 | 0 = No Resource Shifting 1 = Resource Shifting |
| for ( j=0; j<Number of sub bursts; j++) { | __ | __ |
| … |  |  |
| if( Allocation Flag = = 0){ | __ | __ |
| if(Resource Shifting Indicator = = 0){ | __ | __ |
| RCID_IE() | variable |  |
| } | __ | __ |
| MAP ACK Channel Index | 6 | Index to a MAP ACK channel within the Fast Feedback region. When MAP ACK Channel Index = 0b111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| if (Resource shifting indicator = =1) { | __ | __ |
| Duration | variable | Duration in slots. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| Slot Offset | variable | Indicates the start of this persistent allocation in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the HARQ region. OFDMA Frame duration dependant 7 bits – 2.5 ms frame 8 bits – 5 ms frame 9 bits – 10 ms frame 10 bits – 20 ms frame |
| … |  |  |
| } | __ | __ |
| … |  |  |
| } | __ | __ |
| … |  |  |
| } | __ | __ |
| … |  |  |

# Proposed solutions

According above analysis, we suggest to use simplified IE format design in resource de-allocation of persistent allocation, just like Table 2.

Proposed content changed in standard

[note to the editor: Make the following changes to IEEE P802.16Rev2/D7 as shown below;

Blue = editorial insertions relative to R2D7

# References

IEEE Std 802.16Rev2-D7 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” October, 2008.