| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MBS DATA Time Diversity IE Corrections |  |
| Date Submitted | 2008-11-12 |  |
| Source(s) | Erik Colban NextWave Wireless Inc.* | Voice: E-mail: ecolban@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Sponsor Ballot Recirculation 1 (as announced in IEEE 802.16-08/052) |  |
| Abstract | IEEE 802.16 standard specifies an MBS DATA Time Diversity IE. This IE has several deficiencies that this contribution attempts to correct. |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MBS DATA Time Diversity IE Corrections

Erik Colban, Yair Bourlas, Ken Stanwood, Lei Wang

NextWave Wireless Inc.

# Background

The MBS DATA Time Diversity IE (see section 6.3.2.3.52) is inefficient.  We propose to redesign this IE using vertical optimization techniques used by other REV2 IEs as follow:

- Typically the IE specifies several sub-bursts for the same MCID. For each sub-burst, a12-bit MCID is added to the IE, which is inefficient. We propose to add a 1-bit flag to indicate whether the MCID for the current sub-burst is different from the preceding sub-burst and only include the MCID if it has changed.

- Typically, the sub-bursts in the allocation will have the same NEP sand NSCH values. We propose to add a 1-bit flag to indicate whether the MCID for the current sub-burst is different from the preceding sub-burst and only include the MCID if it has changed.

- Next MBS frame offset or Next MBS Symbol offset only changes if the MCID changes and may remain unchanged even when the MCID has changed. We propose to add a 1-bit flag to indicate whether the Next MBS frame offset or Next MBS Symbol offset for the current sub-burst is different from the preceding sub-burst and only include the Next MBS frame offset and Next MBS Symbol offset if it has changed.

The efficiency improvement resulting from this redesign is considerable. The size of an MBS DATA Time Diversity IE that describes 4 sub-bursts for the same MCID is halved (96 bits vs. 192 bits). If the size of the next MBS-MAP message for the MCID is different from the current MBS-MAP, the reduction is even greater (108 bits vs. 240 bits).

# Proposed Changes

[Page 257, line 35 through page259, line 12, modify text as follows:]

Table 159—MBS DATA Time Diversity IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MBS_DATA_Time_Diversity_IE() { | -- | -- |
| MBS_MAP Type = 1 | 2 | See Table 156 |
| MBS Burst Frame Offset | 2 | This indicates the burst located by this IE will be shown after MBS Burst Frame offset + 2 frames |
| OFDMA symbol offset | 8 | This indicates starting position of the region of MBS bursts with respect to start of the next (MBS Burst Frame offset + 2)-th frame. |
| # of Data Sub-bursts | 4 | n = # of Data Sub-bursts with the same frame offset |
| for( i = 0; i < n; i++ ){ | -- | -- |
| Multicast MCID change indication | 1 | Indicates if the MCID for this sub-burst is different from the MCID for sub-burst in the preceding iteration of this for-loop. The first instance of this field within this IE shall be set to 1. |
| if (Multicast MCID change indication == 1) { | -- | -- |
| Multicast CID | 12 | 12 LSBs of CID for multicast |
| } |  |  |
| NEP, NSCH change indication | 1 | Indicates if the NEP or NSCH for this sub-burst is different from the NEP or NSCH in the preceding iteration of this for-loop. The first instance of this field within this IE shall be set to 1. |
| if (NEP, NSCH change indication == 1) { | -- | -- |
| NEP | 4 | See Table 533 |
| NSCH | 4 | See Table 533 |
| } | -- | -- |
| AI_SN | 1 | -- |
| SPID | 2 | -- |
| ACID | 4 | -- |
| Next MBS MAP change indication | 1 | This indicates whether the size of MBS MAP message of next MBS frame for this multicast CID will be different from the size of this MBS MAP message. |
| Next MBS offset change indication | 1 | Indicates whether the Next MBS frame offset or Next MBS Symbol offset are different from the offsets in the preceding iteration of this for-loop. The first instance of this field within this IE shall be set to 1. |
| if (Next MBS offset change indication == 1) { | -- | -- |
| Next MBS frame offset | 8 | -- |
| Next MBS OFDMA Symbol offset | 8 | -- |
| } | -- | -- |
| if (Multicast MCID change indication & Next MBS MAP change indication == 1) { | -- | -- |
| Next MBS No. OFDMA symbols | 6 | Indicates the size of MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| Next MBS No. OFDMA subchannels | 6 | Indicates the size of MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| } | -- |  |
| Reserved | 1 |  |
| } | -- |  |
| } | -- |  |