| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of byte ordering of some TLV encodings |  |
| Date Submitted | 2008-10-29 |  |
| Source(s) | Yeongmoon Son, Brian Shim Samsung Electronics | Voice: +82-31-279-5845 E-mail: ym1004.son @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Recirculation |  |
| Abstract | This contribution defines operation required for emergency service. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D7. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of byte ordering of some TLV encodings

Yeongmoon Son, Brian Shim

Samsung Electronics

# Problem description

It is defined in IEEE802.16REV2/D7 that a BS can inform MSs when to broadcast ‘Emergency Service Message’ (i.e. ESM) by using Broadcast Control Pointer IE as shown in the table 360 on page 790.

We have so far tried to eliminate ambiguity in decoding TLVs with multiple bytes of Value by replacing the byte-order definition with bit-wise definition. Unfortunately, there are still some TLV encodings that cause ambiguity in the interpretation of byte order as follows.

- Downlink Operational Burst Profile (See table 580 on page 1204)

- Downlink Operational Burst Profile for OFDMA (See table 580 on page 1206)

- Maximum Tx power (See 118.3.2 on page 1226)

Which one does 'Byte #0' mean? Least significant 8 bits or most significant 8 bits?

Fortunately, there is a good example for byte order of TLV encoding  regarding above issue.

If we look at table 600 on page 1276, 'Byte #0' is ‘Least Significant byte’ or ‘Least Significant 8 bits’.

Therefore, we can apply the same rule (i.e. Byte #0 is Bit #0-#7) to the above-mentioned three TLV encodings for the consistency with other TLV encodings.

# Proposed Changes

[Modify the table 600 on page 1276, as follows]

| Table 600—Multicast assignment request message encodings |  |  |  |
| --- | --- | --- | --- |
| Name | Type (1 byte) | Length | Value (variable-length) |
| … | … | … | … |
| Periodic allocation parameters | 4 | 4 | Byte #0 (LS byte)Bit #0-#7 = m Byte #1Bit #8-#15= k Byte #2Bit #16-#23= n Byte #3Bit #24-#31= Reserved; shall be set to zero |
| … | … | … | … |

[Modify the table 580 on page 1204, as follows]

| Name | Type (1 byte) | Length | Value (variable-length) | PHY Scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| Downlink Operational Burst Profile | 7 | 2 | This parameter is sent in response to the RNG-REQ Requested Downlink Burst Profile parameter. Byte 0Bit #0-#7: Specifies the least robust DIUC that may be used by the BS for transmissions to the SS. Byte 1Bit #8-#15: Configuration Change Count value of DCD defining the burst profile associated with DIUC. | All |

[Modify the table 580 on page 1206, as follows]

| Name | Type (1 byte) | Length | Value (variable-length) | PHY Scope |
| --- | --- | --- | --- | --- |
| Downlink Operational Burst Profile for OFDMA | 33 | 2 | Is sent in response to the RNG-REQ Requested Downlink Burst Profile parameter. Byte 0: Bits #0–#3: Specifies the least robust DIUC that may be used by the BS for transmissions to the MS. Bit s #4–#7: Specifies Repetition Coding Indication: 0b0000 – No repetition coding 0b0001 – Repetition coding of 2 0b0010 – Repetition coding of 4 0b0011 – Repetition coding of 6 The repetition coding indication shall be 0b0000 if the DIUC refers to modulations higher than QPSK. Byte 1Bit #8-#15: Configuration Change Count value of DCD defining the burst profile associated with DIUC. | All |

[Modify the section 11.8.3.2 on 1226, as follows]

11.8.3.2 Maximum Tx power

The maximum available power for BPSK, QPSK, 16-QAM, and 64-QAM constellations. The maximum power parameters are reported in dBm and quantized in 0.5 dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. SSs that do not support QAM64 shall report the value of 0x00 in the maximum QAM64 power field. This parameter is only applicable to systems supporting the OFDM or OFDMA PHY specifications.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 3 | 4 | Byte 0Bit #0-#7: Maximum transmitted power for BPSK. Byte 1Bit #8-#15: Maximum transmitted power for QPSK. Byte 2Bit #16-#23: Maximum transmitted power for 16-QAM. Byte 3Bit #24-#31: Maximum transmitted power for 64-QAM. SSs that do not support 64-QAM shall report the value 0x00. | SBC-REQ |

# References

[IEEE802.16-Rev2/D7]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D4 (April 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.