| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications in MBS Section |  |
| Date Submitted | 2008-11-11 |  |
| Source(s) | Vladimir Yanover Alvarion Ltd. | Voice: +972-36457834 E-mail: vladimir.yanover@alvarion.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 REV2/D7 Sponsor Ballot |  |
| Abstract | Text Clarifications in MBS Section |  |
| Purpose | Sponsor Ballot comment resolution |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarifications in MBS Section

Vladimir Yanover

Alvarion Ltd.

# Problem Statement

The text at p. 479, section 6.3.22, line 20 needs some clarification (see highlighted comments):

For all BSs that belong to the same MBS Zone, the following coordination shall be assured:

Mapping of SDUs into the MBS Bursts should be identical,

Does it mean that MBS bursts must be identical at two different BSs meaning they occupy same OFDMA slots in the frame and have same PHY properties (option A below)? An alternative could be that a set of SDUs is transmitted in two bursts in different BSs is the same but these bursts does not necessarily occupy same OFDMA slots and have same PHY properties (option B below);

Figure 1. Option A

Figure 2. Option B

and the same SDU’s shall be transmitted in the same frame in all BS in the same MBS Zone;

# Specific Text Changes

[Change in 6.3.22 p. 479, line 23]

.

•	The set of MAC SDUs carrying MBS content shall be identical in the same frame in all BS in the same MBS zone;

•	The mapping of MAC SDUs carrying MBS content onto MAC PDUs shall be identical in the same frame in all BS in the same MBS Zone meaning in particular identical SDU fragments and identical fragment sequence number (block sequence number) and fragment size