| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Entry in the Presence of Frequency Shift Repeater |  |
| Date Submitted | 2008-10-29 |  |
| Source(s) | Toshiyuki Kuze, Kentarou Sawa Mitsubishi Electric Corp Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Lab | Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Voice: 617-621-{7557, 7595} Fax: 617-621-7550 {tao, jzhang}@merl.com |
| Re: | IEEE802.16Rev2/D7 Sponsor Ballot |  |
| Abstract | IEEE 802.16 Repeater and Standard Clarification in Rev2 |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Network Entry in the Presence of Frequency Shift Repeater

Toshiyuki Kuze, Kentarou Sawa

Mitsubishi Electric Corp

Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Lab

# Introduction

This contribution explains the network entry operation, when frequency shift repeater (FSR) has been deployed in the network.

In contribution C802.16maint-08/295r2 [1], we propose to introduce a new TLV called FSR frequency TLV.  This TLV is broadcast by the BS in DCD/UCD message to indicate the frequency band used by the FSR to communicate with mobiles.  For the sake of simplicity, we will categorize a device (i.e., MS or BS) as Rev2 device or legacy device, depending on whether or not it can support this new FSR frequency TLV.  For example, those MSs that can understand FSR frequency TLV will be referred to as Rev2 MSs.

For simple FSRs, both the outdoor and indoor frequency should be specified in advance by operator. Therefore, the mapping relation between outside and inside frequency would be fixed.  In this contribution, only simple FSRs following this fixed mapping approach will be considered.

# Network Entry

## Rev2 MS and Rev2 BS

- Network entry in outdoor environment

As illustrated in Figure 1, when MS attempts to enter network in outdoor environment, it will connect to outdoor frequency (e.g., F2).  MS decodes the DCD/UCD and gets the center frequency TLV.  Since the frequency (e.g., F2) which MS synchronizes with is equal to the one contained in center frequency TLV, MS shall conclude that it is communicating with BS directly.

Figure 1: Network Entry (outdoor)

- Network entry via FSR

As shown in Figure 2, when MS attempts to enter network in indoor environment during idle mode, it will end up synchronizing with indoor frequency (e.g., F1) after wake-up.  MS will decode DCD/UCD and get the center frequency TLV and FSR frequency TLV.  The MS compares the value of the synchronization frequency with center frequency TLV, and finds that they are different.  Then MS compares the value of the synchronization frequency with the FSR frequency TLV, and finds that these values are matched. Thus MS concludes that it is communicating with BS through an FSR.

Figure 2: Network Entry (indoor)

# Proposed Text

[Insert the following new subclause]

6.3.27 FSR support

Frequency shift repeaters (FSR) use one frequency to communicate with BS, and use a different frequency to talk to MSs. As shown in FigureX, for example, repeaters convert the frequency F2 to frequency F1 for downlink, and vice versa for uplink communications.

Figure X: Frequency shift repeater

For simple FSRs, both the outdoor and indoor frequency should be specified in advance by operator. Therefore, the mapping relation between outside and inside frequency would be fixed.

6.3.27.1 Synchronization

When MS synchronizes with indoor frequency (e.g., F1), MS will decode DCD/UCD and get the center frequency TLV and FSR frequency TLV.  The MS compares the value of the synchronization frequency with center frequency TLV, and finds that they are different.  Then MS compares the value of the synchronization frequency with the FSR frequency TLV, and finds that these values are matched. Thus MS concludes that it is communicating with BS through an FSR.

# Reference

- Toshiyuki Kuze et al., “IEEE 802.16 Repeater and Standard Clarification in Rev2”, IEEE C802.16maint-08/295r2, Kobe, Japan September 2008