| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover in the Presence of Frequency Shift Repeater |  |
| Date Submitted | 2008-10-29 |  |
| Source(s) | Toshiyuki Kuze, Kentarou Sawa Mitsubishi Electric Corp Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Lab | Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Voice: 617-621-{7557, 7595} Fax: 617-621-7550 {tao, jzhang}@merl.com |
| Re: | IEEE802.16Rev2/D7 Sponsor Ballot |  |
| Abstract | IEEE 802.16 Repeater and Standard Clarification in Rev2 |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Handover in the Presence of Frequency Shift Repeater

Toshiyuki Kuze, Kentarou Sawa

Mitsubishi Electric Corp

Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Lab

# Introduction

This contribution explains both the inter-FA and intra-FA handover operation, when frequency shift repeater (FSR) has been deployed in the network.  In addition, intra-cell handover, which is the handover between outdoor frequency and indoor frequency, will also be discussed in this contribution.

In a network that uses FSR, the concept intra-FA handover and inter-FA handover are defined for the outdoor frequencies. Specifically, if the target outdoor frequency is same as the serving outdoor frequency, it’s intra-FA handover. Meanwhile, if the target outdoor frequency is different from the serving outdoor frequency, it’s inter-FA handover.  Note that MS may handover to the indoor frequency that FSR uses as well.

In contribution C802.16maint-08/295r2 [1], we propose to introduce a new TLV called FSR frequency TLV.  This TLV is broadcast by the BS in DCD/UCD message to indicate the frequency band used by the FSR to communicate with mobile stations.  For the sake of simplicity, we will categorize a device (i.e., MS or BS) as Rev2 device or legacy device, depending on whether or not it can support this new FSR frequency TLV.  For example, those MSs that can understand FSR frequency TLV will be referred to as Rev2 MSs.

For simple FSRs, both the outdoor and indoor frequency should be specified in advance by operator. Therefore, the mapping relation between outside and inside frequency would be fixed in the operator network.  In this contribution, only simple FSRs following this fixed mapping approach will be considered.

# Handover

## Rev2 MS and Rev2 BS

      - Intra-FA Handover

Figure 1 illustrates the scenario of intra-FA handover that may involve an FSR.

Figure 1: Intra-FA handover

When an MS receives an FSR center frequency TLV in DCD/UCD message in the serving cell, it knows that FSR may exist in the serving cell.  Similarly, when an MS receives an FSR center frequency TLV in MOB_NBR-ADV message, it learns that FSR may exist in the corresponding neighbor cell.

In the intra-FA handover, MOB_NBR-ADV message can list two target handover frequency for Rev2 MS.  One is outdoor frequency (e.g., F2), and another is indoor frequency (e.g., F1).

- HO from BS#2 to BS#1

  - BS can use outdoor “center frequency TLV “(e.g., F2) in DCD/UCD of the serving cell to start the intra-FA handover operation.

  - If MS cannot synchronize with the target intra-FA HO frequency (e.g., F2), or the signal quality is not good enough on that frequency (e.g., F2), MS may seek handover to FSR indoor frequency (e.g., F1).

- HO from BS#1 to BS#2

  - BS can use center frequency TLV in DCD/UCD of the serving cell to start the intra-FA handover operation.

      - Inter-FA Handover

In the inter-FA handover, the center frequency TLV for outdoor environment of the target cell is included in the MOB_NBR-ADV message. And in order to specify the FSR frequency, BS may include the FSR center frequency TLV in this MOB_NBR-ADV message.

Figure 2: Inter-FA handover

Figure 3: Message flow for inter-FA handover

      - Intra-Cell Handover

Intra-cell handover is the handover between the BS and FSR.  Since FSR and BS belong to the same cell, this handover is called intra-cell handover.

- Outdoor to indoor intra-cell handover

Figure 4 shows the intra-cell handover from outdoor to indoor.

Figure 4: Intra-cell handover from outdoor to indoor

DCD/UCD approach: When MS moves from outdoor to indoor, it may get disconnected and start the hard handover process.  If MS gets synchronized with the indoor frequency (e.g., F1), MS decodes DCD/UCD and compares the BSID in F2 with BSID in F1.  If BSID for outdoor frequency and indoor frequency are the same, MS notices it is an intra-cell handover.  In this intra-cell handover, MS will start the re-entry process instead of the hard handover process.  The detailed message flow is shown in Figure 5 .

Figure 5: Intra-cell handover from outdoor to indoor (DCD/UCD approach)

NBR-ADV approach: As an alternative, if BS includes the FSR frequency TLV for indoor frequency (e.g., F1) in the NBR-ADV message, MS can scan the frequency according to the NBR-ADV message.  When MS selects the indoor frequency in the same cell, MS would start the intra-cell handover.  MS can omit part of the regular handover process and use re-entry process instead.

Figure 6: Intra-cell handover from outdoor to indoor (NBR-ADV approach)

- Indoor to outdoor intra-cell handover

Figure 7: Intra-cell handover from indoor to outdoor

Figure 7 shows the intra-cell handover from indoor to outdoor.

DCD/UCD approach: By decoding of DCD/UCD, MS can detect the intra-cell handover. The BSID in the outdoor frequency (e.g., F2) equals to the BSID in the indoor frequency (e.g., F1). MS will start the reentry process instead of the hard handover process.

Figure 8: Intra-cell handover from indoor to outdoor (DCD/UCD approach)

NBR-ADV approach: By using NBR-ADV, MS can decide the intra-cell handover. In the intra-cell handover, MS will start the reentry process instead of the handover process.

Figure 9: Intra-cell handover from indoor to outdoor (NBR-ADV approach)

# Proposed Text

6.3.21.1 Network topology advertisement

[Change the paragraph as follows]

A BS shall broadcast information about the network topology using the MOB_NBR-ADV message. The message provides channel information for neighboring BSs normally provided by each BS’s own DCD/UCD message transmissions. The message can also provide channel information for the frequency shift repeater in the serving cell.  A BS may obtain that information over the backbone network. Availability of this information facilitates MS synchronization with neighboring BS by removing the need to monitor transmission from the neighboring BS for DCD/UCD broadcasts.

6.3.21.2 HO process

[Change the paragraph as follows]

- Synchronization to target BS DL — The MS shall synchronize to the DL transmissions of the Target BS and obtain DL and UL transmission parameters. If the MS had previously received a MOB_NBR-ADV message including Target BSID, Physical Frequency, DCD and UCD, this process may be shortened. If the Target BS had previously received HO notification from the Serving BS over the backbone, then the Target BS may allocate one or more non-contention-based initial ranging opportunities. If the target BSID is identical to the serving BSID, the serving BS shall not send the HO notification. And then the target BS may allocate one ore more non-contention-based initial ranging opportunities.

# Reference

- Toshiyuki Kuze et. al., “IEEE 802.16 Repeater and Standard Clarification in Rev2”, IEEE C802.16maint-08/295r2, Kobe, Japan September 2008