| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Bit definitions in the Request/Transmission Policy parameter |  |
| Date Submitted | 2008-08-29 |  |
| Source(s) | Joseph Schumacher Mark Marsan Motorola | Voice: +1-847-632-5978 E-mail: mailto:j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Revision 2 |  |
| Abstract | Text describing the bit settings for the Request/Transmission Policy parameter is confusing. |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Bit definitions in the Request/Transmission Policy parameter

Joe Schumacher, Mark Marsan

Motorola

# Introduction

Section 11.13.11, Request/Transmission Policy parameter, describes a TLV that uses inverted logic to change default attributes for the associated service flow. The paragraph states:

“The value of this parameter provides the capability to specify certain attributes for the associated service flow. These attributes include options for PDU formation and, for UL service flows, restrictions on the types of BR options that may be used. An attribute is enabled by setting the corresponding bit position to 1. For attributes affecting UL BR types, a value of zero indicates the default actions described in the scheduling service description in 6.3.5 shall be used. A value of 1 indicates that the action associated with the attribute bit overrides the default action.”

This paragraph states two incompatible requirements for the case where a bit is set to 1. In particular, the last sentence is confusing since it isn’t clear what “overrides” means.

# Proposed Text Changes

11.13.11 Request/Transmission Policy parameter

The value of this parameter provides the capability to specify certain attributes for the associated service flow. These attributes include options for PDU formation and, for UL service flows, restrictions on the types of BR options that may be used. An attribute is enabled by setting the corresponding bit position to 1. For attributes affecting UL BR types, a value of zero indicates the default actions described in the scheduling service description in 6.3.5 shall be used. A value of 1 indicates that the action associated with the attribute bit overrides the default action.