| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink Power Control Clarifications |  |
| Date Submitted | 2008-11-13 |  |
| Source(s) | Teck Hu Nokia Siemens Networks | Voice: E-mail: teck.hu@nsn.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract | This contribution proposes clarifications to the UL power control operations |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D7. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink Power Control Clarifications

Teck Hu

Nokia Siemens Networks

# Problem description

Currently, the specifications related to UL power control are unclear when the following scenarios listed below are encountered:

- when the BS commands the MS to switch PC mode and the BS does not receive any reply from the MS (due to errors in reception of the MS response, or if no bandwidth is allocated to the MS), the operation of the power control operation is not well described, and

- during sleep, idle and scanning periods, and

- the measurement operation for the condition to report UL Tx Power report at the MS, and

- the power control modes during at the Target BS after handover.

# Proposed Changes

Change #1: Insert the following text into Section 6.3.2.3.54:

6.3.2.3.54 PMC_RSP (power control mode change response) message

For OFDM and OFDMA PHY modes, PMC_RSP is sent from BS as a confirmation of MS’s UL power control change intention with PMC_REQ message or it is sent unsolicited manner to command MS to change the UL power control mode as indicated in the PMC_RSP. <insert>The MS should switch to the new power control mode as instructed by the BS through PMC_RSP. The BS may resend PMC_RSP to the MS if the BS fails to receive PMC_REQ (Confirmation = 1) from the MS.</insert> When the open-loop power control is indicated, Offset_BSperMS is included. When the closed-loop power control is indicated, power adjust can be signaled. Before the first PMC_RSP message from BS, the default power control mode shall be the closed power control scheme (Table 161).

Change #2: Insert the following at the end of Section 8.4.10.3

<insert>

The MS shall not change its power control mode during sleep mode. The MS shall not change its power control mode upon exit from sleep mode. In case of network re-entry from idle mode, the MS should use the closed loop power control mode upon completion of the initial ranging until it receives PMC_RSP message from BS.</insert>

Change #3: Insert the following at the end of Section 8.4.10.3.2.1

<insert> The MS should estimate the Mavg(n) on an ongoing basis, using all frames during availability periods instead of only during frames when the MS report its transmission power </insert>

Change #4: Insert the following at the end of Section 8.4.10.3.2.2

<insert> During HO ranging following the first transmission of the first CDMA code until the completion of the ranging process, the power control mode to be used should be up to the MS. Upon completion of HO, the MS should use the power control mode it was in before the HO. </insert>

# References

[IEEE802.16-Rev2/D7]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D7 (October 2008).