| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to MIMO midamble related equations |  |
| Date Submitted | 2008-11-12 |  |
| Source(s) | Erik Colban NextWave Wireless Inc.* | Voice: E-mail: ecolban@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16 Sponsor Ballot Recirculation 1 (as announced in IEEE 802.16-08/052) |  |
| Abstract | This contributions provides clearer instructions regarding the changes to adopt for comment A101. |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Corrections to MIMO midamble related equations

Erik Colban, Wee Peng Goh

NextWave Wireless Inc.

# Background

In ballot 802.16Rev2R0, comment #800 to modify the equation mid_amble_carrier_set on page 1055 was accepted. However, there are two equations in that section and only one of them has been changed. Similar to the equation on line 8, the equation on line 42 for mid_amble_carrier_set contains the term Nused/2 which would lead to a non-integer number if the Nused value is odd. According to section 8.4.6.1.2.1 and 8.4.6.3, Nused, which includes DC tone, is always an odd number for all FFT sizes. So the term (-N_used/2) is not an integer in the above equation.

Similarly, on page 1056, line 36 through 52 in section 8.4.8.5.2, the Nused term needs to be modified.

# Proposed Changes

On page 1055, line 43, change the equation to:

Midamble_carrier_set =−(Nused−1)2+n+Ntk

On page 1056, lines 34-39, change equation (120) to:

PIDcell,S[kfoi]=1−(2)qIDcell[m],kfoi=2mNt2−Nused−12+n, and  m=0,1,....,Nused−12Nt2−10,   otherwise

On page 1056, lines 42-52, change equation (121) to:

qIDcell[m]=R(8m9+mmod9), ifmmod9=0,1,2,.....,7T(m9),ifmmod9=8, where m=0,1,....,Nused−12Nt2−1