| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIB support to the LBS-ADV message |  |
| Date Submitted | 2008-12-17 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Sponsor Ballot Recirculation, on P802.16Rev2/D8 |  |
| Abstract | This contribution proposes wmanIf2mBsLbsAdvNeighborBsTable to support LBS-ADV message. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MIB support to the LBS-ADV message

Joey Chou

Intel

# Introduction

This contribution proposes wmanIf2mBsLbsAdvNeighborBsTable to support LBS-ADV message.

# Proposed Text

  - Proposed text to subclause 13.1.4.1.3

13.1.4.1.3 wmanIf2mBsNeighborAdv

[Add wmanIf2mBsLbsAdvNeighborBsTable to Figure 332]

Figure 332—wmanIf2mBsNeighborAdv structure

<insert>

13.1.4.1.3.5 wmanIf2mBsLbsAdvNeighborBsTable

wmanIf2mBsNeighborBsOfdmaDcdTable contains the attributes that are broadcast in the LBS-ADV message. <insert>

  - ASN.1 code for wmanIf2mBsLbsAdvNeighborBsTable

<insert>

wmanIf2mBsLbsAdvNeighborBsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsLbsAdvNeighborBsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes that are broadcast in

the LBS-ADV message."

REFERENCE

"Subclause 6.3.2.3.59"

::= { wmanIf2mBsNeighborAdv 5 }

wmanIf2mBsLbsAdvNeighborBsEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsLbsAdvNeighborBsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS locaton that are

broadcast in the LBS-ADV message."

INDEX     { wmanIf2mBsLbsBsId }

::= { wmanIf2mBsLbsAdvNeighborBsTable 1 }

WmanIf2mBsLbsAdvNeighborBsEntry::= SEQUENCE {

wmanIf2mBsLbsBsId                       WmanIf2mNbrBsId,

wmanIf2mBsLongitudeLong                 WmanIf2mLocationUnits,

wmanIf2mBsLatitudeLong                  WmanIf2mLocationUnits,

wmanIf2mBsAttitudeLong                  WmanIf2mAttitude,

wmanIf2mBsLongitudeShort                Integer32,

wmanIf2mBsLatitudeShort                 Integer32,

wmanIf2mBsAttitudeShort                 Integer32}

wmanIf2mBsLbsBsId OBJECT-TYPE

SYNTAX      WmanIf2mNbrBsId

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The least significant 24 bits of the Base Station ID

parameter in the DL-MAP message of the Serving BS or

Neighbor BS."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 1 }

wmanIf2mBsLongitudeLong OBJECT-TYPE

SYNTAX      WmanIf2mLocationUnits

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The longetude of the absolute BS location in long format,

as defined in RFC3825. It is represented as a 34 bit

fixed-point 2's complement number, consisting of 9 bits of

integer and 25 bits of fraction, and is normalized to

within +/- 180 degrees."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 3 }

wmanIf2mBsLatitudeLong OBJECT-TYPE

SYNTAX      WmanIf2mLocationUnits

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The latitude of the absolute BS location in long format, as

defined in RFC3825. It is represented as a 34 bit

fixed-point 2's complement number, consisting of 9 bits of

integer and 25 bits of fraction, and is normalized to

within +/- 90 degrees."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 2 }

wmanIf2mBsAttitudeLong OBJECT-TYPE

SYNTAX      WmanIf2mAttitude

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The attitude of the absolute BS location in long format, as

defined in RFC3825. It is represented as a 30 bit

fixed-point 2's complement number with 22 bits of integer

and 8 bits of fraction."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 4 }

wmanIf2mBsLongitudeShort OBJECT-TYPE

SYNTAX      Integer32

UNITS       "1 / 2^-15 degree"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The longitude of the absolute BS location in short format.

It is expressed as 2-15 part of a degree, using 2's

complement notation to express negative (West) values."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 5 }

wmanIf2mBsLatitudeShort OBJECT-TYPE

SYNTAX      Integer32

UNITS       "1 / 2^-16 degree"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The latitude of the absolute BS location in short format.

It is expressed as 2-16 part of a degree, using 2's

complement notation to express negative (South) values."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 6 }

wmanIf2mBsAttitudeShort OBJECT-TYPE

SYNTAX      Integer32

UNITS       "Meters"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The attitude of the absolute BS location in short format.

It is is expressed in meters above sea level using 2's

complement notation to express negative (below sea level)

values."

::= { wmanIf2mBsLbsAdvNeighborBsEntry 7 }<insert>