| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Capability Exchange clean-up for extended features in Rev2 |  |
| Date Submitted | 2008-12-18 |  |
| Source(s) | Brian Shim, Yeongmoon Son Samsung | Voice: E-mail: brian.shim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract | This contribution contains a clean-up for extended capability negotiation. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D8. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Capability Exchange clean-up for extended features in Rev2

Brian Shim, Yeongmoon Son

Samsung

# Problem description

# The Extended capability TLV in SBC-REQ/RSP messages specifies extended capability for ARQ and BS_Controlled_HO. But they are not really necessary to be negotiated during SBC and they should be moved to REG-REQ/RSP messages. Generally PKM related parameters and PHY related ones should be negotiated using SBC-REQ/RSP message.

# By moving this TLV to REG-REQ/RSP, we can reduce the message size of SBC-REQ/RSP which is transmitted on the Basic CID. As you know, management messages transmitted on the Basic CID shall be neither fragmented nor packed.

# Moreover, Group DSx supported TLV in REG-REQ/RSP messages has only a single bit indication. So combining the Group DSx capability into a bit indication in the disjointed ‘Extended capability’ TLV is helpful to reduce the message overhead of REG-REQ/RSP messages.

# Proposed Changes

The proposal has the following two main changes.

- Move ‘Extended capability TLV’ from SBC-REQ/RSP messages to REG-REQ/RSP messages.

- Combine ‘Group DSx supported TLV’ (in REG-REQ/RSP messages) with the Extended capability TLV.

[Adopt the following change in 6.3.2.3.7, on pp.97, line #29]

The REG-REQ may contain the following TLV:

MAC header and extended subheader support (11.7.21)

BS Switching Timer (11.7.12.7)

Extended capability (see 11.7.8.11)

[Adopt the following change in 6.3.2.3.8, on pp.100, line #14]

The REG-RSP may contain the following TLV:

MAC header and extended subheader support (11.7.21)

Handover Indication Readiness Timer (11.7.12.6)

Extended capability (see 11.7.8.11)

[Adopt the following change in 6.3.2.3.23, on pp.132, line #63]

Capabilities for construction and transmission of MAC PDUs (see 11.8.2)

Security Negotiation Parameters (see 11.8.4)

Visited NSP ID (see 11.8.11)

Auth Type for EAP (see 11.8.12)

MIH Capability Supported (see 11.8.10)

Extended capability (see 11.8.15)

SDU MTU capability (see 11.8.16)

[Adopt the following change in 6.3.2.3.24, on pp.134, line #49]

MIH Capability Supported (see 11.8.10)

Extended capability (see 11.8.15)

[Adopt the following change in 6.3.4.2.1, on pp.283, line #13]

6.3.4.2.1 ARQ Feedback IE format with extended capability

Table 169 defines the ARQ Feedback IE used by the receiver to signal positive or negative Acknowledgments when the BS and MS use extended capability (11.8.1511.7.8.11, bit 0 set to 1). A set of IEs of this format may be transported either as a packed payload (“piggybacked”) within a packed MAC PDU or as a payload of a standalone MAC PDU.

[Adopt the following change in 11.7, on pp.1218]

11.7 REG-REQ/RSP management message encodings

| Type | Parameter | Type | Parameter |
| --- | --- | --- | --- |
| 23 | Maximum Number of Bursts Transmitted Concurrently to the MS | 49 | Group DSx supportedExtended capability |

[Adopt the following change in 11.8, on pp.1234]

11.8 SBC-REQ/RSP management message encodings

| Type | Parameter | Type | Parameter |
| --- | --- | --- | --- |
| 25.4 | PN window size | 184 | Extended capabilitySDU MTU capability |
| 27 | Extended subheader capability | 185 | SDU MTU capability |
| 28 | HO trigger metric support |  |  |

[Adopt the following change in 11.7.8.11, on pp.1225]

11.7.8.11 Group DSx supportedExtended capability

The Group DSx supported TLV indicates if Group parameter Create/Change TLV (11.13.39) is supported. SSs and BSs that support the Group parameter Create/Change TLV shall identify themselves by including this Group DSx supported TLV. For each bit, a value of 0 indicates "not supported" while 1 indicates "supported".

The extended capability field specifies extended capability support for the specified features. For each bit, a value of 0 indicates “not supported” while 1 indicates “supported”. If the TLV is not transmitted, the default value of each mentioned capability is “not supported”.

| Type | Length | Value (variable-length) | Scope |
| --- | --- | --- | --- |
| 49 | 1 | Bit#0: Indicates support for Group parameter Create/ Change TLV (11.13.39) Bits #1 - #7: reserved Bit #0: Indicates the capability to sup-port ARQ Map Last Bit concept and the optimized Sequence Block as defined in Table 169. The feature is enabled only in case both MS and BS support it. Bit #1: Indicates the capability to sup-port BS_Controlled_HO (see 6.3.21.2.2). If the MS does not support this capability, it may ignore the BS_Controlled_HO flag in the DCD. Bit #2: Indicates support for Group parameter Create/ Change TLV (11.13.39) Bits 3-7: Reserved, set to zero. | REG-REQ, REG-RSP |

[Delete 11.8.15 on pp.1265]

11.8.15 Extended capability

The extended capability field specifies extended capability support for the specified features. For each bit, a value of 0 indicates “not supported” while 1 indicates “supported”. If the TLV is not transmitted, the default value of each mentioned capability is “not supported”.

| Name | Type | Length | Value (variable-length) | Scope |
| --- | --- | --- | --- | --- |
| Extended capability | 184 | 1 | Bit 0:This bit describes the capability to sup-port ARQ Map Last Bit concept and the optimized Sequence Block as defined in Table 169. The feature is enabled only in case both MS and BS support it. Bit 1:This bit describes the capability to sup-port BS_Controlled_HO (see 6.3.21.2.2). If the MS does not support this capability, it may ignore the BS_Controlled_HO flag in the DCD. Bits 2-7: Reserved, set to zero. | SBC-REQ SBC-RSP |

[Adopt the following change in 11.8.16, on pp.1266]

11.8.165 SDU MTU capability

The maximum MTU capability TLV specifies the upper bound for the SDU MTU size supportable on transport connections.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Maximum MTU capability | 185184 | 2 | An unsigned 16 bit integer. When in SBC-REQ: The maximum SDU, in bytes, that the BS may send to the SS on transport connections. When in SBC-RSP: The maximum SDU, in bytes, that the SS may send to the BS or receive from the BS on transport connections. | SBC-REQ SBC-RSP |

# References

[IEEE802.16-Rev2/D8]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D8 (December 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.