| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Idle Mode Operation |  |
| Date Submitted | 2009-01-14 |  |
| Source(s) | Hassan Yaghoobi Joey Chou Intel Yeongmoon Son Samsung | E-mail: hassan.yaghoobi@intel.com joey.chou@intel.com ym1004.son@samsung.com |
| Re: | REV2/D8 |  |
| Abstract | Proposal for idle mode operation clarification |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for Idle Mode Operation

Hassan Yaghoobi, Joey Chou

Intel Corporation

Yeongmoon Son

Samsung Electronics

- Proposed text

1.1	Changes the Action field of Action Code = 6 in Table 96 as the following

-------------------------------  Text Start  ---------------------------------------------------

<delete> This option is valid only in response to a DREG-REQ message with De-Registration Request Code = 0x01. The MS may retransmit the DREG-REQ message after the time duration (REQduration) provided in the message; BS transmission of a subsequent DREG-CMD message with Action Code 03 shall cancel this restriction. <delete>

<insert> This option is valid only in response to a DREG-REQ message with De-Registration Request Code = 0x01. The behavior of MS to this action code is described in Section 6.3.23.1. <insert>

-----------------------------------Text End -----------------------------------------------------

1.2 	Changes the 1st paragraph in Section 6.3.23.1 as the following:

6.3.23.1 MS idle mode initiation

Idle mode initiation may begin after MS deregistration. During normal operation with its serving BS, an MS may signal intent to begin idle mode by sending a DREG-REQ message with the De-registration_Request_Code parameter = 0x01; request for MS deregistration from serving BS and initiation of MS idle mode. <insert> When the BS decides to reject MS-initiated idle mode request, the BS shall send a DREG-CMD with action code 0x06 in response to this DREG-REQ message. The BS may include REQ-Duration TLV in this DREG-CMD message. In this case, the MS may retransmit the DREG-REQ message after the expiration of REQ_Duration. <insert>If the MS does not receive the DREG-CMD message within T45 timer expiry after it sends the DREG-REQ message to the BS, the MS shall retransmit the DREG-REQ message as long as DREG Request Retry Count has not been exhausted. Otherwise, the MS shall reinitialize MAC. Also, the BS shall start Management_Resource_Holding_Timer to maintain connection information with the MS as soon as it sends the DREG-CMD message to the MS. If Management_Resource_Holding_Timer has been expired, the BS shall release connection information with the MS.

<Note to Editor, start a new paragraph>

Similarly, a serving BS may signal for an MS to begin idle mode by sending a DREG-CMD message with action code 0x05 in unsolicited manner.  <insert> This unsolicited DREG-CMD may include REQ-Duration TLV. When an MS receives an unsolicited DREG-CMD without REQ_Duration TLV, the MS shall immediately send a DREG-REQ message. <insert> In this case of BS-initiated idle mode, the serving BS shall start T46 timer as well as Management_Resource_Holding_Timer at the same time. If the BS does not receive the DREG-REQ message with the De-registration_Request_Code parameter = 0x02 from the MS in response to the unsolicited DREG-CMD message with action code 0x05 within T46 timer expiry, the BS shall retransmit the DREG-CMD message with action code 0x05 in unsolicited manner as long as DREG command retry count has not been exhausted. MS shall enter idle mode after it sends DREG-REQ message with the De-registration_Request Code parameter = 0x02 in response to the unsolicited DREG-CMD message with action code 0x05.

-------------------------------  Text End  ---------------------------------------------------