| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | BS Usage of TEK only described for PKMv1 |  |
| Date Submitted | 1-15-2009 |  |
| Source(s) | David Johnston Intel Corporation Phil Barber Huawei | Voice: 503 200 0214 E-mail: dj.johnston@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Rev2 D8 |  |
| Abstract | The BS usage of TEK section includes a diagram for TEK management with text limiting its applicability to PKMv1. This diagram is actually applicable to PKMv2. Better wording is defined. Clarity is added concerning the freshness of TEKs |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

BS Usage of TEK Only Described for PKMv1

David Johnston

Intel Corporation

Phil Barber

Huawei

The text on page 551 in the BS usage of TEK section:

“The right-hand side of 167 illustrates the BS’s management of an SA’s TEKs in PKMv1, where the shaded portion of a TEK’s lifetime indicates the time period during which that TEK shall be used to encrypt MAC PDU payloads.”

limits the applicability of this diagram to PKMv1 and only makes the right hand side of it relevant. There isn’t an equivalent diagram for the SS usage of TEK section and the limitation to the right hand side leaves this behavior unspecified for the SS an unspecified for PKMv2.

In fact this diagram is correct for both PKMv1 and PKMv2 and both sides (SS and BS) are covered in the same diagram. The proposed text addresses this issue.

# Proposed Text

[Modify first paragraph of page 551, lines 1 through 4 as follows]

The right-hand side of Figure 167 illustrates the BS’s management of an SA’s TEKs in PKMv1, where the shaded portion of a TEK’s lifetime indicates the time period during which that TEK shall be used to encrypt MAC PDU payloads.

[Insert at page 116, line 40, section 6.3.2.3.9.21 :]

The GKEK-Parameters attribute is a compound attribute containing all of the GKEK-related parameters corresponding to a GSAID. This would include the GKEK, the GKEK’s remaining key lifetime, and the GKEK’s key sequence number. The older generation of the GKEK-Parameters attribute is valid within the current lifetime, and the newer generation of the GKEK-Parameters attribute is valid within the next lifetime.

The BS shall always supply fresh (see 7.2.2.2.6) key material in the newer generation of Key Parameters in the PKMv2 Key-Reply message.

The HMAC/CMAC Digest attribute shall be the final attribute in the message’s attribute list.

[On page 551, in Figure 167, modify the figure: change all instances of ‘Key Request’ to ‘Key Request/PKMv2 Key-Request’]

[On page 551, in Figure 167, modify the figure: change all instances of ‘Key Reply’ to ‘Key Reply/PKMv2 Key-Reply’]