| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uncontrolled Handover |  |
| Date Submitted | 2009-03-12 |  |
| Source(s) | Aik Chindapol, NSN Ji-Yun Seol, Samsung Sylvain Labonte, Sequans | Voice: +33 689030089 E-mail: sylvain@sequans.com |
| Re: | Uncontrolled Handover in relation with Liaison letter from WMF |  |
| Abstract | Modified text to handle Uncontrolled Handovers |  |
| Purpose | Include in Rev 2 or its follow up revision |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interoperability Problem Statement

Due to suboptimal radio planning conditions, link loss, or MS implementation, when a MS detects a drop, the MS might begin ranging at a preferred BS which was not contacted by the serving BS for handover preparation.

In such a case, if the MS provides Ranging Purpose Indication TLV Bit #0 set to 1 and Serving BSID TLV in RNG-REQ, the target BS is able to handle the unexpected handover within a reduced amount of time, comparing to the required time for full initial network entry.

This concept that has been called Uncontrolled Handover and has been considered by NWG is missing in IEEE P802.16Rev2[1].

This IOPR identifies the case when Uncontrolled Handover can occur and provides the appropriate usage of TLVs in the air link for the BS to handle such a case.

- 2.3     Changes in the IEEE 802.16 Standards (Optional)

6.3.21.2.6 Drops during HO

A drop is defined as the situation where an MS has stopped communication with its serving BS (either in the DL or in the UL) before the HO process (6.3.21.2) has been started, or the HO Process was started but has not been completed.

An MS can detect a drop by its failure to demodulate the DL, or by exceeding the RNG-REQ retries limit  allowed for the periodic ranging mechanism. A BS can detect a drop when the Number of retries limit   allowed on inviting ranging requests for the periodic ranging mechanism is exceeded.

A “drop during HO” is defined as the situation where the MS loses communication to a target BS during handover attempt to this target BS.  An MS can detect a drop during HO by its failure to demodulate the DL signals from this target BS, or by exceeding the RNG-REQ retries limit.

When the MS has detected a drop during network reentry with a target BS, it may attempt network reentry    with its preferred target BS as through Cell Reselection (see 6.3.21.2.1), which may include resuming communication with the serving BS by sending MOB_HO-IND message with HO_IND type = 0b01 (HO cancel) or performing network reentry at the serving BS.

When the MS has detected a drop or a drop during HO, it may perform network reentry at the preferred target BS as described in 6.3.21.2.7. The preferred target BS may be the serving BS.

If the MS performs network reentry, the MS shall perform CDMA ranging with target BS using codes from HO codes domain.

Upon the preferred target BS’s sending RNG-RSP with Ranging Status = success, the preferred target BS shall provide CDMA ALLOCllocation IE with appropriate UL allocation for RNG-REQ from MS.  MS shall send RNG-REQ with MAC address and HMAC/CMAC.  Serving BSID TLV and Ranging Purpose Indication TLV Bit #0 set to 1 shall be included in RNG-REQ as well.  The preferred target BS may now identify that HO attempt by MS was not coordinated with the serving BS and may request all relevant MS context from the serving BS.   Using this information, the preferred target BS shall now send RNG-RSP with HO process optimization bitmap and network reentry may continue as in the typical, nondrop case.

When the serving BS has detected a drop, it shall react as if a MOB_HO-IND message has been received with HO_IND_type indicating serving BS release.

The network reentry process at the serving BS is identical to the network reentry process at any other target BS, both for the serving BS and for the MS (see 6.3.21.2.7). If the serving BS has discarded the MS context, the network re-entry procedure shall be the same as full network reentry with HO optimization rules and scenarios defined in 6.3.21.2.10.

When the MS has detected a drop during HO, a resumption of communication with the serving BS may also be executed by using  HO Cancellation procedure specified in  6.3.21.2.3.