| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Activation and transition of system states for Sleep Mode |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Jonathan Labs Wavesat, Inc. 1375 Trans-Canada Hwy Dorval, Qc H9P 2W8 Canada | Voice: +1 514 684 0200 x325 E-mail: jlabs@wavesat.com |
| Re: | Change request A.1.3 and A.2.3 in Liaison Statement IEEE L802.16-09/0123 |  |
| Abstract | The contribution editorially cleans up the proposed changes in the CR submitted in Annex A of Liaison Statement IEEE L802.16-09/0123 for the WiMAX TWG |  |
| Purpose | Clarify the exact editorial instructions need for the proposed CR |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Activation and transition of system states for Sleep Mode

- Jonathan Labs

- TG Chair

- Wavesat, Inc.

# Problem Statement

[From A.1.3 of IEEE L802.16-09/0123:]

It should be clearer defined in the standard when the following states are activated.

Because the standard does not mention explicit or remains vague how certain features and mechanisms have to work while the MS is in Sleep/scan mode. This may lead to different behavior and interpretations of MS and BS vendors.

|  | MS has a Scanning pattern | MS in Sleep mode with an active PSC |
| --- | --- | --- |
| MOB_SLP-REQ message with new PSC activation | Add the following text to section 6.3.2.3.40: “In case of activation of a PSC the Scanning pattern should be deactivated at “Start_frame_number” defined in MOB_SLP-RSP message. Note that scanning can continue autonomously by the MS during the unavailability interval” | Add the following text to section 6.3.2.3.40: “In case of activation of a new PSC, the Current PSC is deactivated upon reception of MOB_SLP-RSP message. The new PSC is activated upon “Start_frame_number” defined in MOB_SLP-RSP message.” |
| MOB_HO-REQ message |  | Add the following text to section 6.3.20.1: “Sleep is deactivated upon MOB_HO-REQ message” |
| DREG-REQ message to enter Idle mode | Add the following text to section 6.3.20.1: “Scanning pattern should be deactivated upon DREG-REQ message” | Add the following text to section 6.3.20.1: “Sleep is deactivated upon DREG-REQ message” |

# Proposed Changes

[Add a new section 6.3.20.10:]

6.3.20.10 Activation and transition of MS states

Table 201a summarizes the state transitions of a MS with active PSC/PSCs after the reception of one of the messages listed in the first column:

Table 201a State transitions of a MS with active PSC/PSCs

| New Message | Transition of an MS that is in Sleep mode with an active PSC |
| --- | --- |
| MOB_SLP-RSP or DL sleep control extended subheader. message with new PSC activation | See 6.3.20. Remark: If the MS and BS activate a new PSC that replaces an existing PSC, then the new PSC is activated and the existing PSC is deactivated at start_frame_number defined in MOB_SLP-RSP or in DL sleep control extended subheader. |
| MOB_HO-REQ message | All PSCs are deactivated upon transmission or reception of the MOB_HO-REQ message. This shall apply also to the case that results in HO cancel |
| DREG-REQ message to enter Idle mode | Sleep mode is deactivated upon completion of deregistration. |

[Insert the following at the end of 6.3.21.1.2:]

Table 201b gives a summary about the state transitions that a MS with active scanning pattern shall perform after the reception of the messages listed in the first column.

Table 201b State transitions of a MS with active scanning pattern

| New Message | Transition of an MS that has a Scanning pattern |
| --- | --- |
| MOB_SLP-RSP or DL sleep control extended subheader. message with new PSC activation | Activation of a PSC cancels the entire scanning context. Note that the MS may perform scanning autonomously during the unavailability interval. Any active scanning shall be deactivated upon sleep entry, at start_frame_number defined in MOB_SLP-RSP or in DL sleep control extended subheader. |
| MOB_HO-REQ message | Scanning pattern is deactivated upon transmission or reception of the MOB_HO-REQ message. |
| DREG-REQ message to enter Idle mode | Scanning pattern should be deactivated upon completion of deregistration. |