| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO plus Beamforming Support |  |
| Date Submitted | 2010-11-02 |  |
| Source(s) | Mark Marsan Joe Schumacher Motorola | E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Std 802.16-2009 errata |  |
| Abstract | 802.16-2009 doesn’t support simultaneous specification of MIMO + Beam Forming |  |
| Purpose | Review and approve for inclusion in IEEE 802.16maint-09/0007 and P802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MIMO plus Beamforming Support

Mark Marsan, Joe Schumacher

Motorola

# Introduction

The standard allows the MS to indicate that it supports beamforming by setting bit#10 in the SBC TLV “OFDMA SS demodulator” (type 151) to indicate support for dedicated pilots.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 151 | 1 or 2 | Bit 0: 64-QAM Bit 1: BTC Bit 2: CTC Bit 3: STC Bit 4: CC with optional interleaver Bit 5: HARQ Chase Bit 6: HARQ CTC_IR Bit 7: Reserved; shall be set to zero Bit 8: HARQ CC IR Bit 9: LDPC Bit 10 Dedicated pilots Bits 11–15: Reserved; shall be set to zero. | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

The standard also allows the MS to indicate support for various flavors of MIMO using bit#0 to bit#11 in the SBC TLV “OFDMA SS demodulator for MIMO support” (type 176).

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 176 | 3 | Bit 0 2-antenna STC Matrix A Bit 1 2-antenna STC Matrix B, vertical coding Bit 2 2-antenna STC Matrix B, horizontal coding Bit 3: 4-antenna STC Matrix A Bit 4: 4-antenna STC Matrix B, vertical coding Bit 5: 4-antenna STC Matrix B, horizontal coding Bit 6: 4-antenna STC Matrix C, vertical coding Bit 7: 4-antenna STC Matrix C, horizontal coding Bit 8: 3-antenna STC Matrix A Bit 9: 3-antenna STC Matrix B Bit 10: 3-antenna STC Matrix C, vertical coding Bit 11: 3-antenna STC Matrix C, horizontal coding Bit 12: Capable of calculating precoding weight Bit 13: Capable of adaptive rate control Bit 14: Capable of calculating channel matrix Bit 15: Capable of antenna grouping Bit 16: Capable of antenna selection Bit 17: Capable of codebook based precoding Bit 18: Capable of long-term precoding Bit 19: Capable of MIMO Midamble Bit 20: Allocation granularity in a DL PUSC STC zone with dedicated pilots Bit 21: Concurrent allocation support in a DL PUSC STC zone with dedicated pilots Bit 22: Allocation granularity in a DL AMC STC zone with dedicated pilots for Matrix B Bit 23: Concurrent allocation support in a DL AMC STC zone with dedicated pilots | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

Unfortunately, the standard does not allow the MS to indicate that it supports the combination of MIMO and beamforming in a single DL allocation.  For example, if the MS sets bit#10 in TLV 151 to indicate support for dedicated pilots and bit#1 in TLV 176 to indicate support for 2-antenna STC Matrix B, does this also imply that a DL allocation can use both MIMO matrix B and dedicated pilots? Or will these DL allocations fail because the MS supports either dedicated pilots without MIMO or MIMO without dedicated pilots?

# Solution Outline

The solution is to use a reserved bit in TLV 151 to indicate simultaneous support for beamforming and MIMO. When set to zero, the MS does not simultaneously support Beamforming and MIMO in the same DL allocation.  When set to 1, the MS can simultaneous support Beamforming and MIMO in the same DL allocation where the details of MIMO support are defined by the existing SBC TLV “OFDMA SS demodulator for MIMO support” (type 176). The existing bit#10 is used to indicate support for dedicated pilots without MIMO.

# Proposed Text

11.8.3.5.2 OFDMA SS demodulator

This field indicates the different demodulator options supported by a WirelessMAN-OFDMA PHY SS for DL reception. This field is not used for other PHY specifications. A bit value of 0 indicates “not supported” while 1 indicates “supported.” The SS or BS may include bits 0–7 only and set the TLV length field to 1, in which case the receiving entity shall assume that the options represented by bits 8–15 are not supported.

[Add the following paragraph to 11.8.3.5.2 as indicated:]

When bit#11 is set to 1, it indicates that a DL allocation may use both dedicated pilots and MIMO where MIMO functionality is defined by TLV 176.  When bit#10 is set to 1, it indicates that a DL allocation may use dedicated pilots without MIMO.

[Change the table in 11.8.3.5.2 as indicated:]

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 151 | 1 or 2 | Bit 0: 64-QAM Bit 1: BTC Bit 2: CTC Bit 3: STC Bit 4: CC with optional interleaver Bit 5: HARQ Chase Bit 6: HARQ CTC_IR Bit 7: Reserved; shall be set to zero Bit 8: HARQ CC IR Bit 9: LDPC Bit 10: Dedicated pilots without MIMO Bit 11: Simultaneous support of dedicated pilots and MIMO Bits 11–15: Reserved; shall be set to zero. Bits 12–15: Reserved; shall be set to zero. | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |